3GPP TSG-RAN WG4 Meeting #109	R4-2319745

Chicago, IL, USA, 13 – 17 November 2023

Agenda Item:	8.7.4.3

Source:	Ericsson

Title:	PMI reporting requirements for FR2 UE multi-Rx reception

Document for:	Discussion

# 1	Introduction

RAN4#108bis agreed with the way forward on UE demodulation and CSI requirements for FR2 multi-Rx DL reception [1]. This contribution shows our simulation results and discusses the open issues on the PMI reporting requirements with FR2 UE multi-Rx reception scenario.

# 2	Discussion

## 2.1	Test parameters

RAN4 has agreed to define PMI reporting requirements using the sDCI-based SDM transmission scheme for multi-Rx reception scenario, where PDSCH is transmitted with single layer per TRP, and 2 CSI-RS ports are configured per TRP. Table 1 summarizes the current PMI test assumption according to [1].

Table 1	Test parameters for PMI reporting tests.

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | TRxP #1 | TRxP #2 |
| Transmit TRxP of SSB |  |  | TRxP #1 |  |
| PDCCH configuration | TCI state |  | TCI State #1 |  |
|  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  | TDD |  |
| Bandwidth |  | MHz | 100 |  |
| Subcarrier spacing |  | kHz | 120 |  |
| TDD DL-UL configurations |  |  | FR2.120-1 as specified in Annex A of TS 38.101-4 DDDSU |  |
| Active DL BWP index |  |  | 1 |  |
| Propagation channel |  |  | TDLA30-35 |  |
| Antenna configuration per TRxP |  |  | XP 2x4 (N1,N2) = (2,1), (4 antenna at UE across 2 panels) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 of TS 38.101-4 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 1.75 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| MCS |  |  | Option 1: 11 Option 2: 13 |  |
| Transmission scheme |  |  | sDCI-based SDM |  |

## 2.2	Simulation results

| (a) MCS11 2+2 | (b) MCS13 2+2 |
| --- | --- |

Figure 1	Simulation results for PMI reporting tests.

Table 2	Summary of PMI reporting tests.

| Test number | MCS, rank | FO | TO | rho | γ at SNR corresponding to 70% of peak rate with follow PMI | γ at SNR corresponding to 90% of peak rate with follow PMI |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 11, 2+2 | 0Hz | 0us | -15dB | 12.6dB/1.04 | 13.9dB/1.04 |
| 2-1 | 13, 2+2 | 0Hz | 0us | -15dB | 15.8dB/1.03 | 17.5dB/1.04 |

Figure 1 shows the simulation results for PMI tests and Table 2 summarizes the simulation results. In the simulation we set FO/TO=0Hz/0us since the existing FR1 PMI reporting test for sDCI-based transmission sets FO/TO=0Hz/0us, and the test metric is the ratio of two throughput curves.

According to the simulation results, it is observed the throughput ratio between follow PMI and random PMI is very small regardless of MCS11 or MCS13, or SNR test points whether 70% or 90% of peak rate. Since the existing FR2 PMI reporting test sets SNR test point with 90% of the peak rate, we slight prefer to set the SNR test point to the 90% of peak rate. Regarding the MCS, we propose to choose MCS11 because it is expected slight larger gain compared with MCS13.

Proposal 1: Set test point to the SNR achieving 90% of peak rate with follow PMI.

Proposal 2: Set MCS11 2+2 for PDSCH FRC.

Proposal 3: Set FO=0Hz and TO=0us for PMI reporting test.

# 3	Summary

Proposal 1: Set test point to the SNR achieving 90% of peak rate with follow PMI.

Proposal 2: Set MCS11 2+2 for PDSCH FRC.

Proposal 3: Set FO=0Hz and TO=0us for PMI reporting test.

# References

- R4-2316963, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm.