3GPP TSG-RAN WG4 Meeting # 109							R4-2319777

Chicago, USA, November 13 – 17, 2023

Source:			Samsung

Title:	Results of NTN co-existence study in above 10GHz

Agenda item:		8.26.2

Document for:		Discussion

- Introduction

This document provides coexistence results of NTN co-existence study in above 10GHz bands in accordance with the assumptions agreed in last RAN4 meeting.

- Discussion

Coexistence study

Results at 5% TP degradation of all scenarios defined in [1] are provided in Table 2.1. More detailed data are contained in the collection table in Attachment 1.

Table 2.1 ACIR requirements

|  | Aggressor | Victim | SAN Type | TP degradation @5% in edge UE | Mean TP degradation @5% |  | TP degradation @5% in edge UE | Mean TP degradation @5% |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VSAT elevation angle |  |  |  | 25 |  |  | 90 |  |  |
| Scenario 1 | NTN UL | TN UL | GEO | 0 | 0 |  | 0 | 0 |  |
|  |  |  | LEO600 | 0 | 0 |  | 0 | 0 |  |
|  |  |  | LEO1200 | 0 | 0 |  | 0 | 0 |  |
| Scenario 2 | TN UL | NTN UL | GEO | NaN | 0 |  | 0 | 0 |  |
|  |  |  | LEO600 | NaN | 0 |  | 0 | 0 |  |
|  |  |  | LEO1200 | NaN | 0 |  | 0 | 0 |  |
| Scenario 3 | NTN UL | TN DL | GEO | 0 | 0 |  | 0 | 0 |  |
|  |  |  | LEO600 | 0 | 0 |  | 0 | 0 |  |
|  |  |  | LEO1200 | 0 | 0 |  | 0 | 0 |  |
| Scenario 4 | TN DL | NTN UL | GEO | NaN | 17 |  | 6 | 4 |  |
|  |  |  | LEO600 | NaN | 15 |  | 2 | 0 |  |
|  |  |  | LEO1200 | NaN | 15 |  | 2 | 0 |  |
| Scenario 5 VSAT UE 2.5dB noise figure | TN DL | NTN DL | GEO | NaN | 22 |  | 28 | 20 |  |
|  |  |  | LEO600 | NaN | 26 |  | 24 | 18 |  |
|  |  |  | LEO1200 | NaN | 26 |  | 30 | 20 |  |
| Scenario 5 VSAT UE 6dB noise figure | TN DL | NTN DL | GEO | -- | -- | -- | -- | -- | -- |
|  |  |  | LEO600 | NaN | 22 |  | 22 | 16 |  |
|  |  |  | LEO1200 | NaN | 22 |  | 24 | 16 |  |
| Scenario 6 | NTN DL | TN DL | GEO | 0 | 0 |  | 0 | 0 |  |
|  |  |  | LEO600 | 0 | 0 |  | 0 | 0 |  |
|  |  |  | LEO1200 | 0 | 0 |  | 0 | 0 |  |
| Scenario 7 | NTN DL | TN UL | GEO | 0 | 0 |  | 0 | 0 |  |
|  |  |  | LEO600 | 0 | 0 |  | 0 | 0 |  |
|  |  |  | LEO1200 | 0 | 0 |  | 0 | 0 |  |
| Scenario 8 2.5dB noise figure | TN UL | NTN DL | GEO | NaN | 0 |  | 0 | 0 |  |
|  |  |  | LEO600 | NaN | 4 |  | 2 | 0 |  |
|  |  |  | LEO1200 | NaN | 4 |  | 0 | 0 |  |
| Scenario8 6dB noise figure | TN UL | NTN DL | GEO | - | - | - | - | - | - |
|  |  |  | LEO600 | NaN | 2 |  | 2 | 0 |  |
|  |  |  | LEO1200 | NaN | 2 |  |  |  |  |

Calibration

Since more data of calibration results were received after last RAN4, [2] has been updated as Attachment 2.

Proposal 3: To update calibration results table in accordance with Attachment 2.

- Conclusion

Observation 1: Basic on the study results of Scenario 1, 2, 3, 6, 7 and 8, no observable interference can be identified.

Observation 2: In scenario 4, 0-17dB ACIR results are needed in order to make sure the throughput loss criteria can be met in the NTN network.

Observation 3: In scenario 5, 16-30dB ACIR results are needed in order to make sure the throughput loss criteria in the NTN network.

Observation 4: No edge (5%) UE throughput can be observed from those study results, therefore due to the high coupling loss value between SAN and VSAT UE.

Proposal 1: To include these co-existence simulation results in TR 38.863 and update co-existence results table accordingly.

Proposal 2: To define an ACIR value of 30dB based on following results.

|  | ACIR |
| --- | --- |
| Scenario 4 | 17dB |
| Scenario 5 | 30dB |

Proposal 3: To update calibration results table with in accordance to Attachment 2.

- Reference

[1]R4-2317008 “Simulation assumptions for NTN co-existence study in bands above 10GHz”

[2] R4-2316998 “Co-existence data collection table”

Attachment 1 Co-existence results

Attachment 2 Updated Calibration Table