3GPP TSG-RAN WG4 Meeting #109	R4-2319787

Chicago, USA, November 13 – November 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on less than 5Mhz RRM core requirements

Agenda item:	8.14.4

Document for:	Discussion

Introduction

The following table describes the support of < 5MHz based on RAN1 and RAN4 agreements.

| Channel BW | Transmission BW | Channels | Band n100 | Other bands |
| --- | --- | --- | --- | --- |
| 3MHz | 12 PRB | PBCH (SSB) Note: SSB is punctured from 20 PRB | 12 PRB | Not supported |
|  |  | PDCCH CORESET#0 Note: not punctured, new 12 PRB configuration | 12 PRB |  |
|  |  | PDCCH Dedicated CORESET | 12 PRB Note: only configurations 6 and 12 are supported |  |
|  | 15 PRB | PBCH (SSB) Note: SSB is punctured from 20 PRB | 12 PRB | 12 PRB |
|  |  | PDCCH CORESET#0 Note: 15 PRB CORESET#0 is punctured from 24 PRB | 15 PRB, 12 PRB | 15 PRB, 12 PRB |
|  |  | PDCCH Dedicated CORESET Note: May be revisited based on RAN1 agreement on puncturing support in dedicated coreset from 18 PRB | 12 PRB Note: only configurations 6 and 12 are supported | 12 PRB Note: only configurations 6 and 12, are supported |
| 5MHz | 20 PRB | PBCH (SSB) Note: Legacy SSB | 20 PRB | Not supported |
|  |  | PDCCH CORESET#0 Note: CORESET#0 is punctured from 24 PRB | 20 PRB |  |
|  |  | PDCCH Dedicated CORESET Note: May be revisited based on RAN1 agreement on puncturing support in dedicated coreset from 24 PRB | 18 PRB Note: only configurations 6, 12, 18 are supported |  |

Table 1: Summary of less than 5MHz BW based on RAN1 and RAN4 design.

Discussion

Sub-topic 1-1 Applicability rules of RLM requirements

<Way forward>: For the applicability rules of RLM requirements for different bands. - Further discuss the applicability rules of RLM requirements for different bands - Option 1: For band n100, the PDCCH parameters with 12RB BW apply; for other bands, the PDCCH parameters with 15RB BW apply. - Option 2: RAN4 to define SSB based RLM requirements for only 12 PRBs.

| Channel BW | Transmission BW | Channels | Band n100 | Other bands |
| --- | --- | --- | --- | --- |
| 3MHz | 12 PRB | PBCH (SSB) Note: SSB is punctured from 20 PRB | 12 PRB | Not supported |
|  |  | PDCCH CORESET#0 Note: not punctured, new 12 PRB configuration | 12 PRB |  |
|  |  | PDCCH Dedicated CORESET | 12 PRB Note: only configurations 6 and 12 are supported |  |
|  | 15 PRB | PBCH (SSB) Note: SSB is punctured from 20 PRB | 12 PRB | 12 PRB |
|  |  | PDCCH CORESET#0 Note: 15 PRB CORESET#0 is punctured from 24 PRB | 15 PRB, 12 PRB | 15 PRB, 12 PRB |
|  |  | PDCCH Dedicated CORESET Note: May be revisited based on RAN1 agreement on puncturing support in dedicated coreset from 18 PRB | 12 PRB Note: only configurations 6 and 12 are supported | 12 PRB Note: only configurations 6 and 12, are supported |
| 5MHz | 20 PRB | PBCH (SSB) Note: Legacy SSB | 20 PRB | Not supported |
|  |  | PDCCH CORESET#0 Note: CORESET#0 is punctured from 24 PRB | 20 PRB |  |
|  |  | PDCCH Dedicated CORESET Note: May be revisited based on RAN1 agreement on puncturing support in dedicated coreset from 24 PRB | 18 PRB Note: only configurations 6, 12, 18 are supported |  |

Dedicated CORESET is highlighted in the above table. RLM is using dedicated CORESET, therefore. The configuration of dedicated CORESET currently only supports multiply of 6 configurations (e.g., 6, 12, 18). RAN1 has not agreed explicitly to support puncturing of 18 PRB configuration on dedicated CORESET. Therefore, only 12 PRB is currently supported for RLM for all bands.

For 3MHz, RAN4 to define SSB based RLM requirements for only 12 PRBs.

Sub-topic 1-5 PBCH transmission BW

RAN1 agreement: PBCH transmission BW is always 12 PRB. <Way forward>: Specify PBCH decoding requirements based on 12PRB PBCH.

As described in the Table 1, 12 PRB is always used for PBCH. Therefore RAN4 should  Proposal 1: Specify PBCH decoding requirements based on 12PRB PBCH.

Specify PBCH decoding requirements based on 12PRB PBCH.

Sub-topic 1-9 Applicability rule addition

<Way forward>: For a UE which supports only less than 5MHz CBW, discuss whether to add applicability rule of existing requirements for each of the requirements applicable for the less than 5 MHz UE.

RAN4 has been scoping out some of the features from < 5MHz support in rel-18. For completeness, RAN4 may add applicability rule under each section impacted. However, most of the requirements are not impacted by < 5MHz UE, and therefore it another approach would be to add a section for <5MHz which describes which sections are supported under rel-18 capability.

RAN4 to agree one of the options:

  - To have applicability rule under each section supported by rel-18

  - To have a section referring to the applicable requirements for rel-18.

  - To only address the requirements impacted by < 5MHz (as done currently)

Conclusion

In the paper, the following Observations and Proposals were made:

Proposal 1: For 3MHz, RAN4 to define SSB based RLM requirements for only 12 PRBs.

Proposal 2: Specify PBCH decoding requirements based on 12PRB PBCH.

Proposal 3: RAN4 to agree one of the options:

a.	To have applicability rule under each section supported by rel-18

b.	To have a section referring to the applicable requirements for rel-18.

c.	To only address the requirements impacted by < 5MHz (as done currently)