3GPP TSG-RAN WG4 Meeting #109-bis	R4-2319795

Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for 38.133 on Improvement on SCell/SCG setup delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-0 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Supporting the agreements made for on Improvement on SCell/SCG setup delay |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN4 #108bis Agreements: <Agreement>: Definition of ‘valid’ in solution based on existing measurement - The measurements are considered valid if both of the following conditions are satisfied - A) the measurement are performed within the last [X] seconds before it is reported - X value is network configured. Signalling details are up to RAN2 - FFS on the X value(s) and will be decided by RAN4 - If X is not defined then no requirements will be introduced - B) the reported measurement results satisfy measurement accuracy [at the measurement instance] - FFS on side conditions <Agreement>: Validity indication - Do not introduce a dedicated signalling for indication of measurements validity from UE to gNB RAN4 #108 Agreements: Measurement reporting <Agreement> - For UE which is not capable of EMR, when the measurement results during idle/inactive mode are available and valid, UE can report these results to the network during or after RRC setup or resume procedures. The reporting procedure is up to RAN2. <Agreement> - It is up to UE implementation how to select a band to report unless NW specify a band to report. Starting point of the enhanced measurement <Agreement> - For MT originating call, UE starts to perform additional measurement after paging reception. - And for MO call, UE starts to perform additional measurement after first RACH preamble transmission, i.e. Msg1. - Note: the respective agreement can be used to derive requirements once feasibility of enhanced solution is confirmed Ending point of the enhanced measurement <Agreement> - The ending point of enhanced measurement at the reception of the RRC CONNECTED mode measurement configuration (the 1st RRC_reconfiguration message) - Note: the respective agreement can be used to derive requirements once feasibility of enhanced solution is confirmed RAN4 #107 Agreements: N/A RAN4 #106bis-e Agreements: Agreements: - Candidate criteria for measurements validity definition - A) the measurement are performed within the last [X] seconds before it is reported - B) the reported measurement results satisfy measurement accuracy - C) variation of serving cell RSRP/RSRQ does not exceed [Y] dB - FFS whether a single or several criteria should be used for measurements validity definition. Agreements: - FFS whether to introduce indication of availability and/or validity status of measurement results from UE to NW into the [existing] EMR reports. RAN4 #106bis-e Agreements: Agreement - RAN4 shall focus on inter-band target cell in FR2. If final solution to be agreed can cover intra-band and FR1 without extra standardization effort, it is unnecessary to exclude these two scenarios. Agreement - UE is allowed to reuse existing measurement, including legacy measurement for cell re-selection and EMR. - UE is allowed to perform addition measurement starting from RRC connection setup/resume procedure. - RAN4 can continue discussion on the feasibility of doing additional measurement starting from RRC setup/resume, and requirements shall be defined if feasible solution is agreed. RAN4 #105 <Agreement>: - Enhancements on UE behavior in IDLE/INACTIVE mode are not in the scope - The following aspects can be further studied - Using measurement results obtained during IDLE/INACTIVE mode for measurements during RRC connection procedure - Note: enhancements on IDLE/INACTIVE mode measurement are not in the scope RAN4 #104-e <Agreement >: - Reduce the number of EMR carriers to be measured for improved measurement. - FFS: how to select the frequency layers for improved measurement, how many frequency layers to measure and which scenario that is applicable. <Agreement> - Agreement: DRX is not in use during RRC connection setup/resume procedure for enhanced measurement. The main focus of this CR is on [existing-measurements-solution-r18]. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_mob_enh2 WID objective 7 is not supported, hence incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change 1>>

## 4.x	Measurements for fast CA/DC setup

### 4.x.1 	Introduction

The requirements apply when A UE supporting [Existing-measurement-solution-r18] shall indicate availability of performed measurements on the CA and DC candidate frequencies/cells indicated by higher layers and meet the requirement specified in the corresponding clauses. The UE shall indicate idle mode measurements availability provided that the serving cell requests [Existing-measurement-solution-r18] measurement results for one or more carriers. The requirements apply to a configured carrier frequency and the serving cell are among the supported band combination of the UE.

### 4.x.2	Requirements applicability

The requirements in clause 6.x shall apply provided the

- UE has received a measurement configuration for [Existing-measurement-solution-r18] measurements and reporting.

- The detected cell is among the list of cells or on a carrier frequency provided for [Existing-measurement-solution-r18], and

- The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combinations of the UE.

### 4.x.3	Reporting requirements

UE supporting [Existing-measurement-solution-r18] shall report measurement availability, and measurement results according to [Clause TBD of TS 38.331].

UE supporting [Existing-measurement-solution-r18] is allowed to report measurements for early measurement reporting or cell reselection, by using [Existing-measurement-solution-r18] reporting configuration, given the reported carrier is configured for [Existing-measurement-solution-r18] measurements.

If the network configures more than 1 carrier for reporting, it is up to UE implementation to select which carriers to report.

A UE supporting [Existing-measurement-solution-r18] shall report measurements fulfilling the following criteria:

- At the time of reporting, samples associated with the measurements meeting the accuracy requirements, specified in 10.1.2B and 10.1.5B, is performed within the [RRC-time-X-r18] configured in TS 38.331.

The measurements shall be reported according to the reporting configuration provided by the higher layers.

Editor’s note: FFS for any side conditions

<< End of Change 1>>