3GPP TSG-RAN WG4 Meeting #109             	R4-2319837

Chicago, USA, 13th Nov-17th Nov, 2023

Agenda item:	8.12.3.2

Source:	Samsung

Title:	Simulation results for PDSCH with CA

Document for:	Discussion

- Introduction

In the last RAN4 meeting, all the remaining issues related with PDSCH requirement with CA was under discussed, the related agreement was made and captured in the WF [1]

In this contribution, the ideal and impairment simulation results are provided for requirement derivation.

# 2	Simulation results

In this section, the ideal and impairment results are provided based on agreed test cases listed in the R4-2313920 [2]

Table 2-1 ideal and impairment simulation results for PDSCH requirement with CA

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Ideal results) | SNR (dB) (Impairment results) |
| 1-1 | FRC 1-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.7 | 12.7 |
| 1-2 | FRC 1-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 11 | 13 |
| 1-3 | FRC 1-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 11.4 | 13.4 |
| 1-4 | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 11.5 | 13.5 |
| 2-1 | FRC 2-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.9 | 12.9 |
| 2-2 | FRC 2-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 11.2 | 13.2 |
| 2-3 | FRC 2-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 11.5 | 13.5 |
| 2-4 | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 11.6 | 13.6 |

# 3	Conclusion

In this contribution, the ideal and impairment simulation results are provided for requirement derivation.

# References

R4-2317007   WF on demodulation requirements for Rel-18 FR2 HST    Samsung

R4-2313920    Simulation assumption for FR2 HST demodulation        Samsung