3GPP TSG-RAN WG4 Meeting #109              	R4-2319844

Chicago, USA, 13th Nov-17th Nov, 2023

Agenda item:	8.14.6.2

Source:	Samsung

Title:	Discussion and initial results for BS demodulation requirement for less than 5MHz

Document for:     Discussion

- Introduction

In the last RAN4 meeting, the test scope of performance part of NR support for dedicated spectrum less than 5MHz for FR1 was under discussion. The related agreement was captured in the WF [1] as following

- BS demodulation requirement - PUSCH requirement - Further discuss the scope of PUSCH requirements - FFS whether to introduce full set of Rel-15 requirements for PUSCH - PUSCH with precoding disabled - PUSCH with precoding enabled - UCI multiplexed on PUSCH - FFS the limited set of requirements for PUSCH for BS supporting multiple CBWs (i.e., less and more than 5MHz) - FFS on applicability rules for PUSCH depending on the supported BW - Performance evaluation - Use the following parameters as a starting point for performance evaluation: - PUSCH with precoding disabled (CP-OFDM) - Use parameters from Rel-15 5MHz CBW requirements as a baseline by updating the CBW to 3MHz: - Number of PRBs: 12 - MCS: 16 - 1T2R, 1 layer - Note: Other parameters for simulations and requirements are not precluded. - PUSCH requirements in HST conditions - FFS - Which test to consider for HST conditions in less than 5MHz CBW, e.g. - UL timing adjustment - PUSCH for high-speed 350km/h and/or (500km/h speed) - Which propagation conditions to consider for HST scenario (500km/h) - Option 1: The maximum Doppler - Option 2: TDLC 300-600 - Other options are not precluded - PUCCH requirement - Performance evaluation/simulation - Use the following PUCCH formats and parameters as a starting point - Format 0: UCI bits =1; RB=1; OFDM symbols=1,2 - Format 1: UCI bits =2; RB =1, OFDM symbols=1; - Format 2: UCI bits =4 (when OFDM symbols =1), 22 (when OFDM symbols= 2), RB=4 (when OFDM symbols =1), 9 (when OFDM symbols =2) - Format 3: UCI bits =16 - Introduction of requirements - Based on the performance requirements - Introduce new requirements for all formats/cases if the performance difference is observed in any of the formats - FFS on applicability rules for PUCCH depending on the supported BW - PRACH - Introduction of requirement - No new PRACH requirement need to be introduced for less than 5MHz channel bandwidth - FFS whether applicability rule of note for long PRACH sequence in less than 5MHz channel bandwidths are needed.

In this contribution, the view on remaining test scope of BS demodulation requirement for NR less than 5MHz.

- Test scope of BS demodulation requirement

PUSCH requirement

In the last meeting, the test scope of BS demodulation requirement with less than 5MHz was under discussion.

Based on the WID, 3MHz channel bandwidth was introduced in bands n100, n8, n26 and n28, support different deployment scenarios, in order to be able to start the migration from GSM-R to FRMCS in Europe. And RAN4 will specify necessary UE/BS performance requirements for NR operation in dedicated spectrum allocations from approximately 3MHz up to below 5MHz. Since the 3MHz is targeting for special deployment to support rail communication, it is necessary to introduce the PUSCH requirements with 3MHz for FDD in FR1 to meet the test coverage. Given there is no any physical layer enhancement, the limited test cases of PUSCH requirements with 3MHz for 15KHz SCS could be introduced.

For BS side, the requirements defined is manufacture declaration based. BS vendor can declare to support single CBW or multiple CBWs. For BS supporting only less than 5MHz, based on current test applicability rule defined in Rel-15, there is no BS requirement can be tested. Therefore, the basic requirements introduced in Rel-15, including PUSCH requirement with precoding disabled and PUSCH requirement with precoding enabled.

For UCI multiplex on PUSCH, only the typical BW with 30KHz SCS and 10MHz was selected for requirement definition in Rel-15, to reduce the test effort. Since the basic requirement with less than 5MHz were considered, there is no necessary to introduce UCI multiplex on PUSCH with less than 5MHz.

For BS supporting multiple CBWs including less than 5MHz, to guarantee the requirement of less than 5MHz can be tested and the existing applicability rule be reused, the dedicated PUSCH requirement with less than 5MHz CBW can be introduced.

Proposal 1: RAN4 only define PUSCH requirements with CP-OFDM waveform and DFT-s-OFDM for 3MHz CBWs if BS supporting only less than 5MHz CBW

Proposal 2: RAN4 define PUSCH requirements with CP-OFDM waveform and DFT-s-OFDM for 3MHz CBWs if BS supporting multiple CBWs including less than 5MHz

PUSCH requirement in HST

Based on RAN-P conclusion as following,

RP-230781 Response to LS from RAN4 on spectrum less than 5 MHz RAN Plenary would like to thank RAN WG4 for their LS for spectrum less than 5 MHz. RAN Plenary has discussed question 1 on legacy bands and UE operation, and concluded the following: - In some bands where the <5MHz feature is planned to be deployed there may be legacy NR UEs, whereas in others there are no legacy NR UEs. - In order to limit the impact to any legacy UEs in the same frequency range, it would be helpful if the sync raster can be differentiated for the less-than-5MHz channels. - It is assumed that UE support of the <5MHz feature is band-specific and optional. RAN Plenary has discussed question 2 on the feature list to be considered, and concluded that the less-than-5MHz WI in Rel-18 should consider single-carrier operation, excluding RedCap.In addition, UE speeds up to 500km/h should be targeted for band n100 without impact to RAN1

UE supporting 3MHz, would be supported high speed up to 500km/h, where the targeting band is n100. Therefore, the PUSCH with high doppler scenario can be considered.

Regarding the test scope of HST condition, both UL timing adjustment and PUSCH with 350km/h and 500km/h were introduced in Rel-16 NR HST WI. Since the UE speed will be up to 500km/h, if the BS can pass the requirement with 500km/h, it should no issue to pass the requirement with 300km/h. Therefore, the UL timing adjustment and PUSCH requirements with related with UE speed 500km/h can be introduced.

Proposal 3: UL timing adjustment and PUSCH requirements with related with UE speed 500km/h could be introduced.

PUCCH requirement

The agreement was made in RAN1 for PUCCH as following

- No enhancements are needed for PUCCH to support transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth - FFS the necessity for PUCCH FH disabling

Regarding whether to introduce PUCCH requirements, there is no conclusion yet. Although the number of payload size is not related with channel bandwidth directly, the performance will be different for different channel bandwidths due to the frequency hopping operation. Given there is no conclusion whether to disable the PUCCH frequency hopping operation. PUCCH requirements with frequency hopping for 3MHz channel bandwidth can be introduced. Given that the initial simulation assumption was made in the last meeting for PUCCH performance comparison between 3MHz and 5MHz, we are open to further discuss pending on the simulation results.  Considering there is no physical layer enhancement, if agreed to introduce PUCCH requirement with 3MHz, we prefer to consider the limited test cases of PUCCH requirements to reduce the test effort.

Similar as PUSCH, the requirements defined is manufacture declaration based. For BS supporting only less than 5MHz, or BS supporting multiple bands including less than 5MHz, if no PUCCH requirement with 3MHz introduced, the related test configuration with 5MHz should be modified to support PUCCH requirement with 3MHz test, considering the PRB location will be different based on the frequency hopping pattern due to the maximum number of PRB in 3MHz and 5MHz, where the value of first PRB after frequency hopping should be updated with a note as following

| First PRB after frequency hopping1 | The largest PRB index – (Number of PRBs – 1) |
| --- | --- |
| Note1: - The largest PRB index is 11 for 3MHz test at n100 - The largest PRB index is 14 for 3MHz test except for n100 - The largest PRB index is 24 for 5MHz test |  |

Proposal 4: If agreed to introduce PUCCH requirements with 3MHz, only limited test cases of PUCCH requirement are considered.

Proposal 5: If no PUCCH requirements with 3MHz introduced, the test configuration with 5MHz should be modified to support PUCCH requirement with 3MHz test, where the value of first PRB after frequency hopping should be updated with a note as following

| First PRB after frequency hopping1 | The largest PRB index – (Number of PRBs – 1) |
| --- | --- |
| Note1: - The largest PRB index is 11 for 3MHz test at n100 - The largest PRB index is 14 for 3MHz test except for n100 - The largest PRB index is 24 for 5MHz test |  |

PRACH requirement

Based on RAN1 conclusion, there is no enhancements for PRACH to operate NR on transmission bandwidths of < 5MHz for 3MHz and 5MHz channel bandwidth, which means existing PRACH formats with 15KHz SCS, and long PRACH formats with 1.25KHz SCS can support 3MHz.

As agreed in the last meeting, no new PRACH requirement need to be introduced for less than 5MHz channel bandwidth. The only remaining open issue is that whether applicability rule or note for long RACH sequence in less than 5MHz bandwidths are needed.

In general, the PRACH requirement is no differentiate with transmission BWs if its configuration can be fitted into the transmission BW. In Rel-15, both long PRACH formats and short formats requirements are introduced. And short formats requirements with sequence length as LRA=1151 for 15KHz SCS was introduced in Rel-16, where the main targeting BW should be 20 MHz channel bandwidth.

Based on the RAN4 RF discussion, the minimum number of RB as 15 for 3MHz was introduced excepting for n100, and the minimum number of RB as 12 for 3MHz with n100. Then, the available number of subcarriers is 180 for 15RB, and 144 for 12RB, respectively, where the existing long PRACH format with LRA=839 and short PRACH formats with LRA=139 can be fitted into the transmission BW of 3MHz, while the existing short PRACH formats with LRA=1151 cannot be fitted into the transmission BW of 3MHz.

Observation 1: Existing long PRACH formats with 1.25KHz SCS with sequence length LRA=839, and long PRACH formats with 15KHz SCS with sequence length LRA=139 can support 3MHz channel bandwidth.

Observation 2: Existing PRACH formats with 15KHz SCS with sequence length LRA=1151, cannot apply for 3MHz channel bandwidth.

Therefore, not all the PRACH requirements defined in existing spec can be applied for 3MHz channel bandwidth. It was also captured in RAN1 agreement as

- Note: PRACH formats and configurations not fitting into transmission BW are not applicable.

Observation 3: A note is added to indicate PRACH formats and configurations are not fitting into transmission BW are not applicable in RAN1 agreement

So, adding a note to indicate the existing short PRACH formats with 15KHz SCS with sequence length LRA=1151 can not apply for 3MHz channel bandwidth may be more clearly in RAN4 specification, similar as RAN1 note. While, we are open to further discuss whether it is necessary.

Proposal 6: FFS to add a note indicate the existing short PRACH formats with 15KHz SCS with sequence length LRA=1151 cannot apply for 3MHz channel bandwidth

- Test setup of BS demodulation requirement

In this section, the view on test setup for BS demodulation requirement is provided, including the PUSCH and PUCCH requirement, as analysed in section 2.

## 3.1	Test setup of PUSCH requirements

Channel Model

Based on RAN-P conclusion, UE supporting 3MHz, would be supported high speed up to 500km/h. From RRM aspect, the channel model used for specifying RRM requirement was under discussed. The following agreement was made

Agreement: Use single tap channel model at least for band n100. FFS for other bands. FFS on other channel models for other bands. Way forward: Other channel models - Proposals - Option 1: AWGN - Option 2: TDL-A 30ns - Option 3: TDL-B 100ns - Option 4: TDL-C 300ns

From demod aspect, we think the fading channel should be enough, such as TDL channels in current requirements for PUSCH with UE normal speed.

For HST condition, as agreed, single tap channel model at least for band n100, therefore, single tap channel model can be used for PUSCH demodulation requirement under HST condition.

Proposal 7: TDL channel model could be considered for specifying BS demodulation requirement. Single tap can be considered for specifying BS modulation requirement under HST condition.

Maximum Doppler

For UE with normal speed, in Rel-15, several values were considered for specifying the related PUSCH requirements, with considering the test coverage. For 3MHz, we prefer to reuse the existing doppler value as 100Hz for performance definition.

For HST condition, since UE speeds up to 500km/h should be targeted for band n100, where the carrier frequency of band n100 is 880MHz, then, the maximum Doppler shall be around 815Hz for Uplink. In Rel-16 NR HST WI, there is a targeting HST deployment scenario, such Ds and Dmin, as well as the transmission scheme, where the related doppler shift trajectory can be calculated accordingly. Given there is no clear HST deployment modeling for rail communication, how to derive the doppler shift trajectory should be further discussed. From demodulation requirement aspect, the performance difference is expected to be minor with different Ds and Dmin. To simplify the test setup, the existing Ds (Ds=700m) and Dmin (Dmin=100m) in Rel-16 FR1 NR HST can be reused as a starting point to derive the Doppler shift trajectory

Proposal 8: Existing Doppler value as 100Hz can be considered for PUSCH requirement with normal UE speed

Proposal 9: The maximum Doppler is 815Hz assuming UE with 500km/h speed if HST single tap is introduced. The existing Ds (Ds=700m) and Dmin (Dmin=100m) in Rel-16 FR1 NR HST deployment can be reused as a starting point to derive the Doppler shift trajectory.

Antenna configuration

Regarding the antenna configuration, since RAN4 has introduced the test applicability rule for different antenna configuration, as preferred, since there is no any physical layer enhancement for PUSCH with 3MHz, we prefer to only consider 1Tx2Rx for specifying PUSCH demodulation with 3MHz

Proposal 10: Specify BS demodulation requirement with 1T2Rx only

PUSCH mapping type

Both PUSCH mapping type A and type B were considered for PUSCH requirement with larger than 5MHz in Rel-15. For NR less than 3MHz, we would like to consider only PUSCH mapping type A, we are also open to further discuss the necessity of PUSCH mapping type B, considering the related test applicability rule has introduced in RAN4.

Proposal 11: Specify PUSCH requirement with DMRS mapping type A

Number of DMRS symbols

The number of DMRS symbols pending on the maximum doppler value. For normal speed scenario, 2 DMRS should be the typical configuration.

Meanwhile, since UE speeds up to 500km/h should be targeted for band n100, where the maximum Doppler shall be around 815Hz for Uplink. 3 DMRS symbols can be configured if HST single tap with 815Hz doppler frequency was introduced.

Proposal 12: Specify PUSCH requirement with 2 DMRS symbols for normal UE speed and Specify PUSCH with 3DMRS if HST single tap is introduced

Waveform

Regarding the waveform, as mentioned in the test scope in section 2, the PUSCH requirement with both CP-OFDM and DFT-s-OFDM could be considered for 3MHz.

For HST requirement, similar as Rel-16 NR HST, only PUSCH requirement with CP-OFDM will be considered

Proposal 13: Specify PUSCH requirement with CP-OFDM and DFT-s-OFDM waveform for normal UE speed

Proposal 14: Specify PUSCH requirement with CP-OFDM for HST if HST single tap is introduced

MCS

Regarding MCS, MCS2, MCS16 and MCS 20 are considered for PUSCH requirements with different channel bandwidths to guarantee the test coverage of MCS.

We would like to reuse as a starting point for specifying PUSCH requirement with 3MHz. We are also open to further discuss whether down selection one of them for PUSCH requirement

Proposal 15: The following MCS could be considered as a starting point for specifying PUSCH requirement

- MCS 2, MCS 16 and MCS 20

- FFS on down selection one of MCS for requirement

Other

For other test parameters, existing PUSCH test parameters can be reused.

Proposal 16: Reusing existing PUSCH test parameters for specifying PUSCH requirement with 3MHz under normal UE speed

Table 1:  Test parameters for PUSCH requirement with normal UE speed

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled and Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | [Pos1] |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | [0] |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

Table 2:  Test parameters for PUSCH requirement with UE 500km/h speed if introduced

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | [Pos2] |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | [0] |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

## 3.2	Test setup of PUCCH requirements

In Rel-15, the PUCCH requirements with all the formats were considered for different CBWs. If PUCCH requirement with 3MHz introduced, the same PUCCH formats can be considered to specify the related PUCCH requirement.

Antenna configuration

Regarding the antenna configuration, similar as PUSCH, we prefer to only consider 1Tx2Rx for specifying PUCCH demodulation with 3MHz.

For other test parameters for PUCCH formats, the same value can be reused for requirement definition.

Proposal 17: Introduce PUCCH requirement with 3MHz under 1Tx2Rx antenna configuration.

Proposal 18: Reusing the existing PUCCH test parameters for specifying PUCCH requirement with 3MHz

Table 3:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 4:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | NACK to ACK probability ACK missed detection probability |

Table 5:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 6:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 7:  Test parameters of PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Test metric | BLER |  |

Table 8:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER |

- Simulation results

In this section, the initial simulation results are provided for evaluation the performance with less than 5MHz.

| Test case | SCS&BW | RB | MCS | Channel | Mapping type | Length of symbol | Number of Tx and Rx | Number of DMRS | SNR@70% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 15KHz, 3MHz | 12 | 16 | TDLC300-100 | A | 14 | 1T2R | 1+1 (2,11) | 8.005 |
| Case 2 | 15KHz, 3MHz | 15 | 16 | TDLC300-100 | A | 14 | 1T2R | 1+1 (2,11) | 7.854 |
| Case 3 | 15KHz, 5MHz | 25 | 16 | TDLC300-100 | A | 14 | 1T2R | 1+1 (2,11) | 8.128 |

Observation 4:  Similar performance can be achieved with 3MHz and 5MHz CBW

- Conclusion

In this paper, the view on the test scope and test setup of BS demodulation requirement to support less than 3MHz was provided.

Proposal 1: RAN4 only define PUSCH requirements with CP-OFDM waveform and DFT-s-OFDM for 3MHz CBWs if BS supporting only less than 5MHz CBW

Proposal 2: RAN4 define PUSCH requirements with CP-OFDM waveform and DFT-s-OFDM for 3MHz CBWs if BS supporting multiple CBWs including less than 5MHz

Proposal 3: UL timing adjustment and PUSCH requirements with related with UE speed 500km/h could be introduced.

Proposal 4: If agreed to introduce PUCCH requirements with 3MHz, only limited test cases of PUCCH requirement are considered.

Proposal 5: If no PUCCH requirements with 3MHz introduced, the test configuration with 5MHz should be modified to support PUCCH requirement with 3MHz test, where the value of first PRB after frequency hopping should be updated with a note as following

| First PRB after frequency hopping1 | The largest PRB index – (Number of PRBs – 1) |
| --- | --- |
| Note1: - The largest PRB index is 11 for 3MHz test at n100 - The largest PRB index is 14 for 3MHz test except for n100 - The largest PRB index is 24 for 5MHz test |  |

Observation 1: Existing long PRACH formats with 1.25KHz SCS with sequence length LRA=839, and long PRACH formats with 15KHz SCS with sequence length LRA=139 can support 3MHz channel bandwidth.

Observation 2: Existing PRACH formats with 15KHz SCS with sequence length LRA=1151, cannot apply for 3MHz channel bandwidth.

Observation 3: A note is added to indicate PRACH formats and configurations are not fitting into transmission BW are not applicable in RAN1 agreement

Proposal 6: FFS to add a note indicate the existing short PRACH formats with 15KHz SCS with sequence length LRA=1151 cannot apply for 3MHz channel bandwidth

Proposal 7: TDL channel model could be considered for specifying BS demodulation requirement. Single tap can be considered for specifying BS modulation requirement under HST condition.

Proposal 8: Existing Doppler value as 100Hz can be considered for PUSCH requirement with normal UE speed

Proposal 9: The maximum Doppler is 815Hz assuming UE with 500km/h speed if HST single tap is introduced. The existing Ds (Ds=700m) and Dmin (Dmin=100m) in Rel-16 FR1 NR HST deployment can be reused as a starting point to derive the Doppler shift trajectory.

Proposal 10: Specify BS demodulation requirement with 1T2Rx only

Proposal 11: Specify PUSCH requirement with DMRS mapping type A

Proposal 12: Specify PUSCH requirement with 2 DMRS symbols for normal UE speed and Specify PUSCH with 3DMRS if HST single tap is introduced

Proposal 13: Specify PUSCH requirement with CP-OFDM and DFT-s-OFDM waveform for normal UE speed

Proposal 14: Specify PUSCH requirement with CP-OFDM for HST if HST single tap is introduced

Proposal 15: The following MCS could be considered as a starting point for specifying PUSCH requirement

- MCS 2, MCS 16 and MCS 20

- FFS on down selection one of MCS for requirement

Proposal 16: Reusing existing PUSCH test parameters for specifying PUSCH requirement with 3MHz under normal UE speed

Table 1:  Test parameters for PUSCH requirement with normal UE speed

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled and Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | [Pos1] |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | [0] |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

Table 2:  Test parameters for PUSCH requirement with UE 500km/h speed if introduced

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | [Pos2] |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | [0] |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

Proposal 17: Introduce PUCCH requirement with 3MHz under 1Tx2Rx antenna configuration.

Proposal 18: Reusing the existing PUCCH test parameters for specifying PUCCH requirement with 3MHz

Table 3:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 4:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | NACK to ACK probability ACK missed detection probability |

Table 5:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK probability ACK missed detection probability |

Table 6:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 7:  Test parameters of PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Test metric | BLER |  |

Table 8:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER |

Observation 4:  Similar performance can be achieved with 3MHz and 5MHz CBW

# References

R4-2316924, WF on NR_FR1_lessthan_5MHz_BW_demod