3GPP TSG-RAN WG4 Meeting #109              	R4- 2319848

Chicago, USA, 13th Nov-17th Nov, 2023

Agenda item:	6.2.4.1.4

Source:	Samsung

Title:	Simulation results for eMTC and NB-IoT over NTN

Document for:	Discussion

- Introduction

In this contribution, the updated simulation results are provided for alignment and performance requirement derivation.

# 2	Discussion

## 2.1	SAN demodulation requirements derivation

Based on the simulation results summary submitted in RAN4# 108, large performance gap was observed.  Considering the complete deadline for this WI, additional margin was added for requirement. Based on chairman note, companies can further provide simulation results to align the results and update the requirements accordingly for NB-IoT/eMTC SAN demodulation requirement

- Session chair note: Companies can further provide simulation results to align the results and update requirements accordingly for NB-IoT/eMTC SAN demodulation requirements.

Although additional margin is added for requirement definition, the gap between companies is still very large

The similar situation will be expected to happen in the following IoT NTN enhancement. To avoid the misalignment, companies are encouraged to select the simplified test parameters to further alignment.

For IoT NTN, additional timing offset and frequency offset are introduced, expecting for new introduced NTN channel. For alignment purpose, the additional timing offset and frequency offset can be set as 0 in the first stage.

Proposal 1: Companies are encouraged to select the simplified test parameters to further alignment

# 3	Simulation results for IoT over NTN

Based on agreed simulation assumption, the updated simulation results are provided for IoT over NTN for alignment and performance requirement derivation

## 3.1	eMTC over NTN

In this section, the initial results for eMTC PUSCH CE mode A, CE mode B, PUCCH and PRACH. The detail simulation assumptions for them are indicated in the following tables

Table 3:  simulation assumption for eMTC PUSCH CE mode A

| Parameters | CE Mode A |
| --- | --- |
| Max number of HARQ transmissions | 4 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 |
| Number of PUSCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLA100-5 Option 2: NTN-TDLC5-5 |
| FRC | TS36.104 A3-2 |
| System BW | 1.4MHz only |
| Doppler shift offset | 4Hz |
| Transmit timing offset | [0.01] us per subframe |
| Fraction of maximum throughput | 70% |

Table 4:  simulation assumption for eMTC PUSCH CE mode B

| Parameters | CE Mode B |
| --- | --- |
| Max number of HARQ transmissions | 2 |
| RV sequences | 0,0,0,0,2,2,2,2, 3,3,3,3,1,1,1,1 |
| Number of PUSCH repetitions | 256 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLA100-5 Option 2: NTN-TDLC5-5 |
| FRC | TS36.104 A3-1 |
| System BW | 1.4MHz only |
| Doppler shift offset | 128 |
| Transmit timing offset | [0.01] us per subframe |
| Fraction of maximum throughput | 70% |

Table 5:  simulation assumption for eMTC PUCCH

| Parameters | Values |
| --- | --- |
| Number of PUCCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | NTN-TDLA100-5 |
| System BW | 1.4MHz |
| ACK missed detection probability | 1% |

Table 6:  simulation assumption for eMTC PRACH

| Parameters | Values |  |
| --- | --- | --- |
| Formats | Format 0/1/2/3 |  |
| Number of PRACH repetitions | Format 0 | 8 |
|  | Format 1 | 8 |
|  | Format 2 | 16 |
|  | Format 3 | 16 |
| Frequency hopping | OFF |  |
| Duplex mode | FDD only |  |
| Number of Tx antennas | 1 |  |
| Number of Rx antennas | 1 and 2 |  |
| Propagation channel | NTN-TDLA100-5 |  |
| System BW | 1.4MHz |  |
| Missed detection probability | 1% |  |
| False alarm | 0.1% |  |

eMTC CE mode A and CE mode B

Table 7:  Ideal and impairment results for eMTC CE mode A and B

| Case | Antenna configuration | Rep | Channel | CE mode | Additional Doppler | Additional Timing offset | FRC | SNR@70% Ideal | SNR@70% Impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T1R | 8 | NTN TDLAC5-5 | A | 4Hz | [0.01:0.01:0.08]us | 36.104 A3-2 | -7.2 | -5.2 |
| 2 | 1T1R | 8 | NTN TDLA100-5 | A | 4Hz | [0.01:0.01:0.08]us | 36.104 A3-2 | -4.5 | -2.5 |
| 3 | 1T2R | 8 | NTN TDLAC5-5 | A | 4Hz | [0.01:0.01:0.08]us | 36.104 A3-2 | -10.3 | -8.3 |
| 4 | 1T2R | 8 | NTN TDLA100-5 | A | 4Hz | [0.01:0.01:0.08]us | 36.104 A3-2 | -8.8 | -6.8 |
| 5 | 1T1R | 256 | NTN TDLAC5-5 | B | 128Hz | [0.01:0.01:2.56]us | 36.104 A3-1 | -14.1 | -12.1 |
| 6 | 1T1R | 256 | NTN TDLA100-5 | B | 128Hz | [0.01:0.01:2.56]us | 36.104 A3-1 | -12.4 | -10.4 |
| 7 | 1T2R | 256 | NTN TDLAC5-5 | B | 128Hz | [0.01:0.01:2.56]us | 36.104 A3-1 | -16.4 | -14.4 |
| 8 | 1T2R | 256 | NTN TDLA100-5 | B | 128Hz | [0.01:0.01:2.56]us | 36.104 A3-1 | -15.4 | -13.4 |

eMTC PUCCH format 1

Table 8:  Ideal and impairment results for eMTC PUCCH format 1

| Case | Format | Frequency hopping | Antenna configuration | BW | Repetition | Channel | Additional Doppler | Additional Timing offsets | SNR@1% ACK missed | SNR@70% Impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 1a | Off | 1T1R | 1.4MHz | 8 | NTN-TDLA100-5 | 4Hz | [0.01:0.01:0.08]us | -3.6 | -1.6 |
| 2 | Format 1a | off | 1T2R | 1.4MHz | 8 | NTN-TDLA100-5 | 4Hz | [0.01:0.01:0.08]us | -9.8 | -7.8 |

eMTC PRACH

Table 9:  Ideal and impairment results for eMTC PRACH

| Case |  | Format | Antenna configuration | Repetition | Channel | Frequency offset | Timing estimation error | 1% MD ideal | 1% MD impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 0 |  | 1T1R | 8 | NTN-TDLA100-5 | 270Hz | 2.08us | -12.0 | -10.0 |
| 2 | Format 0 |  | 1T2R | 8 | NTN-TDLA100-5 | 270Hz | 2.08us | -17.8 | -15.8 |
| 3 | Format 1 |  | 1T1R | 8 | NTN-TDLA100-5 | 270Hz | 2.08us | -11.4 | -9.4 |
| 4 | Format 1 |  | 1T2R | 8 | NTN-TDLA100-5 | 270Hz | 2.08us | -17.2 | -15.2 |
| 5 | Format 2 |  | 1T1R | 16 | NTN-TDLA100-5 | 270Hz | 2.08us | -17.8 | -15.8 |
| 6 | Format 2 |  | 1T2R | 16 | NTN-TDLA100-5 | 270Hz | 2.08us | -22.7 | -20.7 |
| 7 | Format 3 |  | 1T1R | 16 | NTN-TDLA100-5 | 270Hz | 2.08us | -17.4 | -15.4 |
| 8 | Format 3 |  | 1T2R | 16 | NTN-TDLA100-5 | 270Hz | 2.08us | -22.3 | -20.3 |

## 3.2	NB-IoT over NTN

In this section, similar as eMTC over NTN, the simulation results for NPUSCH1/2 and NPRACH are provided.

NPUSCH format 1

Table 10:  Ideal and impairment results for NPUSCH1

| Case | Antenna configuration | Repetition | Channel | Tone | SCS | Additional Doppler | Additional Timing offset | FRC | SNR@70% ideal | SNR@70% Impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T1R | 4 | NTN TDLC5-1 | 1 | 3.75KHz | 128 | [0.32:0.32:2.56] | A16-1 | -5.5 | -3.5 |
| 2 | 1T1R | 4 | NTN TDLA100-1 | 1 | 3.75KHz | 128 | [0.32:0.32:2.56] | A16-1 | -3.3 | -1.3 |
| 3 | 1T2R | 4 | NTN TDLC5-1 | 1 | 3.75KHz | 128 | [0.32:0.32:2.56] | A16-1 | -8.0 | -6.0 |
| 4 | 1T2R | 4 | NTN TDLA100-1 | 1 | 3.75KHz | 128 | [0.32:0.32:2.56] | A16-1 | -6.6 | -4.6 |
| 5 | 1T1R | 16 | NTN TDLC5-1 | 12 | 15KHz | 8 | [0.01:0.01:0.16] | A16-5 | -8.4 | -6.4 |
| 6 | 1T1R | 16 | NTN TDLA100-1 | 12 | 15KHz | 8 | [0.01:0.01:0.16] | A16-5 | -6.0 | -4.0 |
| 7 | 1T2R | 16 | NTN TDLC5-1 | 12 | 15KHz | 8 | [0.01:0.01:0.16] | A16-5 | -11.4 | -9.4 |
| 8 | 1T2R | 16 | NTN TDLA100-1 | 12 | 15KHz | 8 | [0.01:0.01:0.16] | A16-5 | -10.3 | -8.3 |

NPUSCH format 2

Table 11:  Ideal and impairment results for NPUSCH2

| Case | Antenna configuration | Repetition | Channel | Tone | SCS | Additional Doppler | Additional Timing offset | SNR@70% ideal | SNR@70% Impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1T1R | 16 | NTN TDLA100-1 | 1 | 3.75KHz | 64Hz | [0.08:0.08:1.28] | 0.84 | 2.84 |
| 2 | 1T2R | 16 | NTN TDLA100-1 | 1 | 3.75KHz | 64Hz | [0.08:0.08:1.28] | -4.91 | -2.91 |
| 3 | 1T1R | 16 | NTN TDLA100-1 | 1 | 15KHz | 16Hz | [0.02:0.02:0.32] | 3.74 | 5.74 |
| 4 | 1T2R | 16 | NTN TDLA100-1 | 1 | 15KHz | 16Hz | [0.02:0.02:0.32] | -4.8 | -2.8 |

NPRACH

Table 12:  Ideal and impairment results for NPRACH

| Case |  | Format | Antenna configuration | Repetition | Channel | Frequency offset | Timing estimation error | 1% MD ideal | 1% MD impair |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 0 |  | 1T1R | 8 | NTN-TDLA100-1 | 200Hz | 3.646us | 12.4 | 14.4 |
| 2 | Format 0 |  | 1T2R | 8 | NTN-TDLA100-1 | 200Hz | 3.646us | 3.2 | 5.2 |
| 3 | Format 1 |  | 1T1R | 8 | NTN-TDLA100-1 | 200Hz | 3.646us | 12.8 | 14.8 |
| 4 | Format 1 |  | 1T2R | 8 | NTN-TDLA100-1 | 200Hz | 3.646us | 3.5 | 5.5 |
| 5 | Format 0 |  | 1T1R | 16 | NTN-TDLA100-1 | 200Hz | 3.646us | 8.5 | 10.5 |
| 6 | Format 0 |  | 1T2R | 16 | NTN-TDLA100-1 | 200Hz | 3.646us | 1.0 | 3.0 |
| 7 | Format 1 |  | 1T1R | 16 | NTN-TDLA100-1 | 200Hz | 3.646us | 8.3 | 10.3 |
| 8 | Format 1 |  | 1T2R | 16 | NTN-TDLA100-1 | 200Hz | 3.646us | 1.0 | 3.0 |

# 4	Conclusion

In this contribution, the ideal and impairment results are provided for alignment and performance requirement derivation

Proposal 1: Companies are encouraged to select the simplified test parameters to further alignment

# References

R4-2309827 WF on SAN demodulation requirements for IoT over NTN