3GPP TSG-RAN WG4 Meeting # 109                               R4-2319855

Chicago, US, November 13 – November 17, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on MSD for CA_n34A-n40A_BCS4 and 5

Agenda Item:	7.1.1.2

Document for:	Approval

Introduction

In last meeting, a WF [1] was approved for further evaluation of MSD for CA_n34A-n40A_BCS4 and 5. CA_n34A-n40A_BCS0 has been introduced since Rel-17 without MSD definition.

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n34A-n40A | CA_n34A-n40A | n34 | 5, 10, 15 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |

In Rel-18, new BCS 4 and 5 for CA_n34A-n40A was requested with 100MHz n40. For this new larger channel bandwidth in band n40, it may bring MSD. In this paper, we’d like to provide our evaluation.

Discussion

The RF architecture for CA_n34-n40 is proposed below.

Figure 1 The RF architecture for CA_n34-n40

The cross-band isolation MSD of Band n34 and Band n40 is evaluated using the following parameters:

- PC3 operation, Tx power level: 23 dBm

- Post-PA insertion losses: 4 dB

- Primary antenna to diversity antenna isolation: 10 dB

- MRC combining and -1 dB Signal-to-Noise Ratio (SNR)

- Band n34 Tx filter rejection at n40 DL frequency range: 30 dB

- Band n40 Tx filter rejection at n34 DL frequency range: 33 dB

- Band n34 PA + RF transceiver noise floor at n40 Rx frequency range: -125 dBm/Hz

- Band n40 PA ACLR3: 53dB.

We can get the following MSD evaluation:

Table 1 MSD evaluation for DL band n40 with UL aggressor band n34

|  | parameters | n40 main path | n40 diversity path |
| --- | --- | --- | --- |
| transmit power for B34, dBm |  | 23 | 23 |
| B34 Diplexer loss, dB | 1 |  |  |
| Diplexer isolation at B34 uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| B34 receival singal at ANT port, dBm |  | 14 | 3 |
| n40 diplexer loss, dB | 1 |  |  |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n40 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -98.4 | -98.4 |
| B34 PA noise PSD at PA output port, dBm/Hz | -125 |  |  |
| B34 PA noise PSD at PA output port, dBm |  | -58.4 |  |
| B34 Tx filter rejection at 2300~2400MHz, dB | 30 |  |  |
| B34 PA noise power at ant port at 2300~2400MHz, dBm |  | -98.4 | -108.4 |
| Total noise level at ANT port |  | -94.5 | -98.0 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(10MHz BW) |  | -95.5 | -98.0 |
| combined REFSENS(10MHz BW), dBm | -100 |  |  |
| MSD | 0.0 |  |  |

Table 2 MSD evaluation for DL band n34 with UL aggressor band n40

|  | parameters | n34 main path | n34 diversity path |
| --- | --- | --- | --- |
| transmit power for n40, dBm |  | 23 | 23 |
| n40 Diplexer loss, dB | 1 |  |  |
| Diplexer isolation at B34 uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| B34 receival singal at ANT port, dBm |  | 14 | 3 |
| n40 diplexer loss, dB | 1 |  |  |
| n34 Rx filter rejection at 2300~2400MHz, dB | 30 |  |  |
| signal After n1 filter, dBm |  | -17 | -28 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -84 | -106 |
| n40 front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -80 | -102 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n34 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -98.4 | -98.4 |
| n40 UL CBW, MHz | 100 |  |  |
| n40 PA 3nd adjacent channel rejection at PA output port, dB | 53 |  |  |
| n40 PA 3nd adjacent channel interefence level at PA output port, dBm |  | -26 |  |
| n40 PA 3nd adjacent channel interference level at PA output port, dBm/RxBW |  | -39.4 |  |
| n40 Tx filter rejection at 2010~2015MHz, dB | 33 |  |  |
| n40 PA 2nd adjacent channel interference level at ant port at 2110~2170MHz, dBm |  | -82.4 | -92.4 |
| Total noise level at ANT port |  | -78.0 | -91.1 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(10MHz BW) |  | -79.0 | -91.1 |
| combined REFSENS(5MHz BW), dBm | -91.4 |  |  |
| MSD | 8.617 |  |  |

Proposal 1: It’s proposed to specify the following MSD for CA_n34A-n40A_BCS4/5.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n34 | 2350 | 100 | 30 | 270 (RBstart=0) | 2022.5 | 5 | 8.6 | >ACLR2 |

Summary

Proposal 1: It’s proposed to specify the following MSD for CA_n34A-n40A_BCS4/5.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n34 | 2350 | 100 | 30 | 270 (RBstart=0) | 2022.5 | 5 | 8.6 | >ACLR2 |

- References

[1] R4-2317657, WF on CA_n34-n40 MSD, Skyworks Solutions Inc, Huawei