3GPP TSG-RAN WG4 Meeting # 109                               R4-230xxxx

Chicago, US, November 13 – November 17, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on how to specify the MSD due to harmonic mixing issue

Agenda Item:	8.1.3

Document for:	Approval

Introduction

In the RAN4#108 meeting, we provided a discussion paper [1] to go through the whole discussion and official agreements on MSD due to harmonic mixing issue. And we tried to reach a consensus on some general rules of how to specify the harmonic mixing MSD. This topic was discussed extensively.

In the RAN4#108bis meeting, the MSD due to Even harmonic mixing was discussed in the paper [2]. A good question was proposed: which DL frequency range and power level should be specified for different even harmonic mixing MSD with the following proposals.

Proposal: Even harmonic mixing MSD should be investigated and specified if necessary, for:

- UL1/DL2 for all FR1 DL bands and all power classes

- UL1/DL4 for all FR1 DL bands for PC2 and PC1.5 and for DL bands >1GHz for PC3

- UL3/DL2 for all FR1 DL bands >3GHz for PC3 and >1GHz for PC2 and PC1.5

- UL3/DL4 for all FR1 DL bands >5GHz for PC5 and PC3 and >3GHz for PC2 and PC1.5.

In this paper, we’d like to share our views on this topic.

Discussion

Referring to the discussion papers [3][4][5][6][7], the Rx 2nd and 3rd harmonic rejection of Receiver LO from companies’ contributions are summarized below.

Table 1 The summary of Rx harmonic rejection of Receiver LO

| Rx harmonic order | Proposed Values |
| --- | --- |
| Rx 3rd harmonic rejection of Receiver LO | 20dB [3] 30dB [4] |
| Rx 2nd harmonic rejection of Receiver LO Rx 2nd harmonic rejection of Receiver LO for DL band above 1.7GHz Rx 2nd harmonic rejection of Receiver LO for DL band above 1.7GHz | At least 60dB [5] 46dB [6] 52dB [7] |

Ccurrent harmonic mixing cases in 38.101-1 in the last column of the table were summarized below:

Table 2 Current harmonic mixing cases in 38.101-1

| UL band | Power Class | DL band | DL band range | UL/DL collision | Case specified |
| --- | --- | --- | --- | --- | --- |
| n3 | PC3 | n26 | <1GHz | UL1/DL2 | Yes for all power classes |
| n7 | PC3 | n71 | <1GHz | UL1/ DL4 | No for PC3 |
| n40 | PC3 | n77/78 | >3GHz | UL3/DL2 | Yes for all power classes |
| n41 | PC3 | n39 | >1GHz and <3 GHz | UL3/DL4 | No for all power classes |
| n41 | PC3 | n77/78 | >3GHz | UL3/DL2 | Yes for all power classes |
| n46 | PC5 | n7 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes |
| n77 | PC3/2/1.5 | n2 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes |
| n77 | PC3/2/1.5 | n25 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes |
| n77 | PC2/PC1.5 | n3 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes (PC3 missing?) |
| n77 | PC2/PC1.5 | n5 | <1GHz | UL1/DL4 | Yes for PC2/PC1.5 |
| n77 | PC3 | n8 | <1GHz | UL1/DL4 | No for PC3 |
| n77 | PC3 | n70 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes |
| n78 | PC2 | n3 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes (PC3 missing?) |
| n78 | PC2 | n8 | <1GHz | UL1/DL4 | Yes for PC2 |

CA_n5-n77 is the first band combination that the MSD in DL band n5 below 1GHz is introduced due to 4th order harmonic mixing in Rel-16.

CA_n3-n26 is the first band combination that the MSD in DL band n26 below 1GHz is introduced due to 2nd order harmonic mixing in Rel-18.

However, MSD for these two highlighted band combinations were introduced without any specific technical investigation.

Referring to TS 38.101-1-gh0, the MSD exceptions due to due to harmonic mixing for CA in NR FR1 are copied below.

Table 7.3A.4-4: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1 NR Band / Channel bandwidth of the affected DL band. UL band; DL band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB); 25 MHz (dB); 30 MHz(dB); 40 MHz (dB); 50 MHz (dB); 60 MHz (dB); 70 MHz(dB); 80 MHz (dB); 90 MHz (dB); 100 MHz (dB). n25; n713,4; 26.5; 23.3; 20.9; 15.3. n40; n284; 37.8; 34.8; 33; 30.3. n77; n2; 6.7; 5.0; 4.0; 3.7. n77; n5; 5.7; 4.0; 3.0; 2.7. n78; n402; 10.4; 10.4; 10.4; 10.4; 7.2; 6.2; 5.5; 4.5. n78; n412; 10.4; 10.4; 10.4; 8.2; 7.6; 7.3; 6.6; 6.4; 6.3. n79; n85; 25; 21.8; 19.4; 13.8. NOTE 1: Void. NOTE 2: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz..

Table 7.3B.2.3.2-1 of TS 38.101-3 is shown below.

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| 2 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| n38 | 59 | N/A | N/A |  |  |  |  |  |  |  |  |  |
| n40 | 284 | 37.8 | 34.8 | 33 | 30.3 |  |  |  |  |  |  |  |
| n41 | 264 | 24.3 | 24.3 | 22.5 | N/A |  |  |  |  |  |  |  |
| 48 | n122 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n77 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n78 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |

Obviously, it’s lack of technical analysis about Rx 4th harmonic rejection of Receiver LO for DL band below 1.0 GHz. And the MSD value should not be same as the 2nd order harmonic mixing MSD of victim band 3 from aggressor band n78 for DC_3_n78.

Referring to the TR 38.718-02-01 [8], the REFSENS analysis in clause 5.2.1.5 is copied below.

- 5.2.1.5 REFSENS requirements As can be seen in the co-existence studies in 5.2.1.3 there are no harmonics issues. Based on the co-existence studies there are 2nd harmonic mixing from band n26 DL into band n3 UL. MSD values based on CA_n2-n77 corrected for lower Rx harmonic mixing gain for band n26. Table 5.2.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1 UL band; DL band; UL BW; SCS of UL band; UL RB Allocation; DL BW; MSD; UL/DL fc condition; UL/DL harmonic order. (MHz); (kHz); LCRB; (MHz); (dB). n3; n26; 5; 15; 25 (RBstart=0); 5; 3.7; NOTE 1; UL1/DL2.

For CA_n3-n26, the MSD due to 2nd harmonic mixing is 3dB less than the MSD due to 2nd harmonic mixing for CA_n2-n77 without any technical reason. However, in LTE phase, both CA_3-5 and CA_4-5 were specified without any degradation and they were deployed so well without any issues.

Observation 1: The even order harmonic mixing MSD on DL bands below 1GHz were specified without any technical analysis. And it’s contradicted with LTE specification logically.

As the main issue is which DL frequency range and power level should be specified for different even harmonic mixing MSD. I summarized the following table based on the previous agreement and specification logic.

Table 3 Which DL frequency range and power level should be specified for different even harmonic mixing MSD.

|  | 1UL/2DL and 1UL/4DL cases |  |
| --- | --- | --- |
|  | Victim bands below 1.7GHz | Victim bands above 1.7GHz |
| PC3 in Aggressor bands | There is no degradation cases in LTE spec for 1UL/2DL and 1UL/4DL cases. For NR, it’s proposed to follow the same requirement. | Based on Rel-16 agreement: MSD due to 1UL/2DL will be considered. |
| PC2 in Aggressor bands | Up to 3dB MSD for 1UL/2DL and 1UL/4DL cases | Additional 3dB based on the evaluation from PC3 aggressor band. |
| PC1.5 in Aggressor bands | Up to 6dB MSD for 1UL/2DL and 1UL/4DL cases | Additional 6dB based on the evaluation from PC3 aggressor band. |

Proposal 1: In order to align with LTE spec as much as possible, it’s proposed to remove the MSD test configurations due to 2nd and 4th order harmonic mixing in victim bands below 1.7GHz when the aggressor band is power class 3.

Summary

Observation 1: The even order harmonic mixing MSD on DL bands below 1GHz were specified without any technical analysis. And it’s contradicted with LTE specification logically.

Proposal 1: In order to align with LTE spec as much as possible, it’s proposed to remove the MSD test configurations due to 2nd and 4th order harmonic mixing in victim bands below 1.7GHz when the aggressor band is power class 3.

- References

[1] R4-2312932, Discussion on MSD due to harmonic mixing interference, Huawei, HiSilicon

[2] R4-2316859, On UL1-DL4 harmonic mixing and table templated for 1UL-CC and 2UL-CC MSD studies, Skyworks Solutions Inc

[3] R4-152188, MSD analysis for inter-band CA with lower band Rx harmonic mixing problem, MediaTek Inc.

[4] R4-152871, TP for TR 36.852-13 UE RF requirements for CA_28A-40A, Huawei, Hisilicon

[5] R4-1707995, On harmonic mixing for LTE NR DC band combinations, Huawei, HiSilicon

[6] R4-1909937, 2nd Harmonic Mixing for MB+NR and Missing MSD for B3_n77/n78, Qualcomm Incorporated

[7] R4-1911161, The 2nd Harmonic Mixing MSD for ENDC B3_n77/n78, MediaTek Inc., Murata

[8] R4-2312479, TR 38.718-02-01 V0.7.0, Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for2 bands DL with x bands UL (x=1,2)