3GPP TSG-RAN4 Meeting #109	R4-2319869

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1926 | rev | - | Current version: | 15.23.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_newRAT-Core] CR for 38.101-1 to clarify the applicable bands for additional UTRA ACLR requirements. (R15) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Generally, UTRA ACLR requirements as additional requirements are only applicable to the NR bands of NS_100, which may have UTRA service deployed. However, in R16, R17 and R18 spec, there are some statements that claified UTRAACLR is not applicable in some bands. “UTRAACLR is not applicable to the power class 3 UE operating in Band n12, n14, n17, n30, n100 and n101. UTRAACLR is not applicable to the power class 1 UE operating in Band n14, n71, n85, n100 and n101.” But these clarifications copied from LTE spec may cause some confusion that UTRAACLR is also applicable to all the other bands (e.g. n41 and n78) except for these bands n12, n14, n17, n30, n100 and n101. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify that UTRA ACLR requirements are additional requirements and clearly state that which bands are applicable when NS_03U, NS_05U, NS_43U or NS_100 is signalled by the network. And remove some statements that UTRAACLR is not applicable in some bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current specy may cause some confusion that UTRAACLR is also applicable to all the other bands (e.g. n41 and n78) except for these bands n12, n14, n17, n30, n100 and n101. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change>>

6.5.2.4.2	UTRA ACLR

UTRA adjacent channel leakage power ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent(s) UTRA channel frequency.

UTRAACLR is specified for the first adjacent UTRA channel (UTRAACLR1) which center frequency is ± 2.5 MHz from NR channel edge and for the 2nd adjacent UTRA channel (UTRAACLR2) which center frequency is ± 7.5 MHz from NR channel edge.

The UTRA channel power is measured with a RRC filter with roll-off factor = 0.22 and bandwidth of 3.84 MHz. The assigned NR channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.5.2.4.1-1.

If the measured adjacent channel power is greater than – 50 dBm then the UTRAACLR1 and UTRAACLR2 shall be higher than the value specified in Table 6.5.2.4.2-1.

Table 6.5.2.4.2-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB |
| UTRAACLR2 | 36 dB |

Since UTRA ACLR requirement is the additional requirement for the NR bands of NS_100 specified in Table 6.2.3.1-1, which may have UTRA services deployed, UTRA ACLR requirement is applicable when  the network signalling value NS_03U, NS_05U, NS_43U or NS_100 is signalled by the network in the field additionalSpectrumEmission.

<<End of Change>>