3GPP TSG-RAN WG4 Meeting # 109                                R4-2319882

# Chicago, US, November 13 – 17, 2023

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.876 to introduce some Rx requirements

Agenda Item:	8.13.3.2

Document for:	Approval

Introduction

It’s proposed to introduce maximum input level and Rx intermodulation requirements based on the approved WF [1].

- References

[1] R4-2317748, WF on NR_ATG_UERF_part2, Huawei

[2]

- Text proposal

- <<Start of Change for TR 38.876>>

- 7.1.3.4 Maximum input level

The calculation and results for maximum input level are listed below.

Table 7.1.3.4-1 the calculation for maximum input level requirements

| Parameters | Omni-directional antenna | phased array antenna |
| --- | --- | --- |
| Center frequency GHz | 2 | 4 |
| Distance meter | 3000 | 3000 |
| Pathloss dB | 108.063 | 114.0836 |
| ATG BS output power dBm | 46 | 53 |
| ATG BS antenna gain | 21 | 21 |
| ATG UE antenna gain | 0 | 11 |
| ATG UE body loss | 1 | 1 |
| maximum input level dBm | -42.063 | -30.0836 |

The maximum input levels are specified below.

Table 7.1.3.4-2 the calculation for maximum input level requirements

| Rx Parameter | Units | ATG UE Types |  | Reference measurement channel |
| --- | --- | --- | --- | --- |
|  |  | ATG UE with omni-directional antenna | ATG UE with phased antenna array |  |
| Power in Transmission Bandwidth Configuration | dBm | -42 | -30 | A.3.2.3 or A.3.3.3 for 64 QAM |
|  |  | -44 | -32 | A.3.2.4 or A.3.3.4 for 256 QAM |
| The applicable channel bandwidths | MHz | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table [7.3.2-3] with PCMAX_L,f,c as defined in clause [6.2.4]. |  |  |  |  |

- <<Next of Change>>

- 7.1.3.9 Intermodulation characteristics

Intermodulation characteristics requirements specified in TS 38.101-1 can be reused for ATG UE.

- <<End of Change>>