# 3GPP TSG-RAN WG4 Meeting # 109	R4-2319883

# Chicago, US, November 13 – November 17, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on how to specify SEM requirements for ATG UE

Agenda Item:	8.13.3.1

Document for:	Approval

Introduction

In the RAN4#108bis meeting, WF [1] was approved with the following agreements for SEM requirements.

Issue 2-2-1: SEM requirements <Way forward>: If ATG UE output power is less than or equal to 31dBm, the FR1 UE SEM mask specified in TS 38.101-1 apply. If ATG UE output power is larger than 31dBm, the FR1 UE SEM mask specified in TS 38.101-1 shall be relaxed by the scaling factor (Pout – 31) dB. NOTE: This scaling factor is only applicable to ATG airborne UE. (This sentence will be captured into the specification.)

In this paper, we’d like to discuss how to specify the SEM requirements for ATG UE.

Discussion

In the draft CR [2], the following wordings were proposed based on the “rated maximum output power”.

If rated maximum output power of ATG UE is less than or equal to 31dBm, the requirements for spectrum emission mask in clause 6.5.2.2 apply; if[ rated maximum output power] of ATG UE is larger than 31dBm, the requirements of spectrum emission mask in clause 6.5.2.2 shall be relaxed with scaling factor equal to ([rated maximum output power] minus 31) dB. NOTE: This scaling factor is only applicable to ATG airborne UE.

Proposal 1: To reuse the wordings in RAN4#108bis meeting way forward.

- 6.5J.2.2	Spectrum emission mask

If ATG UE output power is less than or equal to 31dBm, the FR1 UE SEM mask specified in TS 38.101-1 apply.

If ATG UE output power is larger than 31dBm, the FR1 UE SEM mask specified in TS 38.101-1 shall be relaxed by the scaling factor (ATG UE output power – 31) dB.

NOTE: This scaling factor is only applicable to ATG airborne UE.

Summary

Proposal 1: To reuse the wordings in RAN4#108bis meeting way forward.

- 6.5J.2.2	Spectrum emission mask

If ATG UE output power is less than or equal to 31dBm, the FR1 UE SEM mask specified in TS 38.101-1 apply.

If ATG UE output power is larger than 31dBm, the FR1 UE SEM mask specified in TS 38.101-1 shall be relaxed by the scaling factor (ATG UE output power – 31) dB.

NOTE: This scaling factor is only applicable to ATG airborne UE.

- References

[1] R4-2317748, WF on NR_ATG_UERF_part2, Huawei

[2] R4-2317744, draft CR to TS 38.101-1: clause 6.5J,7.1J,7.2J, ZTE Corporation