# 3GPP TSG-RAN WG4 Meeting # 109	R4-2319885

# Chicago, US, November 13 – November 17, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on RF requirements for CA_n26(2A)

Agenda Item:	7.30.2.5

Document for:	Approval

Introduction

In the RAN#101 meeting, a revised WID [1] was approved to include more sub-1GHz band combinations, i.e. CA_n26(2A) and CA_n5A-n28A-n105A. As it is the first UL intra-band non-contiguous CA in FDD band that will be discussed in RAN4 for UL CA_n26(2A). The NR CA configurations are copied as below.

NR CA configuration; Uplink CA configuration or single uplink carrier; NR Band; Channel bandwidth (MHz); Bandwidth combination set. CA_n5A-n8A; -; n5; 5, 10; 0. n8; 5, 10. CA_n5A-n28A; CA_n5A-n28A; n5; 5, 10, 15, 20; 0. n28; 5, 10, 15, 20, 30. CA_n8A-n20A-n28A; CA_n8A-n20A CA_n8A-n28A CA_n20A-n28A; n8; 5, 10, 15, 20; 0. n20; 5, 10, 15, 20. n28; 5, 10, 15, 20. CA_n5A-n105A; CA_n5A-n105A; n5; 5, 10, 15, 20; 0. n105; 5, 10, 15, 20, 25, 30, 35. CA_n28A-n105A; -; n28; 5, 10, 15, 20, 25, 30; 0. n105; 5, 10, 15, 20, 25, 30, 35. CA_n26A-n28A; CA_n26A-n28A; n26; 5, 10, 15, 20, 25, 30; 0. n28; 5, 10, 15, 20, 25, 30. CA_n5A-n28A-n105A; CA_n5A-n105A CA_n5A-n28A; n5; 5, 10, 15, 20; 0. n28; 5, 10, 15, 20, 25, 30. n105; 5, 10, 15, 20, 25, 30, 35. CA_n26(2A); CA_n26(2A); n26; 5, 10, 15; 0.

The demands are from Telstra and the spectrum holding in band n5 and additional n26 is shown below.

figure 1 spectrum holding in band n5 and additional n26 from operator

In RAN4#108bis meeting, a WF [2] for CA_n26(2A) has been approved with the following agreements.

Issue 1-1: Baseline RF architecture - Assume dual PA architecture only. Issue 1-2: Current MPR applicability for UL CA_n26(2A) - Current 2PA MPR requirements can apply for UL CA_n26(2A) Issue 1-3: NS_12/ 13/ 14/ 15, NS_100 applicability for UL CA_n26(2A) - The following idea is the guideline for the discussion in next meeting. - Define the CA NC NS_12/13/14/15/100 with relaxed values for AMPR UE is allowed to apply any backoff to meet emission requirements Issue 1-4: Coexistence requirements with neighboring bands - Companies are encouraged to study how to meet this requirement for UL NC CA. - Max Channel BW per CC for this BCS is 15MHz Issue 1-5: MSD test points and value - The general approach/framework for 2UL/2DL FDD NC CA MSD will be discussed. - The format in the following table can be considered as a starting point. CA configuration; PCC/SCC Bandwidth (MHz); PCC UL Fc (MHz); SCC UL Fc (MHz); UL PCC allocation (LCRB); UL SCC allocation (LCRB); PCC DL Fc (MHz); SCC DL Fc (MHz); PCC ΔRIBC (dB); SCC ΔRIBC (dB); Duplex mode. CA_n26(2A); 5/5; 816.5; 839; 12 (RBSTART = [0]); 12(RBSTART = [3]); 861.5; 884; TBD; TBD; FDD. - The following principles can be considered and further discussed for the MSD requirement. - Channel allocation is 5MHz+5MHz - 15kHz SCS - Total of equally distributed RB is equivalent to the 10MHz UL configuration - PCC DL channel overlaps with the lowest order IMD product of the two CCs - RB start of each CC can be adjusted to better center the IMD product.

We have agreed to assume dual PA architecture only. In this paper, we’d like to analyze the remaining RF requirements for UL CA_n26(2A).

Discussion on MPR and AMPR requirements for UL CA_n26(2A)

Since no operators are interested in non-contiguous CA NS_12/ 13/ 14/ 15/NS_100 for UL CA_n26(2A), RAN4 has agreed to allow UE to apply any backoff to meet therse additional requirements. However, the minimum output power is -40dBm. Thus, the maximum output power reduction is 60dB for PC3 UL intra-band non-contiguous CA.

Proposal 1: To specify 60dB AMPR for UL CA_n26(2A) to meet additional requirements below.

Table 6.2A.3.1.2-1: Additional Maximum Power Reduction (A-MPR) for intra-band non-contiguous CA

| CA Network Signalling value | Requirements (clause) | Uplink CA Configuration | A-MPR for sub-blocks in order of increasing uplink carrier frequency |
| --- | --- | --- | --- |
|  |  |  | A-MPR [dB] (clause) |
| CA_NC_NS_01 | 6.5A.2.2.2 6.5A.3.2.2 | All applicaple NR CA configurations | N/A |
| CA_NC_NS_04 | 6.5A.2.3.2.1 6.5A.3.3.2.1 | CA_n41(2A) | 6.2A.3.1.2.1 |
| CA_NC_NS_12 | 6.5A.3.3.2.2 | CA_n26(2A) | 60 dB |
| CA_NC_NS_13 | 6.5A.3.3.2.3 | CA_n26(2A) | 60 dB |
| CA_NC_NS_14 | 6.5A.3.3.2.4 | CA_n26(2A) | 60 dB |
| CA_NC_NS_15 | 6.5A.3.3.2.5 | CA_n26(2A) | 60 dB |
| CA_NC_NS_55, CA_NC_NS_57 | See CA_NC_NS_01 | CA_n77(2A) | See CA_NC_NS_01 |
| CA_NC_NS_100 | 6.5A.2.4.2.2 | CA_n26(2A) | 60 dB |

Discussion on MSD requirements for UL CA_n26(2A)

In last RAN4 meeting, the format in the following table can be considered for UL CA_n26(2A)as a starting point.

| CA configuration | PCC/SCC Bandwidth (MHz) | PCC UL Fc (MHz) | SCC UL Fc (MHz) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC DL Fc (MHz) | SCC DL Fc (MHz) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 5/5 | 816.5 | 839 | 12 (RBSTART = [0]) | 12(RBSTART = [3]) | 861.5 | 884 | TBD | TBD | FDD |

Based on the assumed spectrum allocation, it’s observed that IMD3 and IMD5 will hit the DL PCC and SCC carriers respectively as below.

figure 2 MSD test configuration

Based on the agreement in last meeting, only dual PA architecture will be considered for UL CA_n26(2A) in Rel-18. Thus, we have the following RF architecture to evaluate the MSD for UL CA_n26(2A).

figure 3 RF architecture for UL CA_n26(2A)

After analysis, the IMD3 generated by PA reverse mixing will dominate the MSD level. Thus, the following link budget is provided to evaluate MSD for UL CA_n26(2A).

Table 1 MSD analysis for UL CA_n26(2A)

|  | Parameters | Main path (UL PCC) | Interference from SCC | Interference from PCC | diversity path (UL SCC) |
| --- | --- | --- | --- | --- | --- |
| Output power at antenna connector (dBm) |  | 20 | 10 | 10 | 20 |
| Antenna isolation (dB) | 10 |  |  |  |  |
| diplexer insertion loss (dB) | 0.5 |  |  |  |  |
| Switch insertion loss (dB) | 1 |  |  |  |  |
| Duplexer insertion loss (dB) | 2 |  |  |  |  |
| level at PA output port (dBm) |  | 23.5 | 6.5 | 6.5 | 23.5 |
| PA reverse mixing OIP (dBm) | 28 |  |  |  |  |
| duplexer isolation (dB) | 55 |  |  |  |  |
| IMD3 interference (dBm) at Rx ant port (Some impacts on PCC) |  | -71 |  | -54 |  |
| IMD5 interference (dBm) at Rx ant port (Some impacts on SCC) |  | -97 |  | -80 |  |
| noise figure dB | 11.5 |  |  |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -162.5 |  |  |  |  |
| Rx band n28 BW(MHz, NRB) | 4.5 |  |  |  |  |
| Thermal noise, dBm |  | -95.967875 |  | -95.967875 |  |
| Total interference(IMD3 + Thermal) in PCC (dBm) at ant port |  | -70.986186 |  | -53.999724 |  |
| Total interference(IMD5 + Thermal) in SCC (dBm) at ant port |  | -93.443048 |  | -79.891468 |  |
| SNR requirement for QPSK | -1 |  |  |  |  |
| REFSENSE in PCC (dBm) at ant port |  | -71.986186 |  | -54.999724 |  |
| REFSENSE in SCC (dBm) at ant port |  | -94.443048 |  | -80.891468 |  |
| Implementation Margin, dB | 2.5 |  |  |  |  |
| combined REFSENS in PCC (5MHz BW), dBm |  | -69.572252 |  |  |  |
| combined REFSENS in SCC (5MHz BW), dBm |  | -92.130639 |  |  |  |
| MSD in PCC (5MHz BW), dB |  | 27.927748 |  |  |  |
| MSD in SCC (5MHz BW), dB |  | 5.3693605 |  |  |  |
| REFSENS in TS38.101 for n26 (15kHz, 5MHz) | -97.5 |  |  |  |  |

Proposal 2: to specify the following MSD levels for UL CA_n26(2A) and improve the MSD table format.

| CA configuration | PCC/SCC (SCS, BW) | PCC/SCC UL Fc (MHz) | UL PCC/SCC allocation (LCRB) | PCC/SCC DL Fc (MHz) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | (15kHz, 5MHz)/ (15kHz, 5MHz) | 816.5 / 839 | 12 (RBSTART = 0) / 12 (RBSTART = 3) | 861.5 / 884 | 27.9 | 5.4 | FDD |

Summary

Proposal 1: To specify 60dB AMPR for UL CA_n26(2A) to meet additional requirements below.

Table 6.2A.3.1.2-1: Additional Maximum Power Reduction (A-MPR) for intra-band non-contiguous CA

| CA Network Signalling value | Requirements (clause) | Uplink CA Configuration | A-MPR for sub-blocks in order of increasing uplink carrier frequency |
| --- | --- | --- | --- |
|  |  |  | A-MPR [dB] (clause) |
| CA_NC_NS_01 | 6.5A.2.2.2 6.5A.3.2.2 | All applicaple NR CA configurations | N/A |
| CA_NC_NS_04 | 6.5A.2.3.2.1 6.5A.3.3.2.1 | CA_n41(2A) | 6.2A.3.1.2.1 |
| CA_NC_NS_12 | 6.5A.3.3.2.2 | CA_n26(2A) | 60 dB |
| CA_NC_NS_13 | 6.5A.3.3.2.3 | CA_n26(2A) | 60 dB |
| CA_NC_NS_14 | 6.5A.3.3.2.4 | CA_n26(2A) | 60 dB |
| CA_NC_NS_15 | 6.5A.3.3.2.5 | CA_n26(2A) | 60 dB |
| CA_NC_NS_55, CA_NC_NS_57 | See CA_NC_NS_01 | CA_n77(2A) | See CA_NC_NS_01 |
| CA_NC_NS_100 | 6.5A.2.4.2.2 | CA_n26(2A) | 60 dB |

Proposal 2: to specify the following MSD levels for UL CA_n26(2A) and improve the MSD table format.

| CA configuration | PCC/SCC (SCS, BW) | PCC/SCC UL Fc (MHz) | UL PCC/SCC allocation (LCRB) | PCC/SCC DL Fc (MHz) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | (15kHz, 5MHz) / (15kHz, 5MHz) | 816.5 / 839 | 12 (RBSTART = 0) / 12 (RBSTART = 3) | 861.5 / 884 | 27.9 | 5.4 | FDD |

- References

[1] RP-232680, Revised WID on enhancement for 700/800/900MHz band combinations, CATT

[2] R4-2317589, WF on NR_700800900_combo_enh, Moderator (CATT)