3GPP TSG-RAN WG4 Meeting # 109                               R4-2319887

Chicago, US, November 13 – November 17, 2023

Source: 	Huawei, HiSilicon, Spark NZ

Title: 	Discussion on RF requirements for CA_n5-n28-n105

Agenda Item:	7.30.2.2

Document for:	Approval

Introduction

In the RAN4#108 meeting, the CR [1] for TR 38.872 to introduce some potential RF impacts for CA_n5-n28-n105 was agreed to capture the outcomes during SI phase. In last meeting, RAN4 has agreed the Delta Tib and Rib for CA_n5-n28-n105 based on approved WF [2]. It’s encouraged for companies to check the MSD value with brackets as starting point. This paper is resubmitted based on the contribution [3] in the RAN4#108bis meeting.

Discussion on MSD due to intermodulation

UL n5-n105

Table 5 lists Band n5 + Band n105 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n28.

Table 5 UL CA_n5-n105 IMD product

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 703 | 824 | 849 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 121 | 186 | 1487 | 1552 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 477 | 582 | 945 | 1035 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2150 | 2255 | 2311 | 2401 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1140 | 1285 | 1769 | 1884 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2813 | 2958 | 3135 | 3250 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 372 | 242 | 2974 | 3104 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2733 | 2593 | 1988 | 1803 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1221 | 1066 | 461 | 291 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3959 | 4099 | 3476 | 3661 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3798 | 3953 | 3637 | 3807 |

Observation 1: there is no IMD interference which fall into Rx frequencies of third DL band n28 when CA_n5-n105 is the UL configuration.

UL n5-n28

Table 6 lists Band n5 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n105.

Table 6 UL CA_n5-n28 IMD product

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 824 | 849 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 76 | 146 | 1527 | 1597 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 557 | 672 | 900 | 995 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2230 | 2345 | 2351 | 2446 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1260 | 1420 | 1724 | 1844 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2933 | 3093 | 3175 | 3295 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 292 | 152 | 3054 | 3194 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2693 | 2548 | 2168 | 1963 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1141 | 976 | 596 | 411 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3999 | 4144 | 3636 | 3841 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3878 | 4043 | 3757 | 3942 |

Observation 2: 3rd order IMD may also fall into mobile receive frequencies of third DL band n105 when CA_n5-n28 is the UL configuration.

There are network-based mitigation issues discussed later to address any potential impact of the IMD.

Three-antenna RF architecture Fig 2 and Fig 3 respectively are used to derive the MSD of band n105 when UL CA is CA_n5-n28. These architectures are referred to as dual-transmitter antenna architecture Fig 2 and no common diplexer band RF architecture Fig 3 in 3GPP reference TR 36.860 [4].

The component linearity assumptions, the component attenuation and isolation values are listed in Tables 7 and 8 respectively.

Table 7 General linearity parameters

| Component | IP3(dBm) |
| --- | --- |
| Antenna switch | 68 |
| Diplexer | 86 |
| Triplexer/Duplexer | 74 |
| PA forward mixing | 30 |
| PA reverse mixing | 28 |
| LNA | -6 |

The above values are based on common understanding in RAN4. Company’s contribution [5] is referred.

Table 8 Attenuation and isolation values

| Attenuation and Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna ISO | 10 |  |
| PA gain | 25 |  |
| PCB isolation Paout-Pain | 60 | PCB isolation (PA forward mixing) |
| n28 Tx attenuation at Band n105 Rx | 30 |  |
| n28 Tx attenuation at Band n5 Tx | 30 |  |
| n5 Tx attenuation at n105 RX | 30 |  |
| n5 Tx attenuation at n28 TX | 30 |  |
| n105 Rx attenuation at n5 TX | 40 |  |
| n105 Rx attenuation at n28 RX | 40 |  |

The above values are based on common understanding in RAN4 especially for PA gain and PCB/antenna isolation referring to TR 36.860 [4].

Table 9 summarize the MSD analysis results for DL_n5A-n28A-n105A_UL_n5A-n28A. Some main IMD interference paths were depicted in figure 6.

What Fig 2 and Fig 3 are different architectures, but from MSD calculation point of view, they are essentially the same. This is because these two architectures have the same characteristics. Namely, Tx chains of band n28 and n5 are split under two antennas and the main Rx chain of band n105 is combined with the Tx chain of band n5. Therefore, we only show one table of MSD calculation.

Table 9 MSD calculation for victim Rx of band n105 when UL_n5A-n28A

| No. | Component | IMD3(dBm/5MHz) |  |
| --- | --- | --- | --- |
|  |  | Main_@Antenna | Div_@Antenna |
| 1) | n5 PA Forward mixing interference | -95.305 |  |
| 2) | n28 PA Forward mixing interference | -69.15 |  |
| 3) | n5 PA Reserve mixing interference | -112.5 |  |
| 4) | n28 PA Reserve mixing interference | -76 |  |
|  | n5/n105 quadplexer | -210.5 |  |
|  | n28 Duplexer | -156 |  |
| 5) | n105 LNA via filters | -66 | -79 |
|  | n105 LNA via PCB coupling | -94 | -94 |
|  | Diplexer (L-H) | -133 | -142 |
|  | Antenna switch | -99 | -135.5 |
|  | From main path |  | -73.99 |
|  | Total | -63.99 | -72.77 |

Figure 1 The illustration of main IMD interference paths.

Proposal 1: It’s proposed to capture band n105 MSD due to IMD3 of Tx band n5 + band n28 as below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n28-n105 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n105 | 686 | 5 | 25 | 635 | 22.9 | FDD | IMD3 |

Generally, there are some network deployment methods to avoid the IMD product hitting the DL carrier directly. For example, if the UL carrier in band n28 is configured in 703~718MHz, no matter which UL frequency carrier in band n5 is configured, the IMD3 products will not hit the DL carrier in band n105 directly. If the UL carrier in band n28 is configured above 718MHz, the REFSENS degradation in band n105 can be observed due to the IMD product. The RFSENS degradation will be quantified during the specification phase

Observation 3: Generally, there are some network deployment methods to avoid the IMD product hitting the DL carrier directly. For example, if the UL carrier in band n28 is configured in 703~718MHz, no matter which UL frequency carrier in band n5 is configured, the IMD3 products will not hit the DL carrier in band n105 directly. If the UL carrier in band n28 is configured above 718MHz, the REFSENS degradation in band n105 can be observed due to the IMD product. The RFSENS degradation will be quantified during the specification phase.

- Summary

Observation 1: there is no IMD interference which fall into Rx frequencies of third DL band n28 when CA_n5-n105 is the UL configuration.

Observation 2: 3rd order IMD may also fall into mobile receive frequencies of third DL band n105 when CA_n5-n28 is the UL configuration.

Proposal 1: It’s proposed to capture band n105 MSD due to IMD3 of Tx band n5 + band n28 as below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n28-n105 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n105 | 686 | 5 | 25 | 635 | 22.9 | FDD | IMD3 |

Observation 3: Generally, there are some network deployment methods to avoid the IMD product hitting the DL carrier directly. For example, if the UL carrier in band n28 is configured in 703~718MHz, no matter which UL frequency carrier in band n5 is configured, the IMD3 products will not hit the DL carrier in band n105 directly. If the UL carrier in band n28 is configured above 718MHz, the REFSENS degradation in band n105 can be observed due to the IMD product. The RFSENS degradation will be quantified during the specification phase.

- References

[1] R4-2314659, CR for TR 38.872 to introduce some potential RF impacts for CA_n5-n28-n105, Huawei, HiSilicon, Skyworks Solutions Inc., Spark NZ, Apple, Qualcomm, Vivo

[2] R4-2317589, WF on NR_700800900_combo_enh, Moderator (CATT)

[3] R4-2316211	Discussion on RF requirements for CA_n5-n28-n105, Huawei, HiSilicon, Spark NZ

[4] TR 36.860, LTE-Advanced dual uplink inter-band Carrier Aggregation (CA), V12.0.0

[5] R4-1801626, MSD analysis for DC_3A-7A_n78A, DC_3A-20A_n78A and DC_7A-20A_n78A, Huawei, HiSilicon