3GPP TSG-RAN WG4 Meeting #109	R4-2319900
Chicago, US, 13–17 Nov, 2023

Title:	R17 power class applicability for inter-band CA

Source:	OPPO

Agenda item:	11.2.5

Document for:	Approval

- Introduction

The power class ambiguity issue has been discussed for several meetings [1,2,3], and the left issue is the applicable power class for band in a band combination which relates to how UE will apply the MPR/AMPR and its power control. This paper further discusses this issue.

- Applicable power capability and MPR for band in CA

This also has been long discussed, and our understanding is summarized in below table, i.e. MIN {ue-PowerClass, powerClass} will be applied for the Tx power and MPR.

Table 1 Comparison of ue-PowerClass and powerClass in UL CA

| Compare ue-PowerClass and PowerClass | MOP and MPR for the band under band combination |
| --- | --- |
| ue-PowerClass is lower than PowerClass | ue-PowerClass will restrict the MOP and MPR |
| ue-PowerClass is higher than PowerClass | PowerClass will be applied in determining the MOP and MPR |
| ue-PowerClass is equal to PowerClass | Either power class can be applied |

Besides, it should be noted that in real UE implementation the MPR/AMPR settings corresponding to the power class in power control process is relatively stable. So even the max Tx power may be restricted due to for example max UL duty cycle the MPR/AMPR settings might not be changed.

Observation 1:   In UE implementation the MPR/AMPR settings are determined by power class, and it will not be changed due to for example max UL duty cycle though the max Tx power may be restricted.

Proposal 1:   	Min {ue-PowerClass, powerClass} should be used to determine the power capability and MPR for the band under inter-band UL CA.

- Applicable power capability and MPR for DL-only CA

Regarding the DL-only CA scenario, from PA ability perspective there is no difficulty to keep same Tx power as the single band power class.

The only issue with the added CCs in DL is whether MSD is needed to keep the Tx power. However, this will not change the Tx power but only impact the REFSENS with MSD as long as it has specified and supported. This can be covered by the agreed CR [4], i.e. if the MSD has not been specified the higher power class is not supported by spec for certain band combination.

Observation 2:   From PA ability perspective, there is no difficulty to keep same Tx power as the single band power class as long as the MSD if any has been introduced in the spec.

From signalling perspective, the powerClass IE definition and applicability in DL-only CA case is not crystal clear. If UE is allowed to also report the power capability in DL-only CA case via powerClass IE, then it gives UE chance to indicate a different power class than ue-PowerClass IE and we need to consider ue-PowerClass is lower/higher/equal to PowerClass.

Further considering the case of higher power class is specified for single band but not for a DL CA case due to for example MSD hasn’t been finished, UE may need to report lower power class capability in DL CA only case than the single band power class. Then the MIN {powerClass, ue-powerClass} is needed in practice.

Observation 3:   From signalling perspective, if powerClass IE is applicable to DL-only CA case, then it can be different from ue-powerClass IE.

Proposal 2:   	Clarify powerClass IE is applicable to DL-only CA scenario.

Proposal 3:   	For DL-only CA scenario, the MIN {powerClass, ue-powerClass} is applied.

- Applicable power capability for intra-band UL CA in intra +inter band combination

4.1 ue-powerClass IE

This topic actually discussed in Rel-18 coverage enhance WI, however, due to related to how the power classes are applied it is discussed together with the maintenance power class topics.

As captured in RAN4#108bis meeting report, the endorsed CRs [5,6] just cover part of scenarios which it means is about the necessary updates in Pcmax for the HigherPowerLimit feature. In [7] it gives some example updates as below figure 1, where the important change is to clarify that the Ppowerclass,CA,Bi is used instead of Ppowerclass for each CC in inter-band + intra-band UL CA case (there is no power class for each CC in intra-band UL CA actually only have the CA power class). However, what unclear is about the applied power class for the band with intra-band UL CA, e.g. n41C in UL CA n3A-n41C.

Figure 1 Pcmax changes for higherPowerLimit feature of inter+intra UL CA

Below is an example from current spec (shown in figure 2). UE supports PC1.5 in band n41, but can only report PC2 in CA_n41C due to PC1.5 hasn’t been defined yet, and only PC3 is defined for CA_n3A-n41C. In this case, if ue-powerClass IE is applied for the n41C within UL CA_n3A-n41C then the Ppowerclass,CA is 23dBm+29dBm = 30dBm which is much higher than the PC3 of CA_n3A-n41C and much higher than its real capability in n41C.

UL CA_n3A-n41C   -> powerClass        PC3 (PC3 in n3 + PC3 in n41C)

UL CA_n41C           -> powerClass        PC2

UL n41                     -> ue-powerClass   PC1.5

Figure 2 Power class definition for n41, CA_n41C, and CA_n3A-n41C in 38.101-1

Observation 4:   Apply the ue-powerClass IE to determine the intra-band UL CA in inter-band UL CA will overestimate UE power capabilities.

4.2 powerClass IE

The Pcmax,L of intra+inter UL CA is also bounded by the SUM of Pcmax,L of each band according to current spec (shown in table 2), and the Pcmax,L of the band with intra-band UL CA is defined in clause 6.2A.4.1.1 where the intra-band UL CA power class takes effect (shown in table 3).

This means in above CA_n3A-n41C case, the CA_n41C power class is referred instead of n41 ue-powerClass to determine the Pcmax,L of the n41C in CA_n3A-n41C and also the total power. And from hardware perspective, CA_n41C is closer to the Tx power capability of n41C in CA_n3A-n41C.

Table 2 Pcmax of intra+inter UL CA

For combinations of intra-band and inter-band carrier aggregation with UE configured for transmission on three serving cells (up to two contiguously aggregated carriers per operating band), the following apply: The UE power class for the serving cell(s) on the operating band Bi including intra-band carrier aggregation shall be determined by the ue-PowerClassPerBandPerBC-r17 IE [7] as indicated for the band combination if signalled. For the case when p and q belong to the same band and k belongs to a different band, but p, q and k are of the same numerology and slot patterns. PCMAX_L = MIN {10log10∑( pCMAX_L, Bi), PEMAX,CA, PPowerClass.CA } PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass.CA } Where - pCMAX_L, Bi is the linear values of PCMAX_L specified for the specific operating band Bi. - The linear value of PCMAX_L specified for uplink intra-band contiguous carrier aggregation in subclause 6.2A.4.1.1 applies for operating band supporting two contiguous serving cells, designated by its band index Bi. The linear value of PCMAX_L specified for single carrier in subclause 6.2.4 applies for operating band Bj supporting one serving cell.

Table 3 Pcmax of intra band UL CA where powerClass IE will take effect

For uplink intra-band contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells, PCMAX_L = MIN{10 log10 ∑ pEMAX,c - TC , PEMAX,CA,(PPowerClass,CA– ΔPPowerClass,CA) – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPRc ) } PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass,CA– ΔPPowerClass,CA } where - pEMAX,c is the linear value of PEMAX,c which is given by IE P-Max for serving cell c in [7]; - PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.1-1 without taking into account the tolerance;

From above, it is more nature to apply the power class of intra-band UL CA to determine the power capability of intra-band UL CA under inter-band combination.

Observation 5:   It is more nature to apply the power class of intra-band UL CA to determine the power capability of intra+inter band combination and this is also what specified in current spec.

For the issues identified in [7], i.e. the powerClass is an optional capability and may not be reported for UL CA_n41C. From the capability reporting perspective, the power class capability for CA_n41C needs to be reported if it is different from parent BC power class, and it can be skipped if it is same as parent BC power class. Therefore, no matter from direct reporting or inherited from the parent BC power class reporting the power class for UL CA_n41C with certain DL configuration should be clear.

Observation 6:   For the intra-band UL CA, as long as the DL CA configuration is fixed, the applicable power capability is clear no matter it is reported via powerClass IE or inherited from parent BC power class.

However, one possible issue is that if the UL CA_n41C power capability is different when the DL band combination is changed, then this may lead to the question of which DL band combination should be used to determine the n41C power capability.

- For example, UL CA_n41C with DL CA_n41C the power capability is PC2, and for UL CA_n41C with DL CA_n1A-n41C the power capability is PC3, then which power class should be applied is something unclear.

Observation 7:   For the same intra-band UL CA, if UE report different power classes when the DL CA configuration is different, then there will be some ambiguity in which power class should be applied for this intra-band UL CA.

4.3 Min {powerClass, ue-powerClass}

From all the above discussion, we see none of ue-powerClass IE or powerClass IE is perfect to be used in determining the power class of intra-band CA in inter-band UL band combination:

- if use the ue-powerClass IE to determine the intra-band UL CA power capability then it will overestimate UE power capabilities, and considering power class is an important factor in determining the MPR/AMPR, power control, mobility, etc. it needs to be careful in using this.

- if use powerClass IE there might be some ambiguity in determine the intra-band UL CA power capability when the power capability is different for different DL CA configurations.

Observation 8:   Neither ue-powerClass IE or powerClass IE is perfect in determining the power class of intra-band CA in inter-band UL band combination.

To avoid above mentioned problems, it might be easier to directly apply the MIN {powerClass of CA_n1A-n41C, ue-powerClass of n41} for n41C band in UL CA_n1A-n41C if the ue-PowerClassPerBandPerBC-r17 is not reported. The reason is that the actual Tx power of n41C in CA_n1A-n41C will not exceed the total power class and the single band power class.

If UE power class for n41C is different from the MIN{ powerClass of CA_n1A-n41C, ue-powerClass of n41}, then UE can indicate directly via ue-PowerClassPerBandPerBC-r17 and there will be no problem.

Observation 9:   The Tx power of n41C in CA_n1A-n41C will not exceed the total power class and the single band power class which makes apply the MIN {powerClass of CA_n1A-n41C, ue-powerClass of n41} for n41C band in UL CA_n1A-n41C if the ue-PowerClassPerBandPerBC-r17 is not reported is possible. And if this is different from UE implementation, then it can still use the ue-PowerClassPerBandPerBC-r17 IE to report the exact power capability.

Proposal 4:   	Apply MIN {powerClass of intra+inter UL CA, ue-powerClass} to determine the power capability of intra-band UL CA part in intra+inter UL CA.

- Conclusions

In this contribution, Rel-17 power class related clarifications are discussed and got the following observations and proposals.

Observation 1:   In UE implementation the MPR/AMPR settings are determined by power class, and it will not be changed due to for example max UL duty cycle though the max Tx power may be restricted.

Proposal 1:   	Min {ue-PowerClass, powerClass} should be used to determine the power capability and MPR for the band under inter-band UL CA.

Observation 2:   From PA ability perspective, there is no difficulty to keep same Tx power as the single band power class as long as the MSD if any has been introduced in the spec.

Observation 3:   From signalling perspective, if powerClass IE is applicable to DL-only CA case, then it can be different from ue-powerClass IE.

Proposal 2:   	Clarify powerClass IE is applicable to DL-only CA scenario.

Proposal 3:   	For DL-only CA scenario, the MIN {powerClass, ue-powerClass} is applied.

Observation 4:   Apply the ue-powerClass IE to determine the intra-band UL CA in inter-band UL CA will overestimate UE power capabilities.

Observation 5:   It is more nature to apply the power class of intra-band UL CA to determine the power capability of intra+inter band combination and this is also what specified in current spec.

Observation 6:   For the intra-band UL CA, as long as the DL CA configuration is fixed, the applicable power capability is clear no matter it is reported via powerClass IE or inherited from parent BC power class.

Observation 7:   For the same intra-band UL CA, if UE report different power classes when the DL CA configuration is different, then there will be some ambiguity in which power class should be applied for this intra-band UL CA.

Observation 8:   Neither ue-powerClass IE or powerClass IE is perfect in determining the power class of intra-band CA in inter-band UL band combination.

Observation 9:   The Tx power of n41C in CA_n1A-n41C will not exceed the total power class and the single band power class which makes apply the MIN {powerClass of CA_n1A-n41C, ue-powerClass of n41} for n41C band in UL CA_n1A-n41C if the ue-PowerClassPerBandPerBC-r17 is not reported is possible. And if this is different from UE implementation, then it can still use the ue-PowerClassPerBandPerBC-r17 IE to report the exact power capability.

Proposal 4:   	Apply MIN {powerClass of intra+inter UL CA, ue-powerClass} to determine the power capability of intra-band UL CA part in intra+inter UL CA.

- References

- R4-2307735, Correction to UE power classes for CA configurations for HPUE (R17), Ericsson, RAN4#107, May 2023

- R4-2309679, CR for TS38101-1 Clarifying applicable power classes for NR CA (R17), Huawei, RAN4#107, May 2023

- R4-2308961, R17 power class between CA and single band, OPPO, RAN4#107, May 2023

- R4-2314915, CR for TS38101-1 Clarifying applicable power classes for NR CA, Huawei, RAN4#108, Aug 2023

- R4-2315848, draft CR to TS 38.101-1 for Introducing new scenarios for increase higher power limit for CA, vivo, RAN4#108bis, Oct 2023

- R4-2316336, draft CR to TS38.101-3 for Introducing new scenarios for increase higher power limit for ENDC, ZTE, RAN4#108bis, Oct 2023

- R4-2315149, Introduction of higherPowerLimit-r17 into NR CA of PC3+PC5 including UL Intra band CA, Nokia, RAN4#108bis, Oct 2023