3GPP TSG-RAN WG4 Meeting #109	R4-2319915

Chicago, US, November 13 – 17, 2023

Title:	Discussion on single-layer UL MIMO test methodology

Source:	OPPO

Agenda item:	8.15.2.1

Document for:	Approval

- Introduction

In the last RAN4 #108bis meeting, the test methodology together with performance metrics were further discussed based on the previous agreement.

This contribution discusses the test methodology of Single-layer UL-MIMO for fully coherent UE and provides the proposals for identifying metrics.

- Discussion

The agreements of single-layer UL-MIMO test method in the RAN4 #108bis meeting are captured as below [1].

From the agreements, the consensus is that all the four TPMI (2/3/4/5) will be measured for one-layer UL MIMO OTA test methodology. The divergence between companies is how to process the performance metrics. Option 1 is averaging all the TRPs with each TPMI as the performance metrics. And Option 2 is selecting the maximum EIRPs of every measurement direction to calculate the performance metrics. The advantages and disadvantages of the two options are discussed during the past several meetings. To our understanding, both Option 1 and Option 2 have their suitable applicable area. Rather than discussing which option is the best performance metrics, it is better to consider the proper applicability of the two options.

Firstly, the metrics and names are discussed. For Option 1, it should be noted that the averaged TRP is still aligned with the concept of TRP which is commonly used in the previous OTA metrics definition, such as TRP for browsing mode and talk mode in Rel-17 [2].

With this understanding, the metrics name of Option 1 is proposed as TRPaverage_TPMI as below. The averaging method of TRP can be further decided including linear scale averaging and dBm averaging.

TRPaverage_TPMI=10log10TRPTPMI210+10TRPTPMI310+10TRPTPMI410+10TRPTPMI5104      (linear scale averaging)

or         TRPaverage_TPMI=TRPTPMI2+TRPTPMI3+TRPTPMI4+TRPTPMI54                           (dBm averaging)

Proposal 1: The metrics name of Option 1 is proposed as TRPaverage_TPMI.

For Option 2, the maximum EIRP among each TPMI for every measurement direction is integrated on the sphere, where the final integral is not corresponding to any antenna pattern of a single TPMI index. This is not aligned with the general concept of Total Radiated Power. A new name of the metrics for Option 2 rather than TRP is needed. The proposed candidate name for Option 2 is Max EIRP Integrated Power, i.e. MEIP.

MEIP=14πθ=0π∅=02πmaxEIRPTPMI2θ,∅, EIRPTPMI3θ,∅,EIRPTPMI4θ,∅,EIRPTPMI5θ,∅sinθd∅dθ

Proposal 2: Define a new metrics name for Option 2, and the proposed candidate name is Max EIRP Integrated Power i.e. MEIP.

After defining the metrics and names of Option 1 and Option 2, the applicability of the two metrics is considered as below. Option 1 has the same metrics concept with traditional TRP, and derives relatively comparable metrics values of the radiation performance with 1Tx TRP. Therefore, it is proposed to specify Option 1 as the performance metrics of 2Tx TRP. In this case, the TRPs of both 1Tx UEs and 2Tx UEs can be treated and compared in the data pool. Regarding Option 2, it is clearly demonstrated that MEIP values will show UE enhancement with 2Tx radiators. Accordingly, it is proposed to further study the metrics in a future Work Item, rather than TRP TRS WI, to specify performance requirement for the UEs with enhanced features and/or capabilities.

Proposal 3: Specify Option 1 as the performance metrics of 2Tx TRP.

Proposal 4: Further study the metrics including MEIP in a future Work Item, rather than TRP TRS WI, to specify performance requirement for the UEs with enhanced features and/or capabilities.

- Conclusions

This contribution discusses the test methodology of Single-layer UL-MIMO for fully coherent UE. And provide the following proposals.

Proposal 1: The metrics name of Option 1 is proposed as TRPaverage_TPMI.

Proposal 2: Define a new metrics name for Option 2, and the proposed candidate name is Max EIRP Integrated Power i.e. MEIP.

Proposal 3: Specify Option 1 as the performance metrics of 2Tx TRP.

Proposal 4: Further study the metrics including MEIP in a future Work Item, rather than TRP TRS WI, to specify performance requirement for the UEs with enhanced features and/or capabilities.

- References

[1] R4-2317004 WF on NR_FR1_TRP_TRS_enh, vivo

[2] 3GPP TS 38.161