3GPP TSG-RAN WG4 Meeting #109	R4-2319916

Chicago, US, November 13 – 17, 2023

Title:	Study of number of slots for low band FR1 MIMO OTA

Source:	OPPO

Agenda item:	8.16.3

Document for:	Approval

- Introduction

In the last RAN4#108bis meeting, the minimum number of slots for low bands MIMO OTA was discussed. The agreement was that consider to reduce the number of slots to 10k for bands <1GHz based on more measurement data, and conclude the issue no later than RAN4#109 meeting [1].

This contribution presents the measurement data and comparison of Band n28 MIMO OTA measurement with slot number of 10k and 20k separately, and propose to adopt minimum number of slots using 10k.

- Discussion

To comparison the impact on low band MIMO OTA performance, we measure two devices with slot number of 20k and 10k with three test scenarios, i.e. DMP, DML, DMSU. The measurement data is illustrated in the following figure.

From the comparison, the measurement deltas on the test scenarios of DMP and DML are relatively small, i.e. 0.2~0.3dB. While, the delta of DMSU scenario is around 0.8~0.9dB. Besides the test scenario based comparison, the TRMSs of the two devices are also compared as below.

The deltas of TRMS for the two devices are less than 0.4dB.

Observation 1: The deltas of TRMS with measurement slot number of 20k and 10k separately are less than 0.4dB.

The test time of two slot number configurations is also compared. The test times of two devices are reduced by 36.4% and 10.9% separately. The difference of test time reduction between two devices is mainly relying on the connection stability during the measurement. Device B suffers from more connection loss during the measurement, which cancel and conceal the benefit from reduced test slot number. Generally, it can achieve around 1/3 test time reduction with 10k measurement slots.

Observation 2: It can achieve around 1/3 test time reduction with 10k measurement slots.

Considering the measurement deviation of maximum 0.4dB and 1/3 test time reduction, it is proposed to adopt 10k as minimum number of slots per stream for MIMO OTA measurements bands < 1GHz.

Proposal 1: It is proposed to adopt 10k as minimum number of slots per stream for MIMO OTA measurements of bands < 1GHz.

- Conclusions

This contribution presents the measurement data and comparison of Band n28 MIMO OTA measurement with slot number of 10k and 20k separately. And provide the following proposals.

Observation 1: The deltas of TRMS with measurement slot number of 20k and 10k separately are less than 0.4dB.

Observation 2: It can achieve around 1/3 test time reduction with 10k measurement slots.

Proposal 1: It is proposed to adopt 10k as minimum number of slots per stream for MIMO OTA measurements of bands < 1GHz.

- References

[1] R4-2316945, WF on NR_MIMO_OTA_enh, CAICT