3GPP TSG-RAN WG4 Meeting #109	R4-2319917

Chicago, US, November 13 – 17, 2023

Title:	Discussion on the re-positioning for multi-Rx

Source:	OPPO

Agenda item:	8.2.2

Document for:	Approval

- Introduction

In the last RAN4 #108bis meeting, the issue of repositioning was discussed, and there was no consensus on whether adopting the re-positioning concept or not [1].

Considering this is the last meeting for the Multi-Rx test methodology SI, this contribution presents our view on the re-positioning concept and proposes to conclude the issue in this meeting.

- Discussion

The re-positioning concept is applied to reduce the effect of signal ripple which is generally caused by the positioner in the chamber. The key check point that whether the re-positioning concept needed is to inspect whether the positioner or pedestal will be placed between the DUT and the measurement probes of AoA1 and AoA2. Regarding there are two measurement probes, i.e. AoA1 and AoA2, and AoA1 as the reference test point sweeps around the whole sphere together with AoA2 with the required AoA separation, it is unavoidable that one of AoAs is blocked by the positioner for some test points, i.e. an AoA is at/around the position of θ=180 degree with respect to the reference coordinate system. Take an example as below. Supposing the required AoA separation is 120 degree, when AoA1 is swept to the test point of (30, ϕ), the AoA2’s position is (180, ϕ).

Observation 1: It is unavoidable that one of AoAs is blocked by the positioner for some test points.

Proposal 1: It is necessary to adopt the re-positioning concept for multi-Rx measurement to avoid AoA blockage.

It should be notice that when adopting the re-positioning approach, the location of AoA2 should be reorganized to keep the location of the AoA pair aligned with the intended AoA pair to be measured. The illustration is show below. For the reference positioning, the AoA2 equals to AoA1 plus AoA separation on θ angle. While re-positioning approach adopted, the AoA2 should equal to AoA1 minus AoA separation on θ angle for the same AoA pair to be measured.

Proposal 2: when adopting the re-positioning approach, the location of AoA2 should be reorganized to keep the location of the AoA pair aligned with the intended AoA pair to be measured.

- Conclusions

This contribution presents our view on the re-positioning concept for multi-Rx measurement. And provide the following proposals.

Observation 1: It is unavoidable that one of AoAs is blocked by the positioner for some test points.

Proposal 1: It is necessary to adopt the re-positioning concept for multi-Rx measurement to avoid AoA blockage.

Proposal 2: when adopting the re-positioning approach, the location of AoA2 should be reorganized to keep the location of the AoA pair aligned with the intended AoA pair to be measured.

- References

[1] R4-2317002 WF on FS_NR_FR2_OTA_en, Qualcomm Incorporated