# 3GPP TSG-RAN WG4 Meeting # 109	R4-2319926

# Chicago, US, November 13 – 17, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | xxxx0 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TS38.101-1 on SL-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This draftCR is to finalized the SL-U requirement. The MPR and A-MPR part are still missing. The “uplink configuration” has ambiguity and was pointed out during the draftCR procedure hence it is aligned with NR sidelink. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Subclause 6.2E.2F for SL-U MPR is added. 2, Subclause 6.2E.3F for SL-U A-MPR is added. 3, Titile of table 7.3E.2F.2-3 is changed from “uplink configuration” to “Sidelink TX configuration” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for SL-U is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.2F, 6.2E.3F,7.3E.2F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<start of change>>

6.2E.2F	UE maximum output power reduction for Sidelink Unlicensed

6.2E.2F.1	MPR for Power class 5 Sidelink Unlicensed UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for NR V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2E.2F.1-1 for Power class 5 NR Sidelink Unlicensed UE

Table 6.2E.2F.1-1: Maximum Power Reduction (MPR) for PSSCH/PSCCH

For the PSFCH transmission for NR Sidelink Unlicensed UE, the UE allow the follow MPR requirements specified in Table 6.2E.2F.1-2.

Table 6.2E.2F.1-2: Maximum Power Reduction (MPR) for PSFCH

For the S-SSB transmission, the UE allow the follow MPR requirements specified in Table 6.2E.2F.1-3.

Table 6.2E.2F.1-3: Maximum Power Reduction (MPR) for S-SSB

<<end of 1st change>>

<<start of 2nd change>>

6.2E.3F	UE additional maximum output power reduction for Sidelink Unlicensed

6.2E.3F.1	General

For the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in clause 6.2E.2F and A-MPR requirements specified in current clause.

Additional emission requirements can be indicated by the network or pre-configured radio parameters. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field [additionalSpectrumEmission]. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR V2X frequency band number of the applicable operating band, the IE field [freqBandIndicatorNR] and an associated value of [additionalSpectrumEmission] in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1.

Table 6.2E.3F.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2F.3.1-1A and 6.2F.3.1-1B.

Table 6.2E.3F.1-1: Additional Maximum Power Reduction (A-MPR)

| Network Signalling value | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_31 | 6.5F.3.3.4 | n46 | 20, 40, 60, 80 |  | 6.2E.3F.2 |
| NS_53 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2E.3F.3 |
| NS_58 | 6.5F.3.3.6 | n102 | 20, 40, 60, 80 |  | 6.2E.3F.4 |
| NS_60 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2E.3F.5 |
| NS_61 | 6.5F.3.3.7 | n96 | 20, 40, 60, 80 |  | 6.2E.3F.6 |

6.2E.3F.2	A-MPR for V2X UE by NS_31

When "NS_31" is indicated in the cell, the A-MPR is specified in Table 6.2E.3F.2-1.

Table 6.2E.3F.2-1: A-MPR for PSSCH/PSCCH by NS_31

For the simultaneous PSFCH transmission when NS_31 is indicated by the network or pre-configured radio parameters for NR Sidelink Unlicensed UE, the UE allow the follow A-MPR requirements specified in Table 6.2E.3F.2-2.

Table 6.2E.3F.2-2: A-MPR for simultaneous PSFCH by NS_31

For the S-SSB transmission when NS_31 is indicated by the network or pre-configured radio parameters for NR Sidelink Unlicensed UE, the UE allow the follow A-MPR requirements specified in Table 6.2E.3F.2-3.

Table 6.2E.3.2-3: A-MPR for S-SSB transmission by NS_31

6.2E.3F.3	A-MPR for V2X UE by NS_53

When "NS_53" is indicated in the cell, the A-MPR is specified in Table 6.2E.3F.3-1.

Table 6.2E.3F.3-1: A-MPR for PSSCH/PSCCH by NS_53

For the simultaneous PSFCH transmission when NS_53 is indicated by the network or pre-configured radio parameters for NR Sidelink Unlicensed UE, the UE allow the follow A-MPR requirements specified in Table 6.2E.3F.3-2.

Table 6.2E.3F.3-2: A-MPR for simultaneous PSFCH by NS_53

For the S-SSB transmission when NS_53 is indicated by the network or pre-configured radio parameters for NR Sidelink Unlicensed UE, the UE allow the follow A-MPR requirements specified in Table 6.2E.3F.3-3.

Table 6.2E.3F.3-3: A-MPR for S-SSB transmission by NS_53

6.2E.3F.4	A-MPR for V2X UE by NS_58

When "NS_58" is indicated in the cell, the A-MPR is specified in Table 6.2E.3F.4-1.

Table 6.2E.3F.4-1: A-MPR for PSSCH/PSCCH by NS_58

For the simultaneous PSFCH transmission when NS_58 is indicated by the network or pre-configured radio parameters for NR Sidelink Unlicensed UE, the UE allow the follow A-MPR requirements specified in Table 6.2E.3F.4-2.

Table 6.2E.3F.4-2: A-MPR for simultaneous PSFCH by NS_58

For the S-SSB transmission when NS_58 is indicated by the network or pre-configured radio parameters for NR Sidelink Unlicensed UE, the UE allow the follow A-MPR requirements specified in Table 6.2E.3F.4-3.

Table 6.2E.3F.4-3: A-MPR for S-SSB transmission by NS_58

6.2E.3F.5	A-MPR for V2X UE by NS_60

When "NS_60" is indicated in the cell, the A-MPR is specified in Table 6.2E.3F.5-1.

Table 6.2E.3F.5-1: A-MPR for PSSCH/PSCCH by NS_60

For the simultaneous PSFCH transmission when NS_60 is indicated by the network or pre-configured radio parameters for NR Sidelink Unlicensed UE, the UE allow the follow A-MPR requirements specified in Table 6.2E.3F.5-2.

Table 6.2E.3F.5-2: A-MPR for simultaneous PSFCH by NS_60

For the S-SSB transmission when NS_60 is indicated by the network or pre-configured radio parameters for NR Sidelink Unlicensed UE, the UE allow the follow A-MPR requirements specified in Table 6.2E.3F.5-3.

Table 6.2E.3F.5-3: A-MPR for S-SSB transmission by NS_60

6.2E.3F.6	A-MPR for V2X UE by NS_61

When "NS_61" is indicated in the cell, the A-MPR is specified in Table 6.2E.3F.6-1.

Table 6.2E.3F.6-1: A-MPR for PSSCH/PSCCH by NS_61

For the simultaneous PSFCH transmission when NS_61 is indicated by the network or pre-configured radio parameters for NR Sidelink Unlicensed UE, the UE allow the follow A-MPR requirements specified in Table 6.2E.3F.6-2.

Table 6.2E.3F.6-2: A-MPR for simultaneous PSFCH by NS_61

For the S-SSB transmission when NS_61 is indicated by the network or pre-configured radio parameters for NR Sidelink Unlicensed UE, the UE allow the follow A-MPR requirements specified in Table 6.2E.3F.6-3.

Table 6.2E.3F.6-3: A-MPR for S-SSB transmission by NS_61

6.2E.3F.7	A-MPR for V2X con-current operation

For the inter-band con-current NR V2X operation, the allowed additional maximum power reduction (A-MPR) for the maximum output power shall be applied per each component carrier. The A-MPR requirements in clause 6.2.3 apply for NR Uu operation in licensed band, and the A-MPR requirements in clause 6.2E.3F apply for NR sidelink operation in Band n46, n96 and n102.

<<end of 2nd change>>

<<start of 3rd change>>

7.3E.2F	Minimum requirements for Sidelink Unlicensed

7.3E.2F.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

In later clauses of Clause 7 where the value of REFSENS is used as a reference to set the corresponding requirement, the UE shall be verified against those requirements by applying the REFSENS value in Table 7.3E.2F.2-1 with 2 Rx antenna ports tested.

7.3E.2F.2	Reference sensitivity power level

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters specified in Table 7.3E.2F-3.

with parameters specified in Table 7.3E.2F.2-1, Table 7.3E.2F.2-2, and Table 7.3E.2F.2-3.

Table 7.3E.2F.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n46 | 15 | 20, 40 | -89.2 + 10log10(NRB/105) | TDD |
|  | 30 | 20, 40, 60, 80, 100 | -89.4 + 10log10(NRB/50) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.6 + 10log10(NRB/24) |  |
| n96, n102 | 15 | 20, 40 | -88.7 + 10log10(NRB/105) | TDD |
|  | 30 | 20, 40, 60, 80, 100 | -88.9 + 10log10(NRB/50) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.1 + 10log10(NRB/24) |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 7.3E.2F.2-3 |  |  |  |  |

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3E.2F.2-1 shall be modified by the amount given in ΔRIB,4R in Table 7.3E.2F.2-2 for the applicable operating bands.

Table 7.3E.2F.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n46, n96, n102 | -2.2 |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3E.2F.2-1 and Table 7.3E.2F.2-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3E.2F.2-3.

Table 7.3E.2F.2-3: Sidelink TX configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 20 MHz (dBm) | 40 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 100 MHz (dBm) |
| n46 | 15 | 105 | 216 |  |  |  |
|  | 30 | 50 | 105 | 160 | 216 | 270 |
|  | 60 | 24 | 50 | 75 | 105 | 135 |
| n96, n102 | 15 | 105 | 216 |  |  |  |
|  | 30 | 50 | 105 | 160 | 216 | 270 |
|  | 60 | 24 | 50 | 75 | 105 | 135 |

Unless given by Table 7.3E.2F.2-4, the minimum requirements specified in Tables 7.3E.2F.2-1 and 7.3E.2F.2-2 shall be verified with the network signalling value NS_01 (Table 6.2F.3.1-1) configured.

Table 7.3F.2-4: Network signaling value for reference sensitivity

| Operating band | Network Signalling value |
| --- | --- |
| n46 | NS_01 |
| n96 | NS_53 |
| n102 | NS_01 |

<<end of change>>