3GPP TSG-RAN WG4 Meeting # 109							R4-2319933

Chicago, US, November 13 – 17, 2023

Agenda Item:	8.30.1

Source: 	OPPO

Title: 	on the SL-U MRP simulation

Document for:	Approval

- Introduction

The sidelink evolution WID has been fully discussed in Rel-18 and has already come to be finished. In such case, it has been proposed in this paper to further discuss the UE feature list for SL evolution.

- Discussion

For the Rel-18 sidelink evolution WID, there are currently three threads including four different topics as sidelink unlicensed, sidelink CA, concurrent sidelink unlicensed operation and LTE/NR sidelink co-existence. We would like to go with the already agreed UE capability first and then to go over some other features that are still under discussion.

During the last RAN4 meeting, an LS [1] has been agreed and sent to RAN2. Below captures the detail LS content. It has been agreed in RAN4 that to introduce the UE capability of SL CA

RAN4 agreement for SL CA

- Define Power Class 3 for SL CA in Rel-18

- Specify PEMAX,CA for SL CA

  - PEMAX,CA is not applied to S-SSB

Issue 1. Based on the RAN4 agreement above, a capability of UE power class needs to be defined per BC (Band Combination) as NR CA.

The following example can be considered as a capability of UE power class for SL CA.

Table 1: Capability of SL CA power class

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| powerClassSidelink-r18 This parameter indicates power class the UE supports when operating according to this band combination used for sidelink. If the field is absent, the UE supports the default power class. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClassSidelink-r16 in BandNR), the latter determines maximum TX power available in each band. The UE sets the power class parameter only in band combinations that are applicable as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. | BC | No | N/A | FR1 only |

In such case, the UE power class for sidelink CA should be defined as a RAN4 sidelink evolution feature.

Proposal 1: To define the sidelink CA power class feature.

Furthermore, during the discussion of sidelink unlicensed, there has been discussion on the intra-cell guard band design as well as the corresponding singling. The requirement is agreed in the WF [2] and captured as below:

Issue 2-6-1: Wideband operation and Intra-cell guard bands definition

- Proposals:

  - Option 1: Reuse the NR-U requirements for wideband operation and Intra-cell guard bands in 38.101-1 clause 5.3.3 for NR SL-U.

  - Option 2: In SL-U, intraCellGuardBandsSLTX-List and intraCellGuardBandsSLRX-List can be introduced.

Way forward:

  - Option 1 is agreed.

  - FFS on Option 2

Observation 1: The intra-cell guard band requirement has been defined for sidelink unlicensed as similar to NRU.

The specific signalling design is FFS and there is no more discussion on such topic. If we go back to the NR-U feature list in Rel-16, the feature is captured in [3] as shown below:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. NR-based access to unlicensed spectrum | 4-1 | DL reception in intra-carrier guardband | Capability of reception in the non-zero intra-cell guardband between contiguous RB sets in DL wideband carrier operation wider than 20MHz when LBT is successful only in a subset of RB sets | 4-2 | yes | no | UE cannot receive in the intra-cell guardband specified in 38.101-1 | per Band | No | No |  |  | Optional with capability signalling |
|  | 4-2 | DL reception when gNB does not transmit on all RB sets of a carrier as a result of LBT | Capability of reception in a wideband carrier when LBT is successful in a subset of the configured RB sets, which are either contiguous or non-contiguous, of [the carrier]. |  | yes | no | UE can guarantee the performance only when gNB transmission occupies all RB sets. | per Band | No | No |  | There is no restriction for gNB to schedule in mode 2 or mode 3 | Optional with capability signalling |

The feature 4-1 as DL reception in intra-carrier guard band is the “Capability of reception in the non-zero intra-cell guard band between contiguous RB sets in DL wideband carrier operation wider than 20MHz when LBT is successful only in a subset of RB sets”. This is an per band capability and optional with capability signalling.

Observation 2: The NRU feature list 4-1 as DL reception in intra-carrier guard band is defined based on the intra-cell guard band as optional with capability signaling.

In this sense, it is proposed also introduce the optional feature with capability signalling for sidelink unlicensed intra-carrier guard band reception.

Proposal 2: It is proposed to introduce the optional feature with capability signalling for sidelink unlicensed intra-carrier guard band reception.

With the above two proposals, the feature list for sidelink evolution WID is proposed in the last as Annex.

Proposal 3: To agree on the feature list for sidelink evolution.

3	Conclusions

In this contribution, we give initial discussion on the sidelink evolution and the observation and proposals are shown as below:

Observation 1: The intra-cell guard band requirement has been defined for sidelink unlicensed as similar to NRU.

Observation 2: The NRU feature list 4-1 as DL reception in intra-carrier guard band is defined based on the intra-cell guard band as optional with capability signaling.

Proposal 1: To define the sidelink CA power class feature.

Proposal 2: It is proposed to introduce the optional feature with capability signalling for sidelink unlicensed intra-carrier guard band reception.

Proposal 3: To agree on the feature list for sidelink evolution.

4 References

[1] R4-2317751		LS on a capability of UE power class and IE on PEMAX,CA for SL CA, LGE, OPPO

[2] R4-2306630		WF on SLU single CC system parameters, VIVO

[3] R4-2016850		Updated RAN4 UE features list for Rel-16, CMCC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. # NR_SL_enh2 | 45-1 | SL reception in intra-carrier guard band | Capability of reception in the non-zero intra-cell guardband between contiguous RB sets in SL wideband carrier operation wider than 20MHz when LBT is successful only in a subset of RB sets |  | Yes | Yes | UE cannot receive in the intra-cell guard band specified in 38.101-1 | Per band | No | No |  |  | Optional with capability signalling |
|  | 45-2 | Power class for sidelink CA | power class the UE supports when operating according to this band combination used for sidelink. If the field is absent, the UE supports the default power class. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClassSidelink-r16 in BandNR), the latter determines maximum TX power available in each band. The UE sets the power class parameter only in band combinations that are applicable as specified in TS 38.101-1. |  | Yes | Yes | UE cannot transmit in proper power class as specified in 38.101-1 | Per BC | No | No |  |  | Optional with capability signalling |