3GPP TSG-RAN WG4 Meeting # 109							R4-2319935

Chicago, US, November 13 – 17, 2023

Agenda Item:	7.31.2

Source: 	OPPO

Title: 	on the remaining issue for NTN LS band

Document for:	Approval

- Introduction

During the last RAN4 meeting, the NTN LS band topics have been fully discussed. Besides the controversial requirements such as A-MPR, still many requirements have not been settled down yet. Hence in this paper, we try to give some further discussion and finalize the requirement.

- Discussion

Currently in TR 38.741 v2.1.0, for TX requirements,

Table 1 TX requirements for band n254

| (sub) Section in TS 38.101-5 | Discussed or not for L+S NTN band |
| --- | --- |
| 6.2.1 Maximum output power | Agreed |
| 6.2.2 MPR | Agreed |
| 6.2.3 A-MPR | No |
| 6.2.4 Configured transmitted power | No discussion yet |
| 6.3.1 Minimum output power | No discussion yet |
| 6.3.2 Transmit OFF power | No discussion yet |
| 6.3.3 Transmit ON/OFF time mask | No discussion yet |
| 6.3.4 Power control | No discussion yet |
| 6.4.1 Frequency error | No discussion yet |
| 6.4.2 Transmit modulation quality | No discussion yet |
| 6.5.1 Occupied bandwidth | No discussion yet |
| 6.5.2 Out of band emission | No discussion yet |
| 6.5.3 Spurious emission | Agree on co-existence and additional spurious emission |
| 6.5.4 Transmit intermodulation | No discussion yet |

For the TX requirement parts, we see many of the requirements are left as no discussion yet. After a quick look, we see it simple way that currently these requirements left as “no discussion yet” are band agnostic and hence there is no need to further discuss the requirements specifically for band n254. The legacy requirements as already in TS 38.101-5 can be reused。

Observation 1: The TX requirement listed in table 1 as “No discussion yet” are band agnostic.

Proposal 1: For the TX requirement listed in table 1 as “No discussion yet”, Reuse the legacy requirement in TS 38.101-5.

Similarly, we try to go over the RX requirements which is shown below in table 2.

Table 2 RX requirements for band n254

| (sub) Section in TS 38.101-5 | Discussed or not for L+S NTN band |
| --- | --- |
| 7.3 Reference sensitivity | Agreed |
| 7.4 Maximum input level | No discussion yet |
| 7.5 Adjacent channel selectivity | No discussion yet |
| 7.6 Blocking characteristics | Agreed |
| 7.7 Spurious response | No discussion yet |
| 7.8 Intermodulation characteristics | No discussion yet |
| 7.9 Spurious emissions | No discussion yet |

Similar as TX requirements, above table 2 listed the RX requirements. We have similar observation that the listed in table 2 as” No discussion yet” requirements are also band agnostic and hence legacy requirement in TS 38.101-5 can be reused.

Observation 2: The RX requirement listed in table 2 as “No discussion yet” are band agnostic.

Proposal 2: For the RX requirement listed in table 2 as “No discussion yet”, Reuse the legacy requirement in TS 38.101-5.

3	Conclusions

In this contribution, we give initial discussion on the sidelink evolution and the observation and proposals are shown as below:

Observation 1: The TX requirement listed in table 1 as “No discussion yet” are band agnostic.

Proposal 1: For the TX requirement listed in table 1 as “No discussion yet”, Reuse the legacy requirement in TS 38.101-5.

Observation 2: The RX requirement listed in table 2 as “No discussion yet” are band agnostic.

Proposal 2: For the RX requirement listed in table 2 as “No discussion yet”, Reuse the legacy requirement in TS 38.101-5.

4 References

[1] R4-2317727		WF on NR_SL_enh2_UERF_part2, LG Electronics