# 3GPP TSG-RAN WG4 Meeting # 109	R4-2319938

# Chicago, US, November 13 – 17, 2023

Agenda Item:	8.21.2

Source: 	OPPO

Title: 	On specific issues related to use case for AIML

Document for:	Approval

- Introduction

RAN4 scope in the R18 AI/ML SID is listed as below[1]:

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition

In the recent RAN1 meetings,

- A basic R18 (also targeting future 6G) AI/ML framework is considered, including

- Discussion on life cycle management (LCM), performance monitoring, data collection, model/data ID, AI capability, model privacy, etc.

- Six representative sub use cases have been identified, including

- CSI: CSI compression, Time domain CSI prediction

- Beam management: Spatial domain beam prediction, Temporal domain beam prediction

- Positioning: Direct AI/ML positioning, AI/ML assisted positioning

- Performance evaluation methodologies(EVM) are confirmed for the identified sub use cases, including

- Performance evaluation methodologies for different sub use cases

- Simulation results under different EVM assumptions for different sub use cases

In RAN4 #107bis meeting, it has been agreed that all the 6 sub-use cases will be handled in RAN4. In this contribution, we will discuss the related specific issues in detail.

- Discussion

  - Use Cases

For RAN4 performance test, at least two aspects need to be considered: (1) model/functionality input and (2) model/ functionality output.

- For input, the focus of testing is on whether the input information of the model/functionality could be correctly obtained, e.g. measurement accuracy of reference signals that relates CSI/BM/Positioning input should be checked.

- For output, the focus of testing is on whether the performance of a given model/functionality could be guaranteed, e.g. by testing some intermediate results or eventual KPIs.

Proposal 1: For RAN4 performance test, at least two aspects need to be considered:

- Model/functionality input, the focus of testing is on whether the input information of the model/functionality could be correctly obtained

- Model/functionality output, the focus of testing is on whether the performance of a given model/functionality could be guaranteed

    - AI/ML based CSI feedback enhancement

In the recent RAN1 meetings, after several rounds of discussion on the representative sub-use cases of CSI feedback enhancement, CSI compression and time domain CSI prediction were selected as typical sub-use cases that need to be further studied.

Figure 1: CSI feedback compression

For AI/ML based CSI compression, a typical architecture is depicted in figure 1, where the CSI generation part and CSI reconstruction part are deployed at the UE and NW, respectively. Regarding the performance monitoring metrics/methods, several options have been agreed in RAN1 for further study, e.g. the system throughput, the intermediate KPIs as monitoring metrics (e.g., square of generalized cosine similarity (SGCS).

Figure 2.  CSI prediction

In RAN#111 meeting, the time domain CSI prediction has been agreed to be another representative sub use case for further evaluation. As shown in figure 2, when the CSI is derived at time T and is utilized at time T+Xms(e.g. X=1,2,3,4), the CSI prediction would be needed to compensate the difference of channel state caused by the scheduling delay.

Regarding the model/functionality input, precoding matrix (eigenvector of the raw channel matrix estimated by UE) and explicit channel matrix (raw channel matrix estimated by UE) are two candidate input types that agreed in RAN1 for further study.

Regarding the model/functionality output and performance monitoring, intermediate KPIs (e.g. SGCS for CSI compression or prediction) and eventual KPIs (e.g. throughput) are agreed in RAN1 for evaluation.

In RAN4, from our understanding, following aspects need to be studied,

- Input related tests: performance requirement on CSI model/functionality input (e.g. CSI-RS measurement accuracy) should be studied.

- Output related tests:

- As agreed in RAN4#107, throughput should be used to evaluate the model inference performance.

- Existed RAN4 test examples for “reporting of PMI” can serve as a reference, e.g. as captured in 38.101-4, “The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reported PMI compared to the case when the transmitter is using random precoding, respectively.” Requirement of γ and test settings can be reused or updated.

- Performance monitoring

- For performance monitoring, UE/NW can monitor and estimate the performance of AI/ML based CSI model/functionality through Hypothetical BLER or intermediate KPIs, e.g. SGCS.

- Multiple samples within an evaluation window should be used to obtain a stable monitoring result.

- FFS how to perform cell level AI/ML model performance monitoring, considering multi-user involved mechanism, e.g. should the AI/ML model be updated or optimized if only a small number of UEs report its failure?

Proposal 2: For AI/ML based CSI feedback, performance requirement on CSI model/functionality input (e.g. CSI-RS measurement accuracy) should be studied.

Proposal 3: For CSI inference performance, throughput should be used to evaluate the model inference performance, and existed RAN4 test examples for “reporting of PMI” can be reused or serve as a reference. Requirement of γ and test settings can be reused or updated.

Proposal 4: For CSI performance monitoring, UE can monitor and estimate the performance of AI/ML based CSI model/functionality through Hypothetical BLER or intermediate KPIs, e.g. SGCS.

- FFS how to perform cell level AI/ML model/functionality performance monitoring

    - AI/ML based Beam management

Beam management(BM)-Case1 is agreed in RAN1, i.e. Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams. As example is shown in figure 3, the best beam(s) can be estimated through AI/ML based BM methods with L1 measurements on a beam subset.

Figure 3.  Spatial-domain DL beam prediction

Another sub use case is beam management(BM)-Case2, i.e. Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams. As example is shown in figure 4, The best beam in the future can be predicted through AI/ML based BM methods based on the beam quality at the current and historical time.

Figure 4. Temporal DL beam prediction

Regarding the model/functionality input, measurements on a sub set of beams or historical beams are utilized.

Regarding the model/functionality output and performance monitoring, intermediate KPIs (e.g. beam prediction accuracy, including beam prediction accuracy for Tx beam, Tx-Rx beam pair, L1-RSRP difference of the predicted beam) are used in RAN1 for performance evaluation, and system performance related KPIs(e.g. throughput) can also be considered and agreed in RAN1.

In RAN4, from our understanding, following aspects need to be studied,

- Input related tests: performance requirement on BM model/functionality input (e.g. beam measurement accuracy) should be studied.

- Output related tests:

- RSRP accuracy of the predicted beam(s) should be used for BM model inference performance tests.

- The beam prediction based on AI/ML essentially reduces the measurement by using AI/ML to predict the measurement results instead of actual measurements. Therefore, the most direct testing metric is to evaluate whether the predicted measurement accuracy meets the requirements.

- Performance monitoring

- For performance monitoring, UE can monitor and estimate the performance of AI/ML based BM model/functionality through RSRP accuracy as well.

- Whether/how to perform cell level BM model performance monitoring(multi-user involved mechanisms) should be considered, e.g. should the AI/ML model be updated or optimized if only a small number of UEs report its failure?

Proposal 5: For AI/ML based BM, performance requirement on BM model/functionality input (e.g. beam measurement accuracy) should be studied.

Proposal 6: For BM inference performance, RSRP accuracy of the predicted beam(s) should be used for BM model inference performance tests.

Proposal 7: For BM performance monitoring, UE can monitor and estimate the performance of AI/ML based CSI model/functionality through RSRP accuracy.

- FFS how to perform cell level AI/ML model/functionality performance monitoring

    - AI/ML based positioning

For AI/ML based positioning, direct AI/ML positioning and AI/ML assisted positioning are discussed and agreed in RAN1. For direct AI/ML positioning, the output of AI model is the estimated location. For AI/ML assisted positioning, the output of the AI model is the input of legacy positioning methods, and the location can be estimated through a two-stage approach as shown in figure 5.

Figure 5. Examples of AI/ML based positioning

Regarding the model input, potential new measurement (e.g. CIR/PDP) and existing measurement (e.g., RSRP/RSRPP/RSTD) are considered in RAN1 evaluation.

Regarding the model output,

- For direct AI/ML positioning and AI/ML assisted positioning, the performance KPI is the CDF percentiles of horizonal accuracy in RAN1. The CDF percentiles to analyze are: 90% (baseline) and {50%, 67%, 80%} (optional).

- Additionally, for AI/ML assisted positioning, the model output could be RSTD, identification of LoS/NLoS, and other metrics that agreed in RAN1 R18 AI/ML study.

In RAN1 #110bis meeting, following 5 cases of AI/ML based positioning accuracy enhancement are agreed for further study,

- Case 1: 		UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

- Case 2a: 	UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

- Case 2b: 	UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

- Case 3a: 	NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: 	NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

For case 1 and case2a, the AI/ML model is deployed at UE, both model input and model output may need data/reference signal transmission between UE and NW. For case 2b, case 3a and case 3b, the AI/ML model is deployed at NW side(e.g., gNB or LMF), only model input may need data transmission from UE to NW.

Considering the aforementioned agreements and research findings, following aspects require further study for AI/ML-based positioning tests in RAN4.

- Input related tests: performance requirement on positioning model/functionality input(e.g., measurements for positioning inputs: CIR/PDP/RSRP/RSTD) should be studied for all AI/ML positioning cases, i.e., for case 1/2a/2b/3a/3b.

- Output related tests:

- For case1(UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning),

- For UE-side direct AI/ML positioning and UE-side AI/ML assisted positioning, positioning accuracy should be taken as the KPI.

Regarding the concerns on how to perform tests for positioning accuracy in RAN4, from our understanding, “Positioning test data set(s)” prepared in advance by TE could be considered. The positioning test data set(s) can include both model input-related data (e.g. measurement results of PRS matched with positions, or different model inputs obtained from the measurement results) and positioning label data. TE transmits the model input-related data to UE, and then UE outputs the positioning results back to TE. By this way, TE can compare the UE's output positioning results with the local positioning label data to determine the positioning accuracy. This approach can serve as a starting point for further research on model output-related tests for positioning.

- In addition, considering the model output could be RSTD, identification of LoS/NLoS and other metrics that agreed in RAN1 R18 AI/ML study, the accuracy for these intermediate results could be considered for UE-side AI/ML assisted positioning.

- Note: most of these intermediate measurement results are not realistic measurement results. They are from AI/ML model output(with non-linear processing). How to get the label data(the expected intermediate results) and test these intermediate measurement results is also a new and a more challenge work in RAN4. Feasibility of using these candidate metrics should be clarified in WI.

- For case2a(UE-assisted/LMF-based positioning with UE-side model),

- The UE side model output could be RSTD, identification of LoS/NLoS and other metrics that agreed in RAN1 R18 AI/ML study. The accuracy for these intermediate results could be considered. But as discussed in case1, the feasibility of using these candidate metrics should be clarified in WI.

- For case2b,3a,3b (cases without UE-side model),

- Not necessary to test the Positioning model/functionality outputs in RAN4.

Proposal 8: For AI/ML based Positioning, performance requirement on Positioning model/functionality input (e.g. measurement accuracy of CIR/PDP/RSRP/RSTD) should be studied for all AI/ML positioning cases, i.e., for case 1/2a/2b/3a/3b.

Proposal 9: For case1, positioning accuracy should be utilized as the KPI to test the model/functionality output.

Proposal 10: Regarding RAN4 tests on positioning accuracy, using positioning test data set(s) which including model input-related data (e.g. measurement results of PRS that matched with positions, or model inputs obtained from the measurement results) and positioning label data, prepared in advance by TE could be considered.

Proposal 11: For case2 and UE-side AI/ML assisted positioning in case1, considering the accuracy for intermediate results could be considered as the KPI to test the model/functionality output.

Note: most of the intermediate measurement results agreed in RAN1 are not realistic measurement results. They are from AI/ML model output(with non-linear processing). How to get the label data(the expected intermediate measurement results) and test these intermediate measurement results is also a new and a more challenge issue in RAN4. Feasibility of using these candidate metrics should be studied.

Proposal 12: For case2b,3a,3b (cases without UE-side model), not necessary to test the Positioning model/functionality outputs.

- Conclusions

In this contribution, we discussed the Rel-18 AI/ML impacts to RAN4 and got following proposals

Proposal 1: For RAN4 performance test, at least two aspects need to be considered:

- Model/functionality input, the focus of testing is on whether the input information of the model/functionality could be correctly obtained

- Model/functionality output, the focus of testing is on whether the performance of a given model/functionality could be guaranteed

Proposal 2: For AI/ML based CSI feedback, performance requirement on CSI model/functionality input (e.g. CSI-RS measurement accuracy) should be studied.

Proposal 3: For CSI inference performance, throughput should be used to evaluate the model inference performance, and existed RAN4 test examples for “reporting of PMI” can be reused or serve as a reference. Requirement of γ and test settings can be reused or updated.

Proposal 4: For CSI performance monitoring, UE can monitor and estimate the performance of AI/ML based CSI model/functionality through Hypothetical BLER or intermediate KPIs, e.g. SGCS.

- FFS how to perform cell level AI/ML model/functionality performance monitoring

Proposal 5: For AI/ML based BM, performance requirement on BM model/functionality input (e.g. beam measurement accuracy) should be studied.

Proposal 6: For BM inference performance, RSRP accuracy of the predicted beam(s) should be used for BM model inference performance tests.

Proposal 7: For BM performance monitoring, UE can monitor and estimate the performance of AI/ML based CSI model/functionality through RSRP accuracy.

- FFS how to perform cell level AI/ML model/functionality performance monitoring

Proposal 8: For AI/ML based Positioning, performance requirement on Positioning model/functionality input (e.g. measurement accuracy of CIR/PDP/RSRP /RSTD) should be studied for all AI/ML positioning cases, i.e., for case 1/2a/2b/3a/3b.

Proposal 9: For positioning case1, positioning accuracy should be utilized as the KPI to test the model/functionality output.

Proposal 10: Regarding RAN4 tests on positioning accuracy, using positioning test data set(s) which including model input-related data (e.g. measurement results of PRS that matched with positions, or model inputs obtained from the measurement results) and positioning label data, prepared in advance by TE could be considered.

Proposal 11: For case2 and UE-side AI/ML assisted positioning in case1, considering the accuracy for intermediate results could be considered as the KPI to test the model/functionality output.

Note: most of the intermediate measurement results agreed in RAN1 are not realistic measurement results. They are from AI/ML model output(with non-linear processing). How to get the label data(the expected intermediate measurement results) and test these intermediate measurement results is also a new and a more challenge issue in RAN4. Feasibility of using these candidate metrics should be studied.

Proposal 12: For positioning case2b,3a,3b (cases without UE-side model), not necessary to test the Positioning model/functionality outputs.

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94-e, Dec. 2021