# 3GPP TSG-RAN WG4 Meeting # 109	R4-2319939

# Chicago, US, November 13 – 17, 2023

Agenda Item:	8.21.3

Source: 	OPPO

Title: 	On testability issues for two-sided AIML model

Document for:	Approval

- Introduction

In 3GPP RAN#94e meeting, a new study item (SI) on AI/ML for NR air interface in Rel-18 [1] was agreed. According to the SID, the study will focus on the general framework, evaluations for three typical use cases and other aspects relate to specification impacts.

RAN4 scope in the SID is listed as below:

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition

In the recent RAN1 meetings,

- A basic R18 (also targeting future 6G) AI/ML framework is considered, including

- Discussion on life cycle management (LCM), performance monitoring, data collection, model/data ID, AI capability, model privacy, etc.

- Six representative sub use cases have been identified, including

- CSI: CSI compression (two-sided model), Time domain CSI prediction

- Beam management: Spatial domain beam prediction, Temporal domain beam prediction

- Positioning: Direct AI/ML positioning, AI/ML assisted positioning

- Performance evaluation methodologies(EVM) are confirmed for the identified sub use cases, including

- Performance evaluation methodologies for different sub use cases

- Quantitative simulation results under different EVM assumptions for different sub use cases

In this contribution, we will discuss the RAN4 related issues, especially the testability of two-sided AI/ML models.

- Discussion

For a two-sided AI/ML model, the information transmitted through air-interface (e.g. PMI) is generated by AI/ML models (e.g. CSI encoder), rather than codebooks predefined by protocol. Regarding the testability of two-sided model, it is necessary to consider bring in a test model to cooperate with the model under test.

For example, a corresponding encoder/decoder model would be needed to cooperate with the decoder/encoder under test in order to monitor the decoding/encoding effect of the PMI. Only by well matching the models deployed on UE&NW can the performance of the AI/ML based two-sided solutions be guaranteed. Matching at different levels may result in different performance loss.

Proposal 1: Regarding the testability of two-sided model, should introduce test encoder(s) to collaborate with the  NW decoder under test.

Proposal 2: Regarding the testability of two-sided model, should introduce test decoder(s) to collaborate with the UE encoder under test.

In the previous RAN4 meetings, 4 options on test decoder have been proposed and discussed, i.e.,

- Option 1: test decoder is provided by the vendor of the encoder

- Option 2: test decoder is provided by the vendor of the decoder

- Option 3: The test decoder(s) are fully specified and captured in RAN4 spec.

- Option 4: The test decoder(s) are partially specified and captured in RAN4 spec.

According to our understanding, the pros and cons of different options for test decoder are listed below,

Table 1 pros and cons of different options for test decoder

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | DUT vendor | Decoder vendor (infra vendor in case of testing UEs) | RAN4 specifications | TE vendor, decoder developed based on RAN4 specifications |
| Source of decoder training data | Up to DUT vendor (no need to be specified) | Up to decoder implementer (infra vendor) FFS whether coordination with encoder vendor is required | Not needed, decoder fully specified (used as part of the RAN4 procedure to specify the decoder) | RAN4 specifications Alignment with UE/gNB vendors is required to avoid mismatch issue between the test decoder and the encoder under test |
| DUT vendor knowledge of the test decoder | Full knowledge | No or partial or enough or full knowledge based on alignment with infra vendors or specifications | Full knowledge based on the specifications | Partial knowledge – based on the RAN4 specification |
| Supported training collaboration type (source of training data should be consistent with the collaboration type) | In general, the way a model is trained can be decoupled from how the model is tested. However, for some training types, there are specific testing methods that can more closely match the deployment scenario. For example, for option1 and option2, (1) with type1 UE sided training(UE trains a encoder and a decoder, then UE transmits the decoder to NW), it is more reasonable to use option1 in RAN4 tests, where the UE provides the test decoder, because in practical use, the UE also provides the decoder. (2) with type1 NW sided training(NW trains a encoder and a decoder, then NW transmits the encoder to UE), it is more reasonable to use option2 in RAN4 tests, where the NW provides the test decoder, because in practical use, the NW also provides the decoder. Similarly, (3) with type3 UE first training(UE trains a encoder and a decoder, then UE transmits a data set to NW side, which can be used for decoder model training), it is more practical to use option1 in RAN4 testing, (4) with type3 NW first training(NW trains a encoder and a decoder, then NW transmits a data set to UE side, which can be used for encoder model training), it is more practical to use option2 in RAN4 testing. |  |  |  |
|  | Options: - Training up to DUT vendor. The way a model is trained can be decoupled from how the model is tested. - Only for Type1 UE sided training, and Type3 UE first training | Options: - Training up to DUT vendor. The way a model is trained can be decoupled from how the model is tested. - Only for Type1 NW sided training, and Type3 NW first training | Encoder training up to DUT vendor. The way a model is trained can be decoupled from how the model is tested. | Training up to DUT vendor. The way a model is trained can be decoupled from how the model is tested. |
| Test decoder verification procedure at TE and/or DUT | Needed | Needed | Needed | Needed |
| Feasibility of test decoder verification procedure | FFS | FFS | FFS | FFS |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | Low, depends on training data | Low, depends on training data As a data/scenario driven solution, AI/ML models be utilized in different cells may differ from each other. The limited number of test models can not reflect the real deployment (e.g. different cells/scenarios/ channel conditions) | Low, depends on training data | Low, depends on training data |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | High, TE needs to cope with multiple decoders from multiple UE vendors | Relatively high, TE needs to cope with multiple decoders from multiple NW vendors | Low, TE needs to provide support for a limited number of test decoders that specified and captured in RAN4 spec. | Medium/Low, TE needs to provide support for test decoders that specified(partially) and captured in RAN4 spec, training on TE may need. |
| Specification Effort (e.g. test decoder) | Low, Some conditions on the test decoder might be needed to ensure it can be implemented by TE. Some assisted information on the test decoder might be needed to ensure the alignment between the test decoder and the encoder under test. | Low, Some conditions on the test decoder might be needed to ensure it can be implemented by TE. Some assisted information on the test decoder might be needed to ensure the alignment between the test decoder and the encoder under test. | High, Consensus of a test model(s) in RAN4 is a challenging task. | Medium, Less heavy workload than Option 3 Some assisted information on the test decoder might be needed to ensure the alignment between the test decoder and the encoder under test. |
| Confidentiality/IP issues | Need to be considered | Need to be considered | Need to be considered | Need to be considered |
| Applicability to different scenarios/conditions/ configurations | Depends on the granularity of test decoder and data set(s) | Depends on the granularity of test decoder and data set(s) | Depends on the granularity of test decoder and data set(s) | Depends on the granularity of test decoder and data set(s) |
| Complexity of actual testing procedure for the ecosystem | High | High | Low | Low |
| Friendly to STOA(state of the art) model test | Yes | Yes | No | [No] |
| Whether model transfer/delivery is needed during the test procedure | Yes | Yes | No | No |

Observation 1: Pros and cons for different options on test decoder are shown in table 1.

In addition to the test model, the dataset that used for test model training/inference need to be considered as well. Options are listed below:

Option1: Dataset based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”.

Option2: Field dataset

In RAN1, option1(data set based on TR 38.901) is utilized by companies to propose their evaluation results. For example, in CSI cases, the UMa channel model based on TR 38.901 is the data source for training and testing. As for the field data, companies can optionally provide their results based on it.

In RAN4, especially for the construction of test models and tests, evaluation assumptions in RAN1 can be considered. Data sets based on TR 38.901 can be utilized as the starting point.

Proposal 3: Dataset based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, should be considered in RAN4.

Regarding the testability of AI/ML features, different AI/ML capabilities should be considered, e.g. capabilities that support different use cases, different scenarios and different models. In RAN4 AI/ML tests, following aspects need to be considered:

- Definition of basic AI/ML capability and corresponding testing metrics

- Definition of different AI/ML capability levels and different testing metrics for different levels

- Dynamic AI/ML capabilities (e.g. AI capability being influenced by computing resources, transmission resources, and storage resources)

Proposal 4: Regarding the AI/ML capabilities, following aspects should be considered

- Definition of basic AI/ML capability and corresponding testing metrics

- Definition of different AI/ML capability levels and different testing metrics for different levels

- Dynamic AI/ML capabilities

- Conclusions

In this contribution, we discussed the Rel-18 AI/ML impacts to RAN4 and got following proposals

Proposal 1: Regarding the testability of two-sided model, should introduce test encoder(s) to collaborate with the  NW decoder under test.

Proposal 2: Regarding the testability of two-sided model, should introduce test decoder(s) to collaborate with the UE encoder under test.

Observation 1: Pros and cons for different options on test decoder are shown in table 1.

Proposal 3: Dataset based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, should be considered in RAN4.

Proposal 4: Regarding the AI/ML capabilities, following aspects should be considered

- Definition of basic AI/ML capability and corresponding testing metrics

- Definition of different AI/ML capability levels and different testing metrics for different levels

- Dynamic AI/ML capabilities

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94-e, Dec. 2021