3GPP TSG-RAN WG4 Meeting #109		R4-2319940

Chicago, USA, 13th – 17th November, 2023

Agenda item:	8.9.1

Source:	Intel Corporation

Title:	Feature list proposals for measurement gap enhancements

Document for:	Discussion

# Introduction

Discussions on RAN4 Rel-18 UE feature list are scheduled to start from RAN4 #109 (November 2023) and this paper summarizes our views on the proposals for measurements gap enhancements Rel-18 work item. To facilitate efficient discussion in RAN4 and to streamline UE features implementation in RAN2 specifications we recommend a set of principles while preparing the Rel-18 RAN4 UE feature list to be followed and the principles are shown in [1].

# Guidelines

## General guidelines

In accordance with the RAN4 #109 meeting arrangements the initial discussion on RAN4 Rel-18 UE feature list is scheduled to be kicked-off based on the following guidelines:

- The dedicated agenda for Rel-18 feature list/UE capability will be set, and the topic thread & moderator will be assigned. - Moderator will provide the template for Rel-18 feature list/UE capabilities before the meeting. - For the newly proposed UE capabilities - The proponents need submit the contribution with details to the agenda of the corresponding WI, and the decision should be made under the agenda of the individual WI. - The proponents also need submit the contribution to the dedicated agenda for Rel-18 feature list/UE capability. After the decision is made under the agenda of the individual WI, the corresponding UE capability will be captured in the Rel-18 feature list. - If there is no corresponding agenda, the proponents can directly submit the contributions with technique details in the dedicated agenda for Rel-18 feature list/UE capability, and the decision will be made under the dedicated agenda. - For the previous agreed UE capabilities, - The rapporteur or other interested companies can submit the contributions with all the details following the template under the dedicated agenda for Rel-18 feature list, and the review and necessary discussion will be held under the dedicated agenda. - During the meeting, the feature list will be reviewed and will be discussed in the joint sessions.

## General principles and feature list contents

To facilitate efficient discussion in RAN4 and to streamline UE features implementation in RAN2 specifications we recommend a set of principles while preparing the Rel-18 RAN4 UE feature list to be followed and the principles are shown in [1].

# Rel-18 UE feature list

## NR_MG_enh2

Our views on RAN4-centric features for the NR_MG_enh2 WI [WID TBA] are provided below and in Table 5 for NR and Table 6 for LTE. (Table numbering follows our general feature list discussion paper in agenda item 10.)

Regarding the proposed 32-1 below, RAN4 had agreement to reuse NeedForNCSG-EUTRA signaling name and structure implemented in TS 38.331. Having this said this item still needs to be captured in the feature list table by the guidance from RAN2. Further, proposed 32-4 and the LTE feature in table 6 have the similar story to proposed 32-1 feature.

Table 5. Rel-18 NR UE features for NR_MG_enh2 WI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Inter-RAT EUTRAN measurements without measurement gap using vacant RF chain (case b-1) | 1. Support of inter-RAT EUTRAN measurements without gap with or without interruption | No | Yes | NA | The UE does not support inter-RAT EUTRAN measurements without gap for case b-1 | Per UE | No | No | NA | # Component 1 candidate value: true/false # The signalling name and structure are expected to be the same as the implementation of Rel-17 feature NeedForNCSG-EUTRA-r17 in TS38.331 | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-2 | Inter-RAT EUTRAN measurements without measurement gap when target CRS is within UE active bandwidth part (case b-2) | 1. Support of inter-RAT EUTRAN measurements without gap when CRS is fully contained within UE active BWP | No | Yes | NA | The UE does not support inter-RAT EUTRAN measurements without gap for case b-2 | Per UE | No | No | NA | # Component 1 candidate value: true/false | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-3 | Support of effective measurement window (EMW) for Inter-RAT EUTRAN measurements without measurement gap | 1. Supported EMW patterns for Inter-RAT EUTRAN measurements without measurement gap | 32-1 or 32-2 | Yes | NA | The UE does not support the EMW configurations in addition to the mandatory ones specified in TS 38.133 | Per UE | No | No | NA | # 1. UE indicates the bitmap of supported EMW patterns (6 patterns in total) # 2. EMW pattern #0, #1 specified in TS 38.133 are conditionally mandatory if the UE supports feature group 32-1 or 32-2 | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-4 | Intra and Inter-frequency measurement without gap using vacant RF chain | 1. Support of intra- and inter- frequency measurements without gap with or without interruption | No | Yes | NA | The UE does not support intra- and/or inter-frequency measurements without gap with or without interruption | Per UE | No | No | NA | # Component 1 candidate value: true/false # The signalling name and structure are expected to be the same as the implementation of Rel-17 feature NeedForNCSG-NR-r17 in TS38.331 | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-5 | Concurrent measurement gap with Pre-MG | 1. Support of RRM requirements in TS 38.133 for multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. | 1) 19-3-1 or 19-3-2 2) 19-2-1 | Yes | NA | The UE does not support multiple per-UE (or per-FR) measurement gap patterns with at least one Pre-MG and related RRM requirements | Per UE | No | No | NA | # Component 1 candidate value: true/false | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-6 | Concurrent measurement gap with NCSG | 1. Support of RRM requirements in TS 38.133 for multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. | 19-1a and 19-2-1 | Yes | NA | The UE does not support multiple per-UE (or per-FR) measurement gap patterns with at least one NCSG and related RRM requirements | Per UE | No | No | NA | # Component 1 candidate value: true/false | Optional with capability signalling |

Table 6. Rel-18 LTE UE features for NR_MG_enh2 WI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | x-x | Inter-RAT NR measurement without gap using vacant RF chain | 1. Support of inter-RAT NR measurements without gap with or without interruption | No | Yes | NA | The UE does not support inter-RAT NR measurements without gap with or without interruption | Per UE | No | No | NA | # Component 1 candidate value: true/false # The signalling name and structure are expected to be the same as the implementation of Rel-17 feature NeedForNCSG-NR-r17 in TS38.331 | Optional with capability signalling |

# Conclusion

This paper summarizes our proposals for general principles and the specific feature groups in Rel-18 measurement gap enhancement work item.

# References

[1] R4-2319811	Views on RAN4 Rel-18 UE feature list, Intel Corporation