3GPP TSG-RAN WG4 Meeting #109		R4-2319941

Chicago, USA, 13th – 17th November, 2023

Agenda item:	8.12

Source:	Intel Corporation

Title:	Feature list proposals for HST FR2 enhancements

Document for:	Discussion

# Introduction

Discussions on RAN4 Rel-18 UE feature list are scheduled to start from RAN4 #109 (November 2023) and this paper summarizes our views on the proposals for HST enhancements FR2 Rel-18 work item. To facilitate efficient discussion in RAN4 and to streamline UE features implementation in RAN2 specifications we recommend a set of principles while preparing the Rel-18 RAN4 UE feature list to be followed and the principles are shown in [1].

# Guidelines

## General guidelines

In accordance with the RAN4 #109 meeting arrangements the initial discussion on RAN4 Rel-18 UE feature list is scheduled to be kicked-off based on the following guidelines:

- The dedicated agenda for Rel-18 feature list/UE capability will be set, and the topic thread & moderator will be assigned. - Moderator will provide the template for Rel-18 feature list/UE capabilities before the meeting. - For the newly proposed UE capabilities - The proponents need submit the contribution with details to the agenda of the corresponding WI, and the decision should be made under the agenda of the individual WI. - The proponents also need submit the contribution to the dedicated agenda for Rel-18 feature list/UE capability. After the decision is made under the agenda of the individual WI, the corresponding UE capability will be captured in the Rel-18 feature list. - If there is no corresponding agenda, the proponents can directly submit the contributions with technique details in the dedicated agenda for Rel-18 feature list/UE capability, and the decision will be made under the dedicated agenda. - For the previous agreed UE capabilities, - The rapporteur or other interested companies can submit the contributions with all the details following the template under the dedicated agenda for Rel-18 feature list, and the review and necessary discussion will be held under the dedicated agenda. - During the meeting, the feature list will be reviewed and will be discussed in the joint sessions.

## General principles and feature list contents

To facilitate efficient discussion in RAN4 and to streamline UE features implementation in RAN2 specifications we recommend a set of principles while preparing the Rel-18 RAN4 UE feature list to be followed and the principles are shown in [1].

# Rel-18 UE feature list

## NR_HST_FR2_enh

Our views on RAN4-centric features for the NR_HST_FR2_enh WI [WID TBA] are provided below and in Table 8.

To note that for the proposed feature groups on multi-panel simultaneous reception, a single indication element is used to indicate the UE capability to support all three components together.

Table 8. Rel-18 NR UE features for NR_HST_FR2_enh WI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_HST_FR2_enh | 34-1 | Enhanced RRM requirements for CA HST FR2 | 1. Support of the enhanced RRM requirements specified for CA and inter-frequency measurements for HST FR2 UE | 22-1 | Yes | NA | UE does not support enhanced RRM requirements for CA and/or inter-frequency measurements | Per Band | No | FR2 only | N/A | # Component 1 candidate value: true/false | Optional with capability signalling |
| 34. NR_HST_FR2_enh | 34-2 | Enhanced demodulation requirements for CA HST FR2 | 1. Support of the enhanced demodulation requirements specified for CA for HST FR2 UE | 22-1 | Yes | NA | UE does not support enhanced demodulation requirements for CA | Per Band | No | FR2 only | N/A | # Component 1 candidate value: true/false | Optional with capability signalling |
| 34. NR_HST_FR2_enh | 34-3 | Support of enhanced UL timing adjustment | 1. Support of enhanced UL timing adjustment | 22-2 | Yes | NA | UE does not support enhanced UL timing adjustment | Per Band | No | FR2 only | N/A | # Component 1 candidate value: true/false | Optional with capability signalling |
| 34. NR_HST_FR2_enh | 34-4 | Support of FR2 HST with multi-panel simultaneous reception operation | 1. Support of RF requirements for FR2 PC6 UE with two panel simultaneous reception 2. Support of enhanced RRM requirements for FR2 PC6 UE with two panel simultaneous reception 3. Support of enhanced demodulation requirements for HST FR2 UE multi-panel receptions | 22-1 | Yes | NA | UE does not meet FR2 high speed train scenario with two panel simultaneous reception | Per Band | No | FR2 only | N/A | # A single indication element is used to indicate for all three components. # candidate value: true/false | Optional with capability signalling |
| 34. NR_HST_FR2_enh | 34-5 | Enhanced IDLE mode FR2 HST UE measurement requirements | 1. Support of enhanced IDLE mode measurement requirements for FR2 HST UE | No | No | NA | UE does not support enhanced IDLE mode FR2 HST UE mobility | NA | No | FR2 only | NA |  | Optional without capability signalling |

# Conclusion

This paper summarizes our proposals for general principles and the specific feature groups in Rel-18 HST FR2 enhancement work item.

# References

[1] R4-2319811	Views on RAN4 Rel-18 UE feature list, Intel Corporation