3GPP TSG-RAN WG4 Meeting #109	R4-2319956

Chicago, US, November 13 – 17, 2023

Title: 	Discussion on L1-RSRP measurements for R18 FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	8.7.2.2

Document for:	Discussion

Introduction

In this contribution, we will provide the discussion on L1-RSRP measurements for FR2 multi-Rx DL reception.

Discussion

For FR2 multi-Rx, RAN4 discussed the requirements for GBBR L1-RSRP measurements. For CSI-RS based GBBR L1-RSRP measurements in FR2, it was agreed to reuse the existing L1-RSRP measurement period. For SSB based GBBR L1-RSRP measurements in FR2, the following agreements on measurement period were achieved.

- Agreements - For SSB based L1-RSRP measurements in FR2 the measurement period for GBBR defined as: Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*N)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(1.5*M*P*N)*TDRX. Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: N is FFS. - N is FFS - Option 1: N = [reduceNumberRxBeam] for UE supporting faster beam sweeping under multi-Rx operations; otherwise N=8. - Option 2: N = 8 + K, where K is the number of SSBs in each CMR set

The value of beam sweeping factor N used for GBBR L1-RSRP measurement requirements in FR2 needs to be further studied, and the following options were captured in [1] in last RAN4 meeting.

Issue 1-2-2 a: measurement period for SSB based L1-RSRP - Way forward: Companies are requested to bring further analysis. - option 1: N = [reduceNumberRxBeam] for UE supporting faster beam sweeping under multi-Rx operations; otherwise, N=8 - Option 2: N = 8 + K, where K is the number of SSBs in each CMR set (Apple) Issue 1-2-2 b: measurement period for CSI-RS based L1-RSRP - Way forward: Companies are requested to bring further analysis. - Option 1: For CSI-RS based L1-RSRP measurements in FR2, the existing L1-RSRP measurement period is reused when configured for GBBR. - Option 2: N = ceil(maxNumberRxBeam / K) + 1, where K is the number of CSI-RSs in each CMR set

For SSB based GBBR L1-RSRP measurements in FR2, option 1 suggests that whether to use the legacy beam sweeping factor or the reduced beam sweeping factor is up to UE capability, while option 2 suggests to extend the beam sweeping factor for considering the interference measurements for beam management based on the discussion in [2]. However, according to the CSI resource setting configuration specified in TS38.214, CMR resource set is configured for channel measurement for L1-RSRP computation or for channel and interference measurement for L1-SINR computation. For L1-RSRP based beam management, there is no need to consider interference measurement on CMR resource set. Besides, even when L1-SINR based beam management is introduced in R16 eMIMO, SSB resource will not be configured for interference measurement for L1-SINR computation. Hence, we suggest option 1 for SSB based GBBR L1-RSRP measurements in FR2.

Proposal 1: For SSB based GBBR L1-RSRP measurements in FR2, the beam sweeping factor N can be defined as:

- Option 1:  N = [reduceNumberRxBeam] for UE supporting faster beam sweeping under multi-Rx operations; otherwise, N=8

For CSI-RS based GBBR L1-RSRP measurements in FR2, option 1 suggests to reuse the legacy beam sweeping factor, while option 2 suggests to extend the beam sweeping factor for considering the interference measurements for beam management based on the discussion in [2]. As we mentioned above, for L1-RSRP computation, CMR resource set is for channel measurement rather than interference measurement. So, we suggest to reuse the legacy beam sweeping factor for CSI-RS based GBBR L1-RSRP measurements in FR2.

Proposal 2: For CSI-RS based GBBR L1-RSRP measurements in FR2, the legacy beam sweeping factor N can be reused.

Conclusions

In this paper we provided our discussion on open issue on L1-RSRP measurements for FR2 multi-Rx DL reception. The followings are provided.

Proposal 1: For SSB based GBBR L1-RSRP measurements in FR2, the beam sweeping factor N can be defined as:

- Option 1:  N = [reduceNumberRxBeam] for UE supporting faster beam sweeping under multi-Rx operations; otherwise, N=8

Proposal 2: For CSI-RS based GBBR L1-RSRP measurements in FR2, the legacy beam sweeping factor N can be reused.

Reference

- R4-2317427, “WF on FR2_multiRx_part2”, Moderator (Ericsson)

- R4-2315505, “On L1 measurements for NR FR2 multi-Rx chain DL reception”, Apple