3GPP TSG-RAN4 Meeting #109	R4-2319976

# Chicago, US, 13 – 17 November, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3800 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_redcap-Core] CR on INACTIVE requirements R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon,MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2023-10-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In R4-2310122, eDRX requirements for non-RedCap UE are introduced, by reusing the same requirements design from RedCap. However, the CR only added eDRX requirements for IDLE mode. For INACTIVE mode, the current requirements for non-RedCap UE are reusing IDLE mode requirements, and this creates the following issues: - non-RedCap requirements are defined for CN eDRX but not RAN eDRX - non-RedCap requirements are different from RedCap The incompleteness and inconsistency also create issues for CR drafting in Rel-18, e.g. for positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce eDRX requriements for non-Redcap UE in INACTIVE mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | non-RedCap requirements are defined for CN eDRX but not RAN eDRX non-RedCap requirements are different from RedCap |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

5.1.2	Requirements

5.1.2.1	UE measurement capability

The requirements in clause 4.2.2.1 shall apply.

5.1.2.2	Measurement and evaluation of serving cell

The requirements in this clause apply when UE is configured with eDRX_IDLE, otherwise the requirements in clause 4.2.2.2 shall apply.

When UE is configured with eDRX_IDLE, the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1* T for FR1 and N1*T for FR2; where:

-	T is determined according to clause 7.1 in [1],

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and T ≤ 0.64 second, otherwise M1=1.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least T/2.

If the UE has evaluated according to Table 5.1.2.2-1 or and Table 5.1.2.2-2 in Nserv consecutive T cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

Table 5.1.2.2-1: Nserv for UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length[s] | T [s] | Nserv [number of T ] |
| --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 ≤DRX_Inactive cycle length ≤2.56; or 2.56 ≤eDRX_Inactive cycle length ≤10.24 if inactive eDRX is configured | 0.32 | 4*M1 |
|  |  | 0.64 | 4*M1 |
|  |  | 1.28 | 2 |
|  |  | 2.56 | 2 |
|  |  | 5.12 | 2 |
|  |  | 10.24 | 2 |
| Note1: T is dertermined according to clause 7.1 in [1]. |  |  |  |

Table 5.1.2.2-2: Nserv for UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length[s] | T [s] | Scaling Factor (N1) | Nserv [number of T] |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 ≤DRX_Inactive cycle length ≤2.56; or 2.56 ≤eDRX_Inactive cycle length ≤10.24 if inactive eDRX is configured | 0.32 | 8 | 4* N1 |
|  |  | 0.64 | 5 | 4* N1 |
|  |  | 1.28 | 4 | 2* N1 |
|  |  | 2.56 | 3 | 2* N1 |
|  |  | 5.12 | 3 | 2* N1 |
|  |  | 10.24 | 3 | 2* N1 |
| Note1: T is dertermined according to clause 7.1 in [1]. |  |  |  |  |

When UE transitions from measurements within PTW to measurements outside PTW or vice versa during one measurement period, the UE measurement requirements apply based on the longer measurement period requirements before or after the transition.

If the UE in RRC_INACTIVE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information during the time T’, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where:

-	T’= 10 s, if the UE is not configured with eDRX_inactive cycle, or

-	T’= MAX (10 s, one eDRX_INACTIVE cycle) if the UE is configured with eDRX_INACTIVE cycle for FR1, or

-	T’= MAX (10 s, N1* eDRX_INACTIVE cycle) if the UE is configured with eDRX_INACTIVE cycle for FR2.

5.1.2.3	Measurements of intra-frequency NR cells

The requirements in this clause apply when UE is configured with eDRX_IDLE, otherwise the requirements in clause 4.2.2.3 shall apply. The requirements in clause 4.2.2.9 apply for UE configured with relaxed measurement criterion when UE is not configured with eDRX_IDLE.

When UE is configured with eDRX_IDLE, the requirements defined in section 4.2.2.3 shall apply with Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra defined in Table 5.1.2.3-1 and Table 5.1.2.3-2.

Table 5.1.2.3-1: Tdetect, Tmeas and Tevaluate for UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra [s] (number of DRX or INACTIVE eDRX cycles) |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x M2 (36 x M2) | 1.28 x M2 (4 x M2) | 5.12 x M2 (16 x M2) |
|  | 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
|  | 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
|  | 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
|  | 5.12 | 117.76 (23) | 5.12 (1) | 15.36 (3) |
|  | 10.24 | 235.52 (23) | 10.24 (1) | 30.72 (3) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |

Table 5.1.2.3-2: Tdetect, Tmeas and Tevaluate for UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 8 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
|  | 0.64 | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
|  | 1.28 | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
|  | 2.56 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
|  | 5.12 | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 15.36 x N1 (3 x N1) |
|  | 10.24 | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 30.72 x N1 (3 x N1) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

Editor’s Note: requirements with relaxed measurement criterion can be further revisited.

5.1.2.4	Measurements of inter-frequency NR cells

The requirements in this clause apply when UE is configured with eDRX_IDLE, otherwise the requirements in clause 4.2.2.4 shall apply.

When UE is configured with eDRX_IDLE, the requirements defined in section 4.2.2.4 shall apply with Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter defined in Table 5.1.2.4-1 and Table 5.1.2.4-2, and with Kcarrier defined as follows.

Table 5.1.2.4-1: Tdetect, Tmeas and Tevaluate for UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x 1.5 (36 x 1.5) | 1.28 x 1.5 (4 x 1.5) | 5.12 x 1.5 (16 x 1.5) |
|  | 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
|  | 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
|  | 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
|  | 5.12 | 117.76 (23) | 5.12 (1) | 15.36 (3) |
|  | 10.24 | 235.52(23) | 10.24 (1) | 30.72 (3) |

Table 5.1.2.4-2: Tdetect, Tmeas and Tevaluate for UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 8 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
|  | 0.64 | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
|  | 1.28 | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
|  | 2.56 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
|  | 5.12 | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 15.36 x N1 (3 x N1) |
|  | 10.24 | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 30.72 x N1 (3 x N1) |

If UE is not configured to perform PRS measurement, or if UE is configured to perform PRS measurement and supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier in clause 4.2.2.4 shall apply regardless of whether the serving cell is subject to CCA or not.

If UE is configured to perform PRS measurement but does not support parallelPRS-MeasRRC-Inactive-r17, Kcarrier is replaced with Kcarrier + 1, regardless of whether the serving cell is subject to CCA or not.

If UE is not configured to perform PRS measurement, or if UE is configured to perform PRS measurement and supports parallelPRS-MeasRRC-Inactive-r17, the requirements in clause 4.2.2.10 shall apply regardless of whether the serving cell is subject to CCA or not for UE configured with relaxed measurement criterion.

If UE is configured to perform PRS measurement but does not support parallelPRS-MeasRRC-Inactive-r17, the requirements in clause 4.2.2.10 shall apply with Kcarrier being replaced with Kcarrier + 1, regardless of whether the serving cell is subject to CCA or not for UE configured with relaxed measurement criterion.

Editor’s Note: requirements with relaxed measurement criterion can be further revisited.

5.1.2.5	Measurements of inter-RAT E-UTRAN cells

The requirements in this clause apply when UE is configured with eDRX_IDLE, otherwise the requirements in clause 4.2.2.5 shall apply. The requirements in clause 4.2.2.11 shall apply for UE configured with relaxed measurement criterion when UE is not configured with eDRX_IDLE.

When UE is configured with eDRX_IDLE, the requirements defined in section 4.2.2.5 shall apply with Tdetect, EUTRAN, Tmeasure, EUTRAN and Tevaluate, EUTRAN defined in Table 5.1.2.5-1.

Table 5.1.2.5-1: Tdetect, EUTRAN, Tmeasure, EUTRAN and Tevaluate, EUTRAN for inactive UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect, EUTRAN [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure, EUTRAN [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate, EUTRAN [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x 1.5 (36 x 1.5) | 1.28 x 1.5 (4 x 1.5) | 5.12 x 1.5 (16 x 1.5) |
|  | 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
|  | 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
|  | 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
|  | 5.12 | 117.76 (23) | 5.12 (1) | 15.36 (3) |
|  | 10.24 | 235.52(23) | 10.24 (1) | 30.72 (3) |

Editor’s Note: requirements with relaxed measurement criterion can be further revisited.

5.1.2.6	Maximum interruption in paging reception

The requirements in clause 4.2.2.6 shall apply.

5.1.2.7	General requirements

If UE is not configured to perform PRS measurement, or if UE is configured to perform PRS measurement and supports parallelPRS-MeasRRC-Inactive-r17, the requirements in clause 4.2.2.7 shall apply.

If UE is configured to perform PRS measurement but does not support parallelPRS-MeasRRC-Inactive-r17, the requirements in clause 4.2.2.7 shall apply with Nlayers being replaced with Nlayers + 1.

<End of Change 1>