3GPP TSG-RAN4 Meeting #109	R4-2319995

# Chicago, US, 13 – 17 November, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR #14: Cell reselection measurement for positioning for RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on endorsed work split R4-2317388 for Rel-18 positioning, requirements for Cell reselection measurement for positioning for RedCap UE need to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce requirements for Cell reselection measurement for positioning for RedCap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirements for Rel-18 positioning are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6A.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 5.6A.2	Cell re-selection for positioning

The requirements in this clause apply for RedCap UE, when the UE is configured to perform PRS measurements and/or to perform SRS transmission for positioning.

The requirements in clause 5.1B.2 shall apply in the following conditions.

-	UE is not configured with eDRX_IDLE, or

-	UE is configured with eDRX_IDLE but without eDRX_INACTIVE, or

-	UE is configured with both eDRX_IDLE and eDRX_INACTIVE, and eDRX_INACTIVE cycle is smaller or equal to TPOS,

where TPOS is

-	PRS measurement reporting periodicity, if UE is configured to only perform PRS measurements for positioning,

-	SRS transmission periodicity, if UE is configured to only perform SRS transmission for positioning,

-	the minimum of PRS measurement reporting periodicity and SRS transmission periodicity, if UE is configured to both perform PRS measurements and to perform SRS transmission for positioning.

When UE is configured with both eDRX_IDLE and eDRX_INACTIVE, and eDRX_INACTIVE cycle is larger than TPOS, the requirements in clause 5.1B.2 except clause 5.1B.2.2 and 5.1.2B.3 shall apply, and the requirements in clause 5.6A.2.1 apply for measurement and evaluation of serving cell, and the requirements in clause 5.6A.2.2 apply for measurements of intra-frequency NR cells.

5.6A.2.1	Measurement and evaluation of serving cell

When a RedCap UE is configured with both eDRX_IDLE and eDRX_INACTIVE, and eDRX_INACTIVE cycle is larger than TPOS, the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*Tserv for FR1 and N1* Tserv for FR2; where:

-	Tserv is defined as max(T, TPOS), where T is dertermined according to clause 7.1 in [1],

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and Tserv ≤ 0.64 second, otherwise M1=1.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least Tserv/2.

If the UE has evaluated according to Table 5.6A.2.1-1 or and Table 5.6A.2.1-2 in Nserv consecutive Tserv that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

Table 5.6.1A.1-1: Nserv for UE configured with eDRX INACTIVE cycle ≥ TPOS (FR1)

| eDRX_IDLE cycle length [s] | eDRX INACTIVE cycle length[s] | Tserv [s] | Nserv [number of Tserv] |
| --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤10485.76 | eDRX INACTIVE cycle length ≥ TPOS | 0.32 ≤ Tserv < 1.28 | 4*M1 |
|  |  | 1.28 ≤ Tserv | 2 |

Table 5.6.1A.1-2: Nserv for UE configured with eDRX INACTIVE cycle ≥ TPOS (FR2)

| eDRX_IDLE cycle length [s] | eDRX INACTIVE cycle length[s] | Tserv [s] | Scaling Factor (N1) | Nserv [number of Tserv] |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤10485.76 | eDRX INACTIVE cycle length ≥ TPOS | 0.32 ≤ Tserv < 0.64 | 8 | 4*N1 |
|  |  | 0.64 ≤ Tserv < 1.28 | 5 | 4*N1 |
|  |  | 1.28 ≤ Tserv < 2.56 | 4 | 2*N1 |
|  |  | 2.56 ≤ Tserv | 3 | 2*N1 |

If the UE in RRC_INACTIVE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information during the time T’, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where

-	T’ = MAX (10 s, one eDRX_INACTIVE cycle) for FR1, or

-	T’= MAX (10 s, N1* eDRX_INACTIVE cycle) for FR2.

5.6A.2.2	Measurements of intra-frequency NR cells

When a RedCap UE is configured with both eDRX_IDLE and eDRX_INACTIVE, and eDRX_INACTIVE cycle is larger than TPOS, the requirements defined in section 5.1B.2.3 shall apply with Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra defined in Table 5.6A.2.2-1 and Table 5.6A.2.2-2, where

-	Tserv is defined as max(T, TPOS), where T is dertermined according to clause 7.1 in [1],

-	M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1.

Table 5.6A.2.2-1: Tdetect, Tmeasure and Tevaluate for UE configured with eDRX INACTIVE cycle ≥ TPOS (FR1)

| eDRX_IDLE cycle length [s] | Tserv [s] | Tdetect,NR_Intra (number of Tserv) | Tmeasure,NR_Intra (number of Tserv) | Tevaluate,NR_Intra (number of Tserv) |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 ≤ Tserv < 0.64 | 36 x M2 | 4 x M2 | 16 x M2 |
|  | 0.64 ≤ Tserv < 1.28 | 28 | 2 | 8 |
|  | 1.28 ≤ Tserv < 2.56 | 25 | 1 | 5 |
|  | 2.56 ≤ Tserv | 23 | 1 | 3 |
| NOTE 1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra in seconds depend on the number N of Tserv and are calculated as N * Tserv. |  |  |  |  |

Table 5.6A.2.2-2: Tdetect, Tmeasure and Tevaluate for UE configured with eDRX INACTIVE cycle ≥ TPOS (FR2)

| eDRX_IDLE cycle length [s] | Tserv [s] | Scaling Factor (N1) | Tdetect,NR_Intra (number of Tserv) | Tmeasure,NR_Intra (number of Tserv) | Tevaluate,NR_Intra (number of Tserv) |
| --- | --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 ≤ Tserv < 0.64 | 8 | 36 x N1 x M2 | 4 x N1 x M2 | 16 x N1 x M2 |
|  | 0.64 ≤ Tserv < 1.28 | 5 | 28 x N1 | 2 x N1 | 8 x N1 |
|  | 1.28 ≤ Tserv < 2.56 | 4 | 25 x N1 | 1 x N1 | 5 x N1 |
|  | 2.56 ≤ Tserv | 3 | 23 x N1 | 1 x N1 | 3 x N1 |
| NOTE 1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra in seconds depend on the number N of Tserv and are calculated as N * Tserv. |  |  |  |  |  |

<End of Change 1>