3GPP TSG-RAN WG4 Meeting #109	R4-2319997

Chicago, US, 13 – 17 November, 2023

Title: 	Simulation results for PRS measurement with FH

Source: 	Huawei, HiSilicon

Agenda item:	8.22.2.4

Document for:	Discussion

Introduction

Simulation assumption for evaluating the link level performance of PRS measurement with FH by RedCap UE is approved in RAN4#108 [1].

In this paper we will provide our initial simulation results for PRS measurement with FH.

Discussion

In this paper, we present our initial simulation results for

- RSTD and UE Rx-Tx

- in both FR1 and FR2

- under AWGN and fading channels

- with 1 and 4 samples

All the results are for 2RX. The results for 1RX will be provided in future meetings.

Simulation results for RSTD

FR1

Table 1: RSTD error for 4-sample, Es/Iot of (-6,-13)dB, AWGN, FR1

| Parameter | Value |  |  | error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | 15 | 52 | 272 | 43 |
|  | 30 | 52 | 272 | 21 |
|  | 60 | 24 | 136 | 25 |

Table 2: RSTD error for 4-sample, Es/Iot of (-6,-13)dB, TDL-A, FR1

| Parameter | Value |  |  | error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | 15 | 52 | 272 | 45 |
|  | 30 | 52 | 272 | 26 |
|  | 60 | 24 | 136 | 44 |

Table 3: RSTD error for 1-sample, Es/Iot of (-3,-6)dB, AWGN, FR1

| Parameter | Value |  |  | error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | 15 | 52 | 272 |  |
|  | 30 | 52 | 272 |  |
|  | 60 | 24 | 136 |  |

FR2

Table 4: RSTD error for 4-sample, Es/Iot of (-6,-13)dB, AWGN, FR2

| Parameter | Value |  |  | error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | 60 | 52 | 272 |  |
|  | 120 | 52 | 272 |  |

Table 5: RSTD error for 4-sample, Es/Iot of (-6,-13)dB, TDL-C, FR2

| Parameter | Value |  |  | error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | 60 | 52 | 272 |  |
|  | 120 | 52 | 272 |  |

Table 6: RSTD error for 1-sample, Es/Iot of (-3,-6)dB, AWGN, FR2

| Parameter | Value |  |  | error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | 60 | 52 | 272 |  |
|  | 120 | 52 | 272 |  |

Simulation results for UE Rx-Tx

FR1

Table 7: UE Rx-Tx error for 4-sample, Es/Iot of -13dB and -3dB, AWGN, FR1

| Parameter | Value |  |  |  | error |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -3 | 15 | 52 | 272 | 15 |
|  |  | 30 | 52 | 272 | 15 |
|  |  | 60 | 24 | 136 | 24 |
|  | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -13 | 15 | 52 | 272 | 54 |
|  |  | 30 | 52 | 272 | 29 |
|  |  | 60 | 24 | 136 | 33 |

Table 8: UE Rx-Tx error for 4-sample, Es/Iot of -13dB and -3dB, TDL-A, FR1

| Parameter | Value |  |  |  | error |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -3 | 15 | 52 | 272 | 42 |
|  |  | 30 | 52 | 272 | 28 |
|  |  | 60 | 24 | 136 | 31 |
|  | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -13 | 15 | 52 | 272 | 57 |
|  |  | 30 | 52 | 272 | 30 |
|  |  | 60 | 24 | 136 | 57 |

Table 9: UE Rx-Tx error for 1-sample, Es/Iot of -6dB and 0dB, AWGN, FR1

| Parameter | Value |  |  |  | error |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | 0 | 15 | 52 | 272 | 15 |
|  |  | 30 | 52 | 272 | 15 |
|  |  | 60 | 24 | 136 | 25 |
|  | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -6 | 15 | 52 | 272 | 45 |
|  |  | 30 | 52 | 272 | 29 |
|  |  | 60 | 24 | 136 | 37 |

FR2

Table 10: UE Rx-Tx error for 4-sample, Es/Iot of -13dB and -3dB, AWGN, FR2

| Parameter | Value |  |  |  | error |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -3 | 60 | 52 | 272 | 13 |
|  |  | 120 | 52 | 272 | 9 |
|  | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -13 | 60 | 52 | 272 | 42 |
|  |  | 120 | 52 | 272 | 42 |

Table 11: UE Rx-Tx error for 4-sample, Es/Iot of -13dB and -3dB, TDL-C, FR2

| Parameter | Value |  |  |  | error |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -3 | 60 | 52 | 272 | 28 |
|  |  | 120 | 52 | 272 | 28 |
|  | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -13 | 60 | 52 | 272 | 53 |
|  |  | 120 | 52 | 272 | 53 |

Table 12: UE Rx-Tx error for 1-sample, Es/Iot of -6dB and 0dB, AWGN, FR2

| Parameter | Value |  |  |  | error |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -3 | 60 | 52 | 272 | 25 |
|  |  | 120 | 52 | 272 | 10 |
|  | Es/Iot (dB) | SCS (kHz) | RB num per hop | RB num in total | 90% |
|  | -13 | 60 | 52 | 272 | 33 |
|  |  | 120 | 52 | 272 | 33 |

Conclusions

In this paper we provided our initial simulation results for PRS measurement with FH. We suggest RAN to take those results into account for defining the accuracy requirements.

One observation from the results is that there is clear performance degradation due to FH compared to single hop large BW, especially in AWGN and low SINR. For example, for Rx-Tx with 30kHz SCS, performance for 100MHz with FH is even worse than existing requirements for 50MHz. The reason is the phase error between hops which cannot be well compensated with 1 overlapping RB.

Reference

- R4-2314460, Simulation assumptions for RedCap positioning with FH, Ericsson