3GPP TSG-RAN WG4 Meeting #109	R4-2320009

Chicago, US, 13 – 17 November, 2023

Title: 	Discussion on requirements for UE supporting multiple options

Source: 	Huawei, HiSilicon

Agenda item:	8.10.1

Document for:	Discussion

Introduction

RRM requirements for BWP without restriction are discussed in RAN4#108-bis, and outcomes are captured in WF [1]. Based on [1] a generic issue on requirements for UE supporting multiple options need to be further discussed, including the following combinations.

- UE supporting both option B-1-1 and A

- UE supporting both option C and A

- UE supporting both option B-1-1 and B-1-2

In this paper we will provide our views on requirements for UE supporting multiple options for BWP without restriction.

Discussion

UE supporting both option B-1-1 and A

#### Issue 1-4: Requirements/UE behaviour for UE supporting both option B-1-1 and A - FFS - Option 1: - There is no obvious gain if option A and option B-1-1 are supported simultaneously. - For UE supporting both option B-1-1 and A, if CSI-RS is configured within the active BWP for L1 measurements, then UE is NOT expected to perform RLM/BFD/BM based on CD-SSB outside active BWP. - Option 2: - For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario. - Option 3: - For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define requirements/ UE behaviour. - Option 4: - RAN4 does not need to define additional requirements or clarify UE behaviour for the UE supporting options B-1-1 and A - Option 5: - RAN4 not to define additional requirements or clarify UE behaviour for UE supporting multiple options.

When UE supports both option B-1-1 and A, the relevant scenario is where UE is in an active BWP that does not contain CD- or NCD-SSB, but UE is configured to measure L1 both from SSB and CSI-RS. In this case, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP.

This is same to the R15 scenario where CD-SSB is within active BWP and UE is configured to measure L1 both from SSB and CSI-RS. We do not see additional UE behaviour or requirements are needed.

Proposal 1: For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

UE supporting both option C and A

#### Issue 1-5: Requirements/UE behaviour for UE supporting both option C and A - FFS - Option 1: - For UE supporting both option C and option A, UE shall perform L1 measurement according to network configuration, there is no need to define requirements/ UE behaviour. - Option 2: - For UE supporting both option C and A, both CSI-RS and NCD-SSB within the active BWP can be configured for L1 measurements. - Option 3: - RAN4 does not need to define additional requirements or clarify UE behaviour for the UE supporting options C and A. - Option 4: - For UE supporting both option B-1-1 and C, RAN4 shall discuss which SSB UE shall measure when active BWP does NOT contain neither CD-SSB nor NCD-SSB. Candidate solutions: - Alt 1: Leave it to network control. - Alt 2: UE always measures CD-SSB unless the active BWP contains NCD-SSB. - Option 5: - RAN4 not to define additional requirements for UE supporting multiple options.

When UE supports both option C and A, the relevant scenario is where UE is in an active BWP that contains NCD-SSB, and UE is configured to measure L1 both from SSB and CSI-RS. In this case, UE should perform L1 measurement on both NCD-SSB within active BWP and CSI-RS within active BWP.

This is same to the R15 scenario where CD-SSB is within active BWP and UE is configured to measure L1 both from SSB and CSI-RS. We do not see additional UE behaviour or requirements are needed.

Proposal 2: For UE supporting both option C and A, if configured by NW, UE should perform L1 measurement on both NCD-SSB within active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

UE supporting both option B-1-1 and B-1-2

#### Issue 1-6: Reporting behaviour for UE supporting both option B-1-1 and B-1-2 - FFS - Option 1: - A UE shall not indicate support of both option B-1-1 and option B-1-2 per BC.

In the latest RAN1 feature list, the following note is captured for FG 53-1 and 53-2.

Note: UE shall not indicate support of both B-1-1 and B-1-2 for the same band in the same reported band combination

The note is also aligned with RAN4 agreement. Given this note, we do not think RAN4 need to discuss the reporting behaviour for UE supporting both option B-1-1 and B-1-2 since it is not allowed.

Proposal 3: RAN4 not to discuss reporting behaviour for UE supporting both option B-1-1 and B-1-2.

Conclusions

In this paper we provided our views on requirements for UE supporting multiple options for BWP without restriction.

Proposal 1: For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

Proposal 2: For UE supporting both option C and A, if configured by NW, UE should perform L1 measurement on both NCD-SSB within active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

Proposal 3: RAN4 not to discuss reporting behaviour for UE supporting both option B-1-1 and B-1-2.

Reference

- R4-2317337, WF on support for bandwidth part operation without restriction, vivo