3GPP TSG-RAN WG4 Meeting # 109						R4-2320021

Chicago, USA, November 13 – November 17, 2023

Agenda item:			8.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Addition of tripe beat rules for MSD analysis

Document for:	Discussion and Approval

Introduction

In RAN4#106-bis-e Nokia presented the framework surrounding triple beat analysis in [1]. Since then, additions were added to the ULCA analysis in the respective TR’s, however triple beat was still not captured. In this TP we bring again the discussion of the triple beat analysis, and we bring additionally the proposals for the changes in TR38.718-02-01, TR 38.718-03-01 and TR 38.846 to capture triple beat rules.

Triple beat analysis

We present in Table 1 an overview of the different ULCA combinations of FDD and TDD along with special cases of simultaneous RX/TX or not. From the table we show the analysis variants of an inter-band ULCA with non-contiguous UL intra-band CA (treat contiguous UL intra-band CA with non-contiguous resource allocation similarly). Triple beat analysis are highlighted in bold.

Table 1: Overview of analysis for any FDD and TDD combination outlining triple beat.

|  | FDD-FDD | FDD-TDD NC ULCA in FDD | FDD-TDD NC ULCA in TDD | TDD-TDD |
| --- | --- | --- | --- | --- |
| Simultaneous RX/TX | MSD w. Triple beat analysis for both FDD RX bands | 1) MSD w. NC ULCA analysis into TDD band 2) MSD w. triple beat analysis only for FDD RX band | MSD w. triple beat analysis only for FDD RX band | MSD w. NC ULCA analysis into other TDD RX band |
| Non-simultaneous RX/TX |  | 1) MSD w. NC ULCA analysis into TDD band 2) MSD w. triple beat analysis only for FDD RX band | MSD w. triple beat analysis only for FDD RX band |  |

For the collision check we do not see the component carrier spacing in a non-contiguous UL intra-band CA considered so far in the frequency range check calculations. We encourage that both the minimum and the maximum component carrier separation is included in the frequency range check like the non-contiguous resource allocation of as we have shown to be important.

Table 1 captures only the 2 CA cases. When a third band is included there may be triple beat impact in this third band if it is an FDD band or if the TDD operation is non-simultaneous.

Triple beat with (non-) contiguous intra-band ULCA

Same as the approach for intra-band ULCA an analysis that includes the aspects of least and largest channel/PRB separation has been made to identify the products that define the ranges for which the IMD analysis must be based. Murata and Skyworks have looked into the rules for triple beat in [2] and made the observations to look only at FDD-FDD and FDD-TDD, since non-simultaneous RX/TX TDD-TDD has no RX during TX and simultaneous RX/TX TDD-TDD will become similar to the intra-band ULCA case, as captured in Table 1. Additionally, FDD-TDD is only relevant with the impact on the FDD RX, since the TDD RX case falls into regular NC ULCA analysis.

Only the 1st order triple beat is considered since this is already a 3rd order intermodulation product. It is not expected that 2nd order harmonics of the UL signals will cause any impact due to triple beat. This is because the maximum output power of the fundamentals in triple beat already have been backed off to keep compliancy with the power class, which in turn would reduce the harmonic response even further than the back-off of the fundamental TX.

Assuming that the maximum frequency separation may be contained inside the frequency band with additional margin it is necessary to consider both lowest and highest channel allocation including the allocation of the second component carrier.  This is shown in Figure 1 in the form of the use fUxL and fUxH.

Figure 1: Overview of carrier bandwidth for triple beat IMD analysis

a = channel bandwidth of 1st CC in aggressor TX band

b = channel bandwidth of 2nd CC in aggressor TX band

c = minimum CC separation + ULCC1 CBW

d = maximum CC separation + ULCC1 CBW

m = channel bandwidth of ULCC in higher band

n = channel bandwidth of ULCC in lower band

fU1L = minimum frequency in TX aggressor band of ULCC1

fU2L = minimum frequency separation in TX aggressor band of ULCC2 (fU2L= fU1L+c)

fU3L = maximum frequency separation in TX aggressor band of ULCC2(fU3L= fU1L+d)

fU1H = maximum frequency in TX aggressor band of ULCC1

fU2H = minimum frequency separation in TX aggressor band of ULCC2(fU2H= fU1H-c)

fU3H = maximum frequency separation in TX aggressor band of ULCC2(fU3H= fU1H-d)

fSCCL_L = minimum frequency in singleCC lower band

fSCCL_H = maximum frequency in singleCC lower band

fSCCH_L = minimum frequency in singleCC higher band

fSCCH_H = maximum frequency in singleCC higher band

Analyzing the first order triple beat products of FDD-TDD with FDD below TDD, seeking insight into the FDD RX victim band we see from the analysis below that the channel/PRB separation determines how the triple beat extends across the FDD RX band based on the separation between the CC’s/PRBs within the TDD band. An exemplary case of CA_n28A-n77(2A) is presented in Figure 2.

Depending on the FDD duplexing topology (UL-DL or DL-UL) it will be the highest (fUL3 or fUH3) separation within the TDD band that forces the triple-beat product into the FDD RX band (in yellow in Figure 2). Notice how low- separation (fU2L - fU1L - fSCCL_H) products remain inside the UL band until the separation increases to the level that the product starts to enter the FDD RX band from the duplex gap direction. In Figure 2 only some products are shown, this is because the differences fU3L - fU1L equals fU3H - fU1H and so on. Figure 2 also captures the high side difference product range (the two TDD UL components added) subtracted the FDD TX (in blue in Figure 2) while the summation product ranges using the purple and green colors is the range located at the highest frequency range.

Figure 2: Lower side FDD-TDD analysis CA_n28A-n77(2A)

Using the same intervals to determine the triple beat analysis outcome for higher side FDD-TDD we get the following outcome using the exemplary case CA_n7A-n40(2A):

Figure 3: Higher side FDD-TDD analysis CA_n7A-n40(2A)

The first range (lower side product) below the 2UL band (n40) adds the 2UL and subtracts the third UL (n7) (fSCCH). The next two ranges are produced by subtracting the 2UL band (n40) and respectively adding or subtracting the third UL (fSCCH). One range is determined by the lowest allocation in the single UL band, fSCCH_L (Upside 1), followed by a range determined by the highest allocation in the single UL band fSCCH_H (Upside 2). The upper range is again the sum products of all UL.

For the Upper side FDD-TDD it is the Upside 1 and Upside 2 ranges that cause the self-interference. Depending on the duplexing topology (UL-DL, DL-UL) it is either the lowest or highest frequency allocation of the single UL band (FDD in this case) that will cause the triple-beat product to fall inside the FDD RX band.

For the case of CA_n7A-n40(2A) similar to CA_n28A-n77(2A), both having (F(DL) > F(UL)) it is the highest FDD channel that will determine the triple beat range that hits the FDD RX.

The FDD-FDD case will combine the outcome of the analysis in Figure 2 and Figure 3, meaning that either of the FDD RX band may get hit.

The analysis of the ranges is shown in Table 2 for CA_n28A_n77(2A) with 10MHz channel BW,10MHz minimum separation and 600MHz maximum separation.

Table 2: Triple-beat products for the exemplary case CA_n28A-n77(2A)

| CC location | fSCCL | fSCCH | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | 703 | 748 | 3300 | 3320 | 3900 | 4200 | 4180 | 3600 |
| 1st order 3-beat | I fUL3 -fUL1 - fSCCL ¦ | I fUL3 -fUL1 + fSCCL ¦ | I fUL3 -fUL1 - fSCCH ¦ | I fUL3 -fUL1 + fSCCH ¦ | I fUH1 -fUH3 - fSCCL ¦ | I fUH1 -fUH3 + fSCCL ¦ | I fUH1 -fUH3 - fSCCH ¦ | I fUH1 -fUH3 + fSCCH ¦ |
|  | I fUL2 -fUL1 - fSCCL ¦ | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ | I fUL2 -fUL1 + fSCCH ¦ | I fUH1 -fUH2 - fSCCL ¦ | I fUH1 -fUH2 + fSCCL ¦ | I fUH1 -fUH2 - fSCCH ¦ | I fUH1 -fUH2 + fSCCH ¦ |
|  | I fUL3 +fUL1 - fSCCL ¦ | I fUL3 +fUL1 + fSCCL ¦ | I fUL3 +fUL1 - fSCCH ¦ | I fUL3 +fUL1 + fSCCH ¦ | I fUH1 +fUH3 - fSCCL ¦ | I fUH1 +fUH3 + fSCCL ¦ | I fUH1 +fUH3 - fSCCH ¦ | I fUH1 +fUH3 + fSCCH ¦ |
|  | I fUL2 +fUL1 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ | I fUL2 +fUL1 - fSCCH ¦ | I fUL2 +fUL1 + fSCCH ¦ | I fUH1 +fUH2 - fSCCL ¦ | I fUH1 +fUH2 + fSCCL ¦ | I fUH1 +fUH2 - fSCCH ¦ | I fUH1 +fUH2 + fSCCH ¦ |
| Ranges | 103 | 1303 | 148 | 1348 | 103 | 1303 | 148 | 1348 |
|  | 683 | 723 | 728 | 768 | 683 | 723 | 728 | 768 |
|  | 6497 | 7903 | 6452 | 7948 | 7097 | 8503 | 7052 | 8548 |
|  | 5917 | 7323 | 5872 | 7368 | 7677 | 9083 | 7632 | 9128 |

The grey cells are not relevant since these products match (the former mentioned “fUL3 - fUL1 equals fUH3 - fUH1” part). As for the rest of the products there are intervals as shown in Figure 2 also represented through colored cells. All white cells are not needed since the products they produce are inside the colored ranges. They can therefore be skipped.

Similar range analysis is shown in Table 3 for CA_n7A_n40(2A) with 10MHz channel BW, 10MHz minimum separation and 80MHz maximum separation.

Table 3: Triple-beat products for the exemplary case CA_n7A-n40(2A)

| CC location | fSCCL | fSCCH | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2500 | 2570 | 2300 | 2320 | 2380 | 2400 | 2380 | 2320 |
| 1st order 3-beat | I fUL3 -fUL1 - fSCCL ¦ | I fUL3 -fUL1 + fSCCL ¦ | I fUL3 -fUL1 - fSCCH ¦ | I fUL3 -fUL1 + fSCCH ¦ | I fUH1 -fUH3 - fSCCL ¦ | I fUH1 -fUH3 + fSCCL ¦ | I fUH1 -fUH3 - fSCCH ¦ | I fUH1 -fUH3 + fSCCH ¦ |
|  | I fUL2 -fUL1 - fSCCL ¦ | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ | I fUL2 -fUL1 + fSCCH ¦ | I fUH1 -fUH2 - fSCCL ¦ | I fUH1 -fUH2 + fSCCL ¦ | I fUH1 -fUH2 - fSCCH ¦ | I fUH1 -fUH2 + fSCCH ¦ |
|  | I fUL3 +fUL1 - fSCCL ¦ | I fUL3 +fUL1 + fSCCL ¦ | I fUL3 +fUL1 - fSCCH ¦ | I fUL3 +fUL1 + fSCCH ¦ | I fUH1 +fUH3 - fSCCL ¦ | I fUH1 +fUH3 + fSCCL ¦ | I fUH1 +fUH3 - fSCCH ¦ | I fUH1 +fUH3 + fSCCH ¦ |
|  | I fUL2 +fUL1 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ | I fUL2 +fUL1 - fSCCH ¦ | I fUL2 +fUL1 + fSCCH ¦ | I fUH1 +fUH2 - fSCCL ¦ | I fUH1 +fUH2 + fSCCL ¦ | I fUH1 +fUH2 - fSCCH ¦ | I fUH1 +fUH2 + fSCCH ¦ |
| Ranges | 2420 | 2580 | 2490 | 2650 | 2420 | 2580 | 2490 | 2650 |
|  | 2480 | 2520 | 2550 | 2590 | 2480 | 2520 | 2550 | 2590 |
|  | 2180 | 7180 | 2110 | 7250 | 2220 | 7220 | 2150 | 7290 |
|  | 2120 | 7120 | 2050 | 7190 | 2280 | 7280 | 2210 | 7350 |

The numbers in the colored cells of Table 2 and Table 3 match those shown in Figure 2 and Figure 3 respectively.

A simplified version focusing on the ranges that define the range limits (the colored cells only) A similar table as shown in Table 4 (for CA_n7A_n40(2A)) must be used for triple beat analysis.

Table 4: Triple beat analysis table for CA_n7A_n40(2A)

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2300 | 2320 | 2380 | 2400 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min channel separation |
| Frequency | 2500 | 2570 | 2380 | 2320 |  | 10 |
| 1st order triple beat | I fUL3 -fUL1 - fSCCL ¦ | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ | I fUL3 -fUL1 + fSCCH ¦ |  | Max channel separation |
| Ranges | 2420 | 2520 | 2550 | 2650 |  | 80 |
| 1st order triple beat | I fUL2 +fUL1 - fSCCH ¦ | I fUH1 +fUH2 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ | I fUH1 +fUH2 + fSCCH ¦ |  |  |
| Ranges | 2050 | 2280 | 7120 | 7350 |  |  |

Returning to the former two exemplary cases and finishing the frequency range checks we get the following outcome.

For CA_n28A-n77(2A) only the FDD RX is relevant and here we see an issue in the frequency range check.

| CC location | fU1L |  | fU2L | fU3L |  | fU1H |  |  | Channel BW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 |  | 3340 | 3880 |  | 4200 |  |  | 20 |  |
| CC location | fSCCL |  | fSCCH | fU2H |  | fU3H |  |  | Min channel separation |  |
| Frequency | 703 |  | 748 | 4160 |  | 3620 |  |  | 20 |  |
| 1st order triple beat | I fUL3 -fUL1 - fSCCL ¦ |  | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ |  | I fUL3 -fUL1 + fSCCH ¦ |  |  | Max channel separation |  |
| Ranges | 123 |  | 743 | 708 |  | 1328 |  |  | 580 |  |
| 1st order triple beat | I fUL2 +fUL1 - fSCCH ¦ |  | I fUH1 +fUH2 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ |  | I fUH1 +fUH2 + fSCCH ¦ |  |  |  |  |
| Ranges | 5892 |  | 7657 | 7343 |  | 9108 |  |  |  |  |
|  |  | RX n28 |  |  |  |  |  |  |  |
| fx_low |  |  |  |  | fx_high |  |  |  |  |
| 758 |  |  |  |  | 803 |  |  |  |  |
| I fUL3 -fUL1 - fSCCL ¦ |  | I fUL2 -fUL1 + fSCCL ¦ |  |  | I fUL2 -fUL1 - fSCCH ¦ |  | I fUL3 -fUL1 + fSCCH ¦ |  |  |
| Clear |  |  |  |  | Issue |  |  |  |  |
| I fUL2 +fUL1 - fSCCH ¦ |  | I fUH1 +fUH2 - fSCCL ¦ |  |  | I fUL2 +fUL1 + fSCCL ¦ |  | I fUH1 +fUH2 + fSCCH ¦ |  |  |
| Clear |  |  |  |  | Clear |  |  |  |  |

For CA_n7A-n40(2A) only the FDD RX is relevant and here we see an issue in the frequency range check.

| CC location | fU1L |  | fU2L | fU3L |  | fU1H |  |  | Channel BW |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2300 |  | 2340 | 2380 |  | 2400 |  |  | 20 |  |
| CC location | fSCCL |  | fSCCH | fU2H |  | fU3H |  |  | Min channel separation |  |
| Frequency | 2500 |  | 2570 | 2360 |  | 2320 |  |  | 20 |  |
| 1st order triple beat | I fUL3 -fUL1 - fSCCL ¦ |  | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ |  | I fUL3 -fUL1 + fSCCH ¦ |  |  | Max channel separation |  |
| Ranges | 2420 |  | 2540 | 2530 |  | 2650 |  |  | 80 |  |
| 1st order triple beat | I fUL2 +fUL1 - fSCCH ¦ |  | I fUH1 +fUH2 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ |  | I fUH1 +fUH2 + fSCCH ¦ |  |  |  |  |
| Ranges | 2070 |  | 2260 | 7140 |  | 7330 |  |  |  |  |
|  |  |  | RX n7 |  |  |  |  |  |  |  |
| fx_low |  |  |  |  |  | fx_high |  |  |  |  |
| 2620 |  |  |  |  |  | 2690 |  |  |  |  |
| I fUL3 -fUL1 - fSCCL ¦ |  |  | I fUL2 -fUL1 + fSCCL ¦ |  |  | I fUL2 -fUL1 - fSCCH ¦ |  | I fUL3 -fUL1 + fSCCH ¦ |  |  |
| Clear |  |  |  |  |  | Issue |  |  |  |  |
| I fUL2 +fUL1 - fSCCH ¦ |  |  | I fUH1 +fUH2 - fSCCL ¦ |  |  | I fUL2 +fUL1 + fSCCL ¦ |  | I fUH1 +fUH2 + fSCCH ¦ |  |  |
| Clear |  |  |  |  |  | Clear |  |  |  |  |

We would like to return to the rules captured in [1] about same or adjacent band group as at this points and opens up for further agreement. We capture the band group rules in the TR guidelines with the proposed frequency ranges of [1] so that triple beat analysis is not conducted when the two uplink bands of the analysis are not in same or adjacent band groups according to table 1.

Table 5 : Band group definition for adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-1 (LB) | FR1-2 (MB) | FR1-3 (HB) | FR1-4 (VHB) | FR1-5 (UHB) |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5250-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

The restrictions on when the triple beat analysis shall be done as summarized also in the beginning of this document in Table 1, may be unclear also when taking the above discussion frequency ranges into account. It is therefore proposed to capture guidance on this in the TR 38.846 as done in [1].

We capture the band group rules in the TR guidelines with the proposed frequency ranges of [1] so that triple beat analysis is not conducted when the two uplink bands of the analysis are not in same or adjacent band groups according to table 1.

A TP corresponding to proposal 1 have been submitted in [3].

Conclusion

In the paper, the following Observations and Proposals were made:

Proposal 1: We capture the band group rules in the TR guidelines with the proposed frequency ranges of [1] so that triple beat analysis is not conducted when the two uplink bands of the analysis are not in same or adjacent band groups according to table 1.

- References

[1] R4-2304941 Discussion of triple beat rules for MSD analysis, WG4 Meeting #106-bis-e

[2] R4-2220556 WF on triple beat rules and MSD for inter-band with 2UL with intra-band ULCA WG4 Meeting #105

[3] R4-2320022, TP to TR 38.846 Addition of Guidelines on Co-existence analysis for triple beat, Nokia, Nokia Shanghai Bell