3GPP TSG-RAN WG4 Meeting # 109											R4-2320022

Chicago, USA, November 13 – November 17, 2023

Agenda item:			8.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR38.846 of Guidelines om Co-existence analysis for triple beat

Document for:	Discussion and Approval

# Introduction

This is a TP to TR 38.846 to add the guidelines for analysis of triple beat, that may cause self-interference. It is based on the discussion paper [1] where also the proposal to add this analysis was presented.

TP for TR 38.846

**************** Start of TP ***************

## 6.5	Guidelines on Co-Existence analysis

### 6.5.2	Uplink Intra-Band CA with two UL CC transmissions

*************************** Unchanged CLAUSES Omitted ***************************

### 6.5.3	Uplink triple beat

When adding a band combination including three uplink transmission - one with UL intra-band carrier aggregation, which makes two tones and a third in the form of a single uplink component carrier this study is needed regardless if the intra-band CA is non-contiguous or contiguous intra-band uplink CA.

Table 6.5.2-1: Band nX and Band nY triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | - | - | - | - |  | . |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | - | - | - | - |  | - |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | - | - | - | - |  | - |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | - | - | - | - |  |  |

If any issues are identified via the calculations presented in Table 6.5.2-1 additional REFSENS requirements may be needed.

In the Table 6.5.2-1 the following abbreviations is used:

fU1L = minimum frequency of TX aggressor band of ULCC1 lower band range (i.e. Minimum frequency edge of the band)

fU2L = minimum frequency of TX aggressor band of ULCC2 lower band range (i.e. Minimum frequency edge of the band)

fU3L = maximum frequency of TX aggressor band of ULCC2 lower band range (i.e. Minimum frequency edge of the band + 			Maximum Instantaneous UL BW)

fU1H = maximum frequency of TX aggressor band of ULCC1 higher band range (i.e. Maximum frequency edge of the band)

fU2H = minimum frequency of TX aggressor band of ULCC2 higher band range

fU3H = maximum frequency of TX aggressor band of ULCC2 higher band range (i.e. Minimum frequency edge of the band - 			Maximum Instantaneous UL BW)

fSCCL = minimum frequency in single CC band

fSCCH = maximum frequency in single CC band

Channel BW = Channel bandwidth of the component carrier
Minimum channel separation = Minimum frequency separation between the two component carriers or the inter CC GB
Maximum channel separation = Maximum frequency separation between the two CCs or aggregated uplink BW

Figure 6.5.2-1: Co-existence studies triple beat

For third band in triple beat:

  - If the 3rd DL band belongs to the same band group or belongs to a band group which is adjacent to either one of the UL bands, where band groups are defined in Table 6.5.2-2 MSD must be speficied. If it belongs to a different band group the MSD can be skipped.

Table 6.5.2-2: Band group definition for adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-1 (LB) | FR1-2 (MB) | FR1-3 (HB) | FR1-4 (VHB) | FR1-5 (UHB) |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5250-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

**************** End of TP ***************

- References

[1] R4-2304941 Discussion of triple beat rules for MSD analysis, WG4 Meeting #106-bis-e