3GPP TSG-RAN WG4 Meeting # 109										R4-2320024

Chicago, USA, November 13 – November 17, 2023

Agenda item:			7.11.1

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR38.718-03-01 Addition of UE co-existence studies for triple beat

Document for:	Discussion and Approval

# Introduction

This is a TP to TR 38.718-03-01 to add the analysis for triple beat, that may cause self-interference in third RX band. It is based on the discussion paper [1] where also the proposal to add this analysis was presented. Looking at 38.718-03-01 that includes also x bands UL (x=1,2) the framework or IMD analysis is also found missing. This has been added also.

TP for TR 38.718-03-01

**************** Start of TP ***************

## 5.x		CA_nX-nY-nZ

### 5.x.1		Common for 1 band UL and 2 bands UL CA

*************************** Unchanged CLAUSES Omitted ***************************

#### 5.x.2.1	UE co-existence studies

Editor's note:  Co-existence study tables are recommended to be included for the third band, since IMD study only have been incorporated in the corresponding 2DL fallback band combination TR for the two bands.

Table 5.x.2.1-1 lists Band nX + Band nY 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.1-1: Band nX and Band nY UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | - | - | - | - |
| 2nd order IMD products | ¦fx_low – fy_high¦ | ¦fx_high – fy_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | - | - | - | - |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | - | - | - | - |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | - | - | - | - |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | - | - | - | - |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | - | - | - | - |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | - | - | - | - |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | - | - | - | - |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | - | - | - | - |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | - | - | - | - |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | - | - | - | - |

Based on Table 5.x.2.2-1, nth order IMD form band nX and Band nY may also fall into Rx frequencies of band nZ.

Table 5.x.2.2-1 lists Band nX + Band nY 2UL bands CA 1st order triple beat (IMD3) for the UE-to-UE coexistence analysis into the third receive band of Band nZ, where Band nX is the uplink band of the separated RB allocations and Band nY is the contiguous single uplink carrier.

Table 5.x.2.2-1: Band nX and Band nY triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | - | - | - | - |  | . |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | - | - | - | - |  | - |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | - | - | - | - |  | - |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | - | - | - | - |  |  |

Based on Table 5.x.2.2-1, 1st order triple beat IMD may also fall into Rx frequencies of bands nZ.

#### 5.x.2.2	REFSENS requirements

Editor's note: Text will be added on reference sensitivity exceptions if IMD issue due to dual uplink operation or triple beat falling into DL of the third band are identified.

**************** End of TP ***************

- References

[1] R4-2304941 Discussion of triple beat rules for MSD analysis, WG4 Meeting #106-bis-e