3GPP TSG-RAN WG4 Meeting #109	 R4-2320031

Chicago, US, November 13 – 17, 2023

Title:	On completion of MPR/PAR - objective

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	8.27.1.2

Document for:	Approval

# 1	Introduction

This contribution relates to a work item agreed in RAN#94-e, namely “Further NR coverage enhancements” [1]. We consider power domain enhancements and the following objectives captured in the WID:

- Study and if necessary specify following power domain enhancements

  - […]

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

In this paper we discuss completion of MPR/PAR reduction objective in Rel-18.

# 2	Scope of MPR/PAR reduction

Scope of MPR/PAR reduction was discussed in RAN #100 and the Proposal #1 in [2] was endorsed:

- Proposal #1 (Offline consensus)

  - No RAN1 specification impact is expected for MPR/PAR reduction in Rel-18 UL Coverage WI

    - RAN4 will define new optional requirements in the form of at least MPR reduction suitable for a transparent scheme (such as FDSS) that have no RAN1 specification impact

According to endorsed proposal RAN4 will define new optional requirement at least for MPR reduction for a transparent scheme, such as FDSS. Our assumption is that this means at least a set of new MPR requirements, however it could be useful to revise also other related requirements (such as spectrum flatness requirements) to achieve the gains that the feature can offer in real networks. Power boosting has also been discussed in the same context, introducing power boosting may affect the MPR requirements also.

# 3	Priority scenarios for MPR/PAR reduction

As shown below, there was considerable progress in RAN4 #106 to define priority scenarios for MPR/PAR reduction. Based on that RAN4 seems to have common understanding on the priority scenarios for MPR/PAR reduction.

Agreement:

- QPSK is the targeted modulation for further coverage enhancements

  - At least for simulation study

- RAN4 shall prioritizes DFT-S-OFDM as a solution for coverage enhancements

  - FSS on CP-OFDM if companies can show gains

- RAN4 shall evaluate both FR1 and FR2 scenarios.

- RAN4 shall not consider other channels and signals (than PUSCH and the associated DMRS)

- RAN4 shall not consider intra band UL CA scenario in Rel-18 WI

In RAN4 #108 a WF [3] was agreed with following:

Agreement:

-	Consider FDSS for the PAR/MPR reduction requirement.

Since FDSS is agreed to be considered as a scheme for PAR/MPR reduction, we have the following schemes on table:

- Reference case (legacy) without FDSS

- FDSS (transparent scheme)

Observation 1:  According to WID and agreements made until now, there are two transparent MPR/PAR reduction schemes on table:

- Reference case (legacy DFT-s-OFDM) without FDSS

- FDSS (transparent scheme)

Our assumption is that since FDSS is agreed to be considered as a scheme for PAR/MPR reduction, the new optional MPR requirements can be derived based on the FDSS scheme. Scheme itself does not need to be mandated.

Proposal 1:  Derive new MPR requirements based on FDSS.

Excerpt from WID [1] shown in Section 1 clearly states that the topic to be studied and specified, if seen necessary, is enhancement to reduce MPR/PAR, hence other enhancements that do not reduce MPR/PAR are not following the WID.

Following was agreed in RAN4 #108 in a WF [3]:

Agreement:

-	RAN4 proceed with the current simulation result.

- 	New simulation results provided during the work phase of the WI not precluded.

What comes to frequency range and power class, considering the timing of the WI, it would be good to concentrate to the ones that have results already available. This is also encouraged by the agreement from RAN4 #108 above. For FR2 transparent schemes, there are RF-results available only from two companies in the results Excel [5]. Also results for transparent schemes are available only for PC3. Hence it is proposed to prioritize FR1 and PC3 in Rel-18 WI, however also FR1 and PC2 can also be considered if there is strong support from the operators. In such case some analysis of PC2 would be good to be available, preferably from multiple sources.

Proposal 2:  Prioritize FR1 and power class 3 in Rel-18 WI.

Similar to the previous issue also for pi/2 BPSK there are results available only from two companies in the results Excel [5]. Hence our preference is to prioritize QPSK, as agreed also in RAN4 #106. Also pi/2 BPSK has already existing requirements introduced in earlier releases.

Proposal 3:  Prioritize QPSK modulation in Rel-18 WI.

# 4.	FDSS net gain analysis

The following agreement was made in RAN4 #106bis-e.

Agreement:

- Use the following equation as the baseline

- Net Gain [dB]=(Pout_(Filter i)-Pout_Ref )+(SNR10%_Ref-SNR10%_(Filter i) )

- Where Pout_(Filter i) is the achieved (TX) output power of the filtered  waveform being compared against the reference, Pout_Ref is the output power of the reference, SNR10%_Ref is the required SNR to achieve 10% BLER with the reference, and SNR10%_(Filter i) is the required SNR to achieve 10% BLER using the filtered waveform being compared against the reference.

- Use the following equation when providing the results for net gain

- Net Gain [dB]=-DOPR_M0 +(SNR10%_Ref-SNR10%_(Filter i) )

In the following we analyse the net gain using the agreed equation(s).

The set of configurations defining MCS/PRB allocations for simulations was made in RAN1 #111 and the configurations are shown in Table 1. These parameters ensure that such MCS/PRB combinations yield the same spectral efficiency, i.e., the results are comparable.

Table 1 Set of configurations for simulations

|  |  | No spectrum extension |  | With spectrum extension |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TBS value | Tput estimation for DDDSU @4GHz | #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor |
| 2408 | 963.2 kbps | 16 | 7 | 14 | 16 | 8 | 1/8 |
| 5376 | ~2.15 Mbps | 32 | 8 | 28 | 32 | 9 | 1/8 |
| 272 | 108.8 kbps | 8 | 0 | 6 | 8 | 1 | ¼ |
| 1032 | 412.8 kbps | 8 | 6 | 6 | 8 | 8 | ¼ |
| 2152 | ~0.9 Mbps | 40 | 2 | 30 | 40 | 3 | ¼ |
| 4992 | ~2.0 Mbps | 40 | 6 | 30 | 40 | 8 | ¼ |
| 552 | 220.8 kbps | 16 | 0 | 10 | 16 | 2 | 3/8 |
| 1736 | 694.6 kbps | 32 | 2 | 20 | 32 | 4 | 3/8 |
| 432 | 172.8 kbps | 8 | 2 | 6 | 8 | 3 | ¼ |
| 808 | 323.2 kbps | 24 | 0 | 18 | 24 | 1 | ¼ |

Frequency domain response of different filters used in evaluations are shown in Figure 1. As can be seen the filter with the least aggressive response is the 2-tap filter ([1 0.28]), whereas the most aggressive is the 3-tap [0.335 1 0.335]. Other filters have frequency responses in between the two.

Figure 1. Frequency-domain allocation with different spectrum shaping filters

OBO gain and net gain for each filter are shown in Table 2 for 700MHz case and Table 3 for 4GHz case of Nokia results provided in the Results Excel [5]. The filter with highest net gains are highlighted in green. The gains are shown w.r.t. legacy scheme (i.e. transmission without FDSS).

Observation 2: For 700MHz case:

- 2-tap filter (the least aggressive) maximizes the net gain in almost all cases​

- When using filter(s) maximizing the net gain:​

  - OBO gain for outer is 0.6 - 0.9 dB​

  - OBO gain for inner is typically 0.2-0.3 dB (however net gain is close to zero)

Observation 3: For 4GHz case:

- 2-tap filter (the least aggressive) maximizes the net gain in almost all cases​

- When using filter(s) maximizing the net gain:​

  - OBO gain for outer is close to 1dB​

  - OBO gain for inner is typically 0.2-0.4 dB (again, net gain is close to zero)​

There is more analysis of Nokia and other companies results in our contribution to the previous meeting [6].

Proposal 4: For FDSS, RAN4 shall focus on MPR reduction for outer allocations.

As can be seen in both carrier frequencies the 2-tap filter provides the best net gain in almost all cases. Also in the cases where 2-tap is not providing the best gains the difference to the best net gain is rather small. Hence 2-tap filter seems to be a good option for optimizing the net gains. It looks in general that less aggressive filter optimizes the net gain in most cases.

Observation 4:  Less aggressive filter optimizes the net gain in most cases.

As noted above some cases, for narrow allocations, see benefit of different filter settings. It can therefore be proposed that RAN4 should consider different RF requirements (i.e. spectrum flatness) for different allocation sizes.

Proposal 5: RAN4 shall consider the allocation sizes when defining RF requirements (i.e. spectrum flatness).

Table 2 OBO gain and net gain for each filter in FR1 Rural case (700MHz)

Table 3 OBO gain and net gain for each filter in FR1 Urban case (4GHz)

# 5.	Effects to RAN4 requirements

RAN4 requirements relevant to transparent schemes (FDSS) are Maximum Power Reduction (MPR), the spectrum flatness requirements and ACLR. MPR requirement for power class 3 is shown in the Table 4 below. As can be seen for QPSK the MPR for outer and edge allocations is 1 dB and for inner allocations 0 dB. Since the inner MPR is zero, the reference power of 23 dBm can be reached in inner allocations, hence transmission power cannot be increased without power boosting.

Observation 5: Due to already zero MPR requirement, the transmission power for inner allocations cannot be increased without applying power boosting.

Power boosting has been introduced for pi/2 BPSK already in Rel-15, however additional MPR requirement with low PAPR 2 DMRS was added in Rel-16. Hence there are three different requirements for pi/2 BPSK. With power boosting the reference power is 26 dBm as stated in Note 1. Power boosting is applied only if the corresponding duty cycle requirement is met, and the power boosting has been configured.

Table 4 (38.101 Table 6.2.2-1) Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 | ≤ 0.52 | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | 02 | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40 % of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Spectrum flatness requirements are defined in 38.101 to ensure that the base station receiver can operate at adequate performance level due to the FDSS in the transmitter.

Observation 6:  Spectrum flatness requirements for the UE is needed to ensure base station receiver performance.

In RAN4 #108bis following was agreed in a WF [4]:

<Agreement>:

-	If spectrum flatness requirements from clause 6.4.2.4 in 38.101-1 for QPSK can not be met for the enhancement a new requirement shall be defined.

-	FFS if there is different requirements for different RB regions intended for power boosting

Agreement above discusses flatness requirement from clause 6.4.2.4, which is shown in Figure 2. This is a generic requirement that is applied whenever the relaxed requirement for pi/2 BPSK is not applied. Requirement is defined for normal and extreme conditions and the parameter numbers in brackets are for extreme conditions and the ones outside for normal conditions. Generic requirement is meant to allow certain ripple in implementation, e.g. band filtering and it doesn’t depend on the UE allocation. Range 2 is the frequency range next to the band edge allowing larger ripple, whereas Range 1 is farther away from band edges and has stricter requirement.

In order to allow FDSS for the pi/2 BPSK, the spectrum flatness requirement has been relaxed. Limits of spectral flatness requirements are illustrated in Figure 3 and actual requirement for pi/2 BPSK shown in Table 5. In Section 4 it was concluded that the filter with the least aggressive frequency response is optimizing the performance when FDSS is applied to QPSK (Observation 4). Hence for FDSS with QPSK the flatness requirement should be stricter than the one defined for pi/2 BPSK. Two examples of such requirement are shown in figure 4, where parameters X1=4dB, X2=8dB and X1=6dB, X2=10dB are used. It can be seen that even the 2-tap filter would not fit within the 4dB ripple in Range 1 of the generic requirement, hence relaxed requirement is needed. On the other hand it is good to avoid having unnecessarily strict requirement to prevent any impacts to the implementation. Hence X1=6dB, X2=10dB looks like a good option.

Observation 7: Even 2-tap filter does not meet the generic spectral flatness requirement and hence relaxed requirement is needed for FDSS.

Proposal 6: Introduce new spectral flatness requirement for FDSS with QPSK in Rel-18 together with MPR/PAR reduction.

Proposal 7: Discuss values for X1 and X2, X1=6dB and X2=10dB can be used as starting point.

f

FUL_High

FUL_High – 3(5) MHz

< 4(4) dBp-p

Range 1

Range 2

max(Range 1)-min(Range 2) < 5(6) dB

max(Range 2)-min(Range 1) < 7(10) dB

< 8(12) dBp-p

f

FUL_High

FUL_High – 3(5) MHz

< 4(4) dBp-p

Range 1

Range 2

max(Range 1)-min(Range 2) < 5(6) dB

max(Range 2)-min(Range 1) < 7(10) dB

< 8(12) dBp-p

Figure 2. (38.101 Figure 6.4.2.4-1) The limits for EVM equalizer spectral flatness with the maximum allowed variation.

Figure 3. (38.101 Figure 6.4.2.4.1-1) The limits for EVM equalizer spectral flatness with the maximum allowed variation.

Table 5 (38.101 Table 6.4.2.4.1-1) Mask for EVM equalizer coefficients for Pi/2 BPSK, normal conditions

Figure 4. Maximum attenuation masks (X1=4dB, X2=8dB) and (X1=6dB, X2=10dB) with different spectrum shaping filters

In RAN4 #108bis following was agreed in a WF [4]:

<Agreement>:

-	Power boosting for QPSK is available under network control. A new UE capability is assumed for transparent power boosting for QPSK.

- FFS if the same boosting can be applied for 2/pi BPSK with FDSS shall also be available under network control. The assumption is that legacy power boosting for 2/pi BPSK is not changed, and this is added to the QPSK new capability.

- FFS if this new capability is a single capability or there will be independent capabilities for e.g. power boosting alone or with FDSS.

<Agreement>:

-	When boosting is enabled a UE shall only be allowed to boost/increase its output power from its nominal power class with a maximum of [1dB]. The tolerances for the nominal power class applies to the boosted power level.

<Agreement>:

-	Reuse the current RB region definition. i.e. outer/edge and inner.

-	For the inner region define a new sub-set used for the enhancement.

-	Other sub-set(s) regions are not precluded.

<Agreement>:

-	Power boosting will be defined within the inner region.

-	Power boosting is equivalent to [ΔPpowerboosting=1dB] applied to Ppowerclass.

-	FFS on whether power boosting is define for outer region or other region

-	Whether or not to finalize outer region or other region does not impact the completion of WI

WID [1] and RAN #100 endorsed proposal 1 are both assuming MPR reduction to be introduced in Rel-18 WI, hence there is a need to discuss the exact way how to do it. We have the earlier conclusions:

- Inner Allocations:​

  - Net gain is small for inner RB allocation

  - No room for MPR reduction without power boost

- Outer Allocations:

  - OBO gain is up-to 1dB (when using filter maximizing the net gain).

  - The previous bullet corresponds to room for MPR reduction without power boost

Based on these and the agreements above a direction can be anticipated for MPR reduction:

- Extend power boosting framework defined for pi/2 BPSK FDSS to cover also QPSK FDSS

- Update MPR table (PC3) for outer RB allocations

  - Define new MPR values for QPSK, at least for outer allocations

  - New MPR values could depend also on the allocation size.

- Introduce ΔPpowerboosting values (PC3) for both inner and outer RB allocations with FDSS

- Extend flatness requirements defined for pi/2 BPSK FDSS to cover also for QPSK FDSS

  - Define requirements in such that only less aggressive filters (such as 2-tap [1 0.28]) can be used

The spectrum flatness requirement should be introduced to allow FDSS without excessive receiver performance losses, however introduction of relaxed spectral flatness requirement should be connected to corresponding MPR reduction.

The approach presented should be discussed together with MPR values possible to be applied in each case and MPR reduction for Rel-18 should be introduced. Excerpt from WID [1] shown in Section 1 clearly states that the topic to be studied and specified, if seen necessary, is enhancement to reduce MPR/PAR, hence reduction of MPR/PAR is necessary to meet the WID objectives.

Proposal 8: Discuss the approach presented and related MPR values and introduce MPR reduction for Rel-18.

Duty cycle has been discussed in earlier RAN4 meeting. In our view similar approach could be used as has been used for pi/2 BPSK. Actual value of the duty cycle threshold can be discussed in RAN4 when the amount of power boost has been agreed for each case.

Proposal 9: Reuse duty cycle threshold approach used in pi/2 BPSK for Rel-18 MPR/PAR reduction

In RAN4 #108 following was agreed in a WF [3]:

Way forward:

-	Companies are encouraged to bring proposals in showing needed changes to the specification for the next meeting.

A CR has been submitted for this meeting in [7] which shows what is intended change related to the approach above.

Capabilities for new MPR/PAR requirements have also been discussed and there is agreement shown above to introduce capability for transparent power boosting. New capability would be needed also for fulfilling the new optional MPR requirements. Having separate capability for both makes it difficult to benefit from the Rel-18 feature since the coverage gain is limited to small fraction of frequency band allocations. Hence it would be beneficial to avoid fragmenting Rel-18 MPR/PAR reduction further with a separate capability for power boosting and MPR/PAR reduction In any case RAN2 should be informed of the need for the new capability.

Proposal 10: Introduce single capability for Rel-18 MPR/PAR reduction covering both power boosting and transparent MPR/PAR reduction.

# 6.	Conclusion

In this paper we discuss the completion of MPR/PAR reduction in Rel-18. Based on the discussion we make the following observations and proposals:

Observation 1:  According to WID and agreements made until now, there are two transparent MPR/PAR reduction schemes on table:

- Reference case (legacy DFT-s-OFDM) without FDSS

- FDSS (transparent scheme)

Observation 2: For 700MHz case:

- 2-tap filter (the least aggressive) maximizes the net gain in almost all cases​

- When using filter(s) maximizing the net gain:​

  - OBO gain for outer is 0.6 - 0.9 dB​

  - OBO gain for inner is typically 0.2-0.3 dB (however net gain is close to zero)

Observation 3: For 4GHz case:

- 2-tap filter (the least aggressive) maximizes the net gain in almost all cases​

- When using filter(s) maximizing the net gain:​

  - OBO gain for outer is close to 1dB​

  - OBO gain for inner is typically 0.2-0.4 dB (again, net gain is close to zero)​

Observation 4:  Less aggressive filter optimizes the net gain in most cases.

Observation 5: Due to already zero MPR requirement, the transmission power for inner allocations cannot be increased without applying power boosting.

Observation 6:  Spectrum flatness requirements for the UE is needed to ensure base station receiver performance.

Observation 7: Even 2-tap filter does not meet the generic spectral flatness requirement and hence relaxed requirement is needed for FDSS.

Proposal 1:  Derive new MPR requirements based on FDSS.

Proposal 2:  Prioritize FR1 and power class 3 in Rel-18 WI.

Proposal 3:  Prioritize QPSK modulation in Rel-18 WI.

Proposal 4: For FDSS, RAN4 shall focus on MPR reduction for outer allocations.

Proposal 5: RAN4 shall consider the allocation sizes when defining RF requirements (i.e. spectrum flatness).

Proposal 6: Introduce new spectral flatness requirement for FDSS with QPSK in Rel-18 together with MPR/PAR reduction.

Proposal 7: Discuss values for X1 and X2, X1=6dB and X2=10dB can be used as starting point.

Proposal 8: Discuss the approach presented and related MPR values and introduce MPR reduction for Rel-18.

Proposal 9: Reuse duty cycle threshold approach used in pi/2 BPSK for Rel-18 MPR/PAR reduction

Proposal 10: Introduce single capability for Rel-18 MPR/PAR reduction covering both power boosting and transparent MPR/PAR reduction.

# References

- RP-213579, “New WI: Further NR coverage enhancements”, China Telecom

- RP-231498, “Summary for discussion [RAN1-UL-Coverage]”. NTT DoCoMo

- R4-2314910, “WF for NR_cov_enh2_part2”, Nokia, Nokia Shanghai Bell

- R4-2317652, “WF on enhancement for MPR reduction”, Nokia

- R4-2306627, “Simulation Results for NR_cov_enh2_part2_v4”

- R4-2313029, “Next steps for MPR/PAR – objective”, Nokia, Nokia Shanghai Bell

- R4-2320032, “CR to 38.101 introduction of MPR reduction”, Nokia, Nokia Shanghai Bell