3GPP TSG-RAN WG4 Meeting # 109						R4-2320038

Chicago, USA, November 13 – November 17, 2023

Title:	TP to TR 38.718-02-01 Addition of CA_n77-n102 and DC_n77-n102

Source:	Nokia, BT

Agenda item:	7.10.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.718-02-01 to add CA_77A-n102A with UL interband CA and variants of non-contiguous and contiguous intra-band carrier aggregation in downlink including UL interband CA. It also includes the NR DC combinations of the same.

This TP is dependent on fallback also submitted for same meeting in R4-2320035

************************************* Start of TP*****************************************

## 5.52		CA_n77-n102

### 5.52.1	Common for 1 band UL and 2 bands UL CA

#### 5.52.1.1 Operating bands for CA

Table 5.52.1.1-1: CA band combination of band n77+n102

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n102 | 5925 MHz | – | 6425 MHz | 5925 MHz | – | 6425 MHz | TDD |

#### 5.52.1.2	Channel bandwidths per operating band for CA

Table 5.52.1.2-1: Supported bandwidths per CA band combination of band n77+n102

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n77A-n102A | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n77A-n102(2A) | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n77A-n102B | CA_n77A-n102A CA_n77A-n102B | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n77A-n102C | CA_n77A-n102A CA_n77A-n102C | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n77A-n102D | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n77A-n102E | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n77(2A)-n102A | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4&5 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n77(2A)-n102(2A) | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4&5 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n77(2A)-n102B | CA_n77(2A) CA_n77A-n102A CA_n77A-n102B | n77 | CA_n77(2A)_BCS4&5 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n77(2A)-n102C | CA_n77(2A) CA_n77A-n102A CA_n77A-n102C | n77 | CA_n77(2A)_BCS4&5 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n77(2A)-n102D | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4&5 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n77(2A)-n102E | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4&5 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |

#### 5.52.1.3	Co-existence studies

Table 5.52.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n77-n102.

Table 5.52.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on the table above there is no UL harmonic issues of this combination.

Table 5.52.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on the table above there is no Harmonic mixing issues of this combination.

#### 5.52.1.4	∆TIB and ∆RIB values

For CA_n77-n102, the TIB,c is based on CA_n78-n102. For CA_n77-n102, the RIB values are also based on CA_n78-n102.

Table 5.31.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n77-n102 | 1.5 | 1.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.31.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n77-n102 | 0.5 | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.52.1.5	REFSENs requirements

As can be seen in the co-existence studies in 5.52.1.3 there are no harmonics issues.

#### 5.52.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

### 5.52.2		Specific for 2 bands UL CA

#### 5.52.2.1	Maximum output power for inter-band CA

Table 5.52.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n77A-n102A | 23 | +2/-3 |

#### 5.52.2.2		UE co-existence

Table 5.52.2.2-1 gives IMD interference analysis for CA_ n77-n102 with uplink CA.

Table 5.52.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3300 | 4200 | 5925 | 6425 |
| DL Frequency [MHz] | 3300 | 4200 | 5925 | 6425 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1725 | 3125 | 9225 | 10625 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 175 | 2475 | 7650 | 9550 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 12525 | 14825 | 15150 | 17050 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3475 | 6675 | 13575 | 15975 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6250 | 3450 | 18450 | 21250 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 15825 | 19025 | 21075 | 23475 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 22400 | 19500 | 10875 | 6775 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 12675 | 9375 | 750 | 2950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 27000 | 29900 | 19125 | 23225 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 24375 | 27675 | 21750 | 25450 |

Based on the table 5.52.2.2-1 there are no intermodulation products falling inside the RX bands.

Since n102 is located as shared spectrum there is simultaneous RX/TX in this TDD-TDD combination, which requires an analysis of non-contiguous uplink operation. The method of [1] is applied. For the non-contiguous intra-band ULCA of n77(2A) the impact on n102 RX is analysed using the method of [1].

Table 5.31.2.3-2: IMD analysis non-contiguous UL in n77

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3340 | 3900 | 4200 | 4160 | 3600 |  | 20 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |  | Min Ch. separation |
| Interference ranges | 40 | 600 | 6640 | 8360 |  |  |  | 20 |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |  | Max. Ch. separation |
| Interference ranges | 2700 | 4800 | 9940 | 12560 |  |  |  | 600 |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |  |  |
| Interference ranges | 80 | 1200 | 6000 | 9000 | 13240 | 16760 |  |  |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |  |  |
| Interference ranges | 2100 | 5400 | 9300 | 13200 | 16540 | 20960 |  |  |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |  |  |
| Interference ranges | 120 | 1800 | 5400 | 9600 | 12600 | 17400 |  |  |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |  |  |
| Interference ranges | 1500 | 6000 | 8700 | 13800 | 15900 | 21600 |  |  |

The analysis shows that the 4th, 6th and 7th order IMD product of n77(2A) falls inside n102 during simultaneous RX/TX operation. However, 38.101-1 does not capture IMD into unlicensed or shared spectrum bands. No need to define MSD.

Since this is including a share spectrum access band there is no additional protected bands as compared to n77.

As this is a TDD-TDD combination the triple beat analysis for CA_n77A-n102B and CA_n77A-n102C will never impact any downlink of the two bands, since there’s no triple beat uplink if either band is in downlink operation.

#### 5.52.2.3		REFSENS requirements

No additional MSD requirement for IMD is needed.

********************    Removed sections unaffected         *******************************

## 7.9	DC_n77-n102

### 7.9.1	Configurations for DC_n77-n102

Table 7.9.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC Configuration |
| --- | --- |
| DC_n77A-n102A | DC_n77A-n102A |
| DC_n77A-n102B | DC_n77A-n102B |
| DC_n77A-n102C | DC_n77A-n102C |
| DC_n77A-n102D |  |
| DC_n77A-n102E |  |
| DC_n77A-n102(2A) | DC_n77A-n102A |
| DC_n77(2A)-n102A | DC_n77A-n102A |
| DC_n77(2A)-n102B | DC_n77A-n102B |
| DC_n77(2A)-n102C | DC_n77A-n102C |
| DC_n77(2A)-n102D |  |
| DC_n77(2A)-n102E |  |
| DC_n77(2A)-n102(2A) | DC_n77A-n102A |

### 7.9.2	Maximum output power for NR-DC

Table 7.9.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n77A-n102A | 23 | +2/-3 |

************************************* End of TP*****************************************

# 2	References

[1] R4-2304941 Discussion of triple beat rules for MSD analysis, RAN4#106-e