3GPP TSG-RAN WG4 Meeting # 109						R4-2320039

Chicago, USA, November 13 – November 17, 2023

Title:	TP to TR 38.718-02-01 Addition to CA_n1-n102 and DC_n1-n102

Source:	Nokia, BT

Agenda item:	7.10.2

Document for:	Approval

# 1	Introduction

This TP was partially accepted through [1] and this is an addition to TR 38.718-02-01 to add band n1 and contiguous intra-band carrier aggregation in uplink band n102.

This TP is dependent on fallback also submitted for same meeting in R4-2320035

************************************* Start of TP*****************************************

## 5.32		CA_n1-n102

### 5.32.1	Common for 1 band UL and 2 bands UL CA

#### 5.32.1.1 Operating bands for CA

Table 5.32.1.1-1: CA band combination of band n1+n102

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n102 | 5925 MHz | – | 6425 MHz | 5925 MHz | – | 6425 MHz | TDD |

#### 5.32.1.2	Channel bandwidths per operating band for CA

Table 5.32.1.2-1: Supported bandwidths per CA band combination of band n1+n102

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n102A | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n102(2A) | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n1A-n102B | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n1A-n102B | n102 | CA_n102B_BCS0 |  |
| CA_n1A-n102C | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | CA_n1A-n102C | n102 | CA_n102C_BCS0 |  |
| CA_n1A-n102D | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n1A-n102E | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |

#### 5.32.1.3	Co-existence studies

Table 5.32.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n1-n102.

Table 5.32.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 | 9600 | 9900 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, the 3rd UL harmonic of n1 falls inside the n102 RX band. Highest channel of n1 with 5MHz BW results in a 3rd order UL harmonic at 5932.5MHz, which will collide with 7.5MHp overlap of the lowest 20MHz channel allocation of n102 centered at 5935MHz.

Table 5.32.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 | 10550 | 10850 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, the UL n102 falls inside the 3rd order harmonic of n1 RX band (n102 – n1 UL1/DL3). Since CA_n1-n102 is defined with UL CA as well the harmonic mixing case above may occur in the UL CA case, but not the single UL case, since n1 will have the PCell in the 1 UL scenario.

#### 5.32.1.4	∆TIB and ∆RIB values

For CA_n1-n102, the TIB,c is based on CA_n1-n77 having n77 as the highest frequency range among similar CA’s. For CA_n1-n102, the RIB values are also based on CA_n1-n77

Table 5.32.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n1-n102 | 0.6 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.32.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n1-n102 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.32.1.5	REFSENs requirements

Based on the co-existence studies there are 3rd uplink harmonic from band n1 UL that partially falls inside band n102 DL at lowest channel allocation. Since this is a share spectrum access band we apply an exclusion region to mitigate this issue.

Table 7.3A.4-1a: NR-U reference sensitivity measurement exclusion region in MHz.

| NR Band / Harmonic order / Channel BW in UL |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL Band | Harmonic order | DL Band | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40MHz | 50 MHz |
| n1 | 3 | n102 | +/- 10 | +/- 20 | +/- 30 | +/- 40 | +/- 50 | +/- 60 | +/- 80 | +/- 100 |
| NOTE 1: Even though UL harmonic does not fall directly into NR-U band the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the harmonic order. |  |  |  |  |  |  |  |  |  |  |

Based on the co-existence studies there are 3rd harmonic mixing between band n102 UL1 into band n1 DL3. Similar UL1/DL3 cases show in the range of 20dB of relaxation for the larger UL BW case. Since n102 doesn’t support less than 20MHz UL BW we define only this test point. It is expected that n1 and n102 don’t share antenna, which is not the case for other UL1/DL3 cases. So, we assume higher attenuation of the aggressor and therefore use the minimum value of previous UL1/DL3 cases.

Table 5.32.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n102 | n1 | 20 | 15 | 100 (RBstart=0) | 5 | 13.5 | NOTE 4 | UL1/DL3 |
| NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

#### 5.32.1.6	OOB blocking exception requirements

Exceptions added for CA_n1-n102

Table 5.32.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n1-n102

### 5.32.2		Specific for 2 bands UL CA

#### 5.32.2.1	Maximum output power for inter-band CA

Table 5.32.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n102A | 23 | +2/-3 |

#### 5.32.2.2		UE co-existence

Table 5.32.2.2-1 gives IMD interference analysis for CA_ n1-n102 with uplink CA.

Table 5.32.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1920 | 1980 | 5925 | 6425 |
| DL Frequency [MHz] | 2110 | 2170 | 5925 | 6425 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3945 | 4505 | 7845 | 8405 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2585 | 1965 | 9870 | 10930 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9765 | 10385 | 13770 | 14830 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 665 | 15 | 15795 | 17355 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 9010 | 7890 | 15690 | 16810 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11685 | 12365 | 19695 | 21255 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 23780 | 21720 | 1995 | 1255 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 15435 | 13815 | 5910 | 7090 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 25620 | 27680 | 13605 | 14345 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 21615 | 23235 | 17610 | 18790 |

Based on the table 5.32.2.2-1, the 3rd order intermodulation product fall inside the RX band of n1.

Table 5.32.2.2-3 gives an IMD interference analysis for CA_n1-n102 with uplink CA for triple beat using the method of [2].

A triple beat study having n102C transmit in two 20MHz blocks with a 160MHz separation indicates IMD3.

Table 5.32.2.2-3: Triple beat IMD analysis of n102C

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 5925 | 5965 | 6085 | 6425 |  | 20 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 1920 | 1980 | 6385 | 6265 |  | 20 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 1760 | 1960 | 1940 | 2140 |  | 160 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 9910 | 10890 | 13810 | 14790 |  |  |

The uplink of n1A and n102C has 1st order triple beat falling inside n1 DL.

Since this is including a share spectrum access band there is no additional protected bands as compared to n1.

#### 5.32.2.3		REFSENS requirements

Table 5.32.2.3-1 lists the MSD required due to the 3rd IMD product for the dual uplink configuration. The product that hits n1 DL is similar to that which hits in CA-n1-n3, which is also an IMD3 case. Looking at the architecture of the RFFE there would be an additional attenuation between the antennas due to increased frequency selectivity at the large spectral distance. We propose a 10dB improved relaxation to the MSD compared to the CA_n1-n3 case considering also the minimum UL CBW of n102.

As stated for figure 1 in [3] the MSD value is not eligible if the two UL inter-bands aren’t part of the same or adjacent band group. The test below must capture that there is in fact no MSD.

Table 5.32.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n102 | n1 | 1977.5 | 5 | 25 | 2167.5 | 13 | FDD | IMD3 |
|  | n102 | 5935 | 20 | 100 | 5935 | N/A | TDD | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 0 | FDD | IMD3 |
|  | n102 | 6025 | 100 | 1 RBstart=14 | N/A | N/A | TDD | N/A |
|  |  | 6125 | 100 | 1 RBstart=260 | N/A |  |  |  |

********************    Removed sections unaffected         *******************************

## 7.6	DC_n1-n102

### 7.6.1	Configurations for DC_n1-n102

Table 7.6.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC Configuration |
| --- | --- |
| DC_n1A-n102A | DC_n1A-n102A |
| DC_n1A-n102B | DC_n1A-n102B |
| DC_n1A-n102C | DC_n1A-n102C |
| DC_n1A-n102D |  |
| DC_n1A-n102E |  |
| DC_n1A-n102(2A) | DC_n1A-n102A |

### 7.6.2	Maximum output power for NR-DC

Table 7.6.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n1A-n102A | 23 | +2/-3 |

************************************* End of TP*****************************************

# 2	References

[1] R4-2303582 TP to TR 38.718-02-01 Addition of CA_n1-n102 and DC_n1-n102, RAN4#106

[2] R4-2304941 Discussion of triple beat rules for MSD analysis, RAN4#106-e

[3] R4-2220556 WF on triple beat rules and MSD for inter-band with 2UL with intra-band ULCA, RAN4#105