3GPP TSG-RAN4 Meeting # 109	R4-2320051

Chicago, USA, November 13- 17, 2023

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.858 – MR BS feasibility in FR1

Agenda Item:	8.19.2.2.1

Document for:	endorsement

- Introduction

In this document we propose a TP to TR 38.858, adding additional analysis on Nokia’s views regarding the co-channel inter-sub-band co-site inter-sector interference. Moreover, conclusions and summary for the inter-sector interference are added using the latest endorsed TP to TR 38.858 (R4-2317010) as baseline.

# Text proposal

## <<End of Change for TR 38.858>>

## 9.3	Feasibility of FR1 medium range BS aspects

### 9.3.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results.

#### 9.3.1.1	Summary table for self-interference analysis

Table 9.3.1.1-1 summarizes the analysis from different companies on the FR1 MR base station. This section is based on the self-interference analysis framework.

Table 9.3.1.1-1: Summary of FR1 medium range BS self-interference analysis

| FR1 |  |  |  |  | Nokia | Ericsson |  |  |  | ZTE | Huawei | Samsung |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  |  | Medium Range BS | Medium Range (3GPP minimum requirements) | Medium range (Realistic) | Medium Range (Optimistic RX) | Medium Range (Realistic, lower power) | MR | Medium Range BS | Medium Range BS |
| BS TX Power dBPTX = ① dBm |  |  |  |  | 36 dBm | 38 dBm | 38 dBm | 38 dBm | 35 dBm | 30 | 38 dBm | 38 dBm |
| Component capability and parameters | Frequency isolation at TX |  | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 51 | 45 | 45 dBc |
|  |  |  | Frequency isolation techniques used |  | Digital filtering or windowing to clean UL sub-band; DPD to suppress PA distortion | Digital filtering, CFR, DPD |  |  |  | DPD utilized | DPD | DPD utilized |
|  | Spatial isolation |  | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 60 dBc (if omnidirectional antennas are used, this would be less) | 65-70 dBc | 65-70 dBc | 65-70 dBc | 65-70 dBc | 60 | 80 | 80 dBc |
|  |  |  | Spatial isolation techniques used |  | Spatial separation between TX/RX panels; EM shielding structures between TX/RX panels | A combination of spatial isolation, chokes, absorption, mushroom EBG. 70dB is indicative average; isolation varies from around 55dB to more than 80dB |  |  |  | TX/RX panel separation | spatial separation between TX/RX panel | TX/RX panel separation and RF barrier structure |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  |  | 0-5 dBc | 10 dBc Note that the TX beam nulling reduces the variation due to beam direction, and hence spatial isolation + TX nulling can be assumed to be 80dB for most directions. |  |  |  | 0 | 5 | 5 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  | 1 dB maximum | Up to 5dB EIRP loss, depending on beam direction |  |  |  | N/A | Limited | Limited, ~0dB |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  |  | -75 dBm/20 MHz | -87 dBm | -87 dBm | - 87 dBm | -90 dBm | -101 | -98 | -92 dBm Note: provided by ①-②-③-⑨ dBm |
|  | RF IC and other tech. (before LNA) |  | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 20 | N/A | 0 dBc |
|  |  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 dBc | 0 | N/A | 0 dBc |
|  |  |  | RF IC techniques and other tech. (before LNA) |  | None apply due to feasibility concerns (depends on the number of TRX) | None; see section 9.3.1.2.2 for analysis. |  |  |  | Subband filtering | N/A | N/A |
|  |  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | N/A dBc | >=5dBc if e.g. filtering or analogue IC would be applied. |  |  |  |  | N/A | N/A |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  |  | -24 dBm to -29 dBm depending on TX beam | -42 dBm | -42 dBm | -42 dBm | -45 dBm | -50 | -47 | -47 dBm Note: provided by ①-③-④-⑤dBm |
|  | Blocker Suppression at RX |  | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 0 dBc | 0 dBc |  |  |  | 60 | digital filtering | 40 dBc |
|  |  |  | Frequency isolation techniques |  | None apply due to feasibility concerns | Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. Digital baseband combining may improve self-interference suppression, at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  |  | Digital Filtering | digital filtering | Filtering |
|  |  |  | RX IMD | Rx IIP3 capability (dBm) | -10 dBm at maximum sensitivity; +10 dBm at maximum linearity (at NF penalty) | -27.6 dBm | -17.6 dBm | -13 dBm | -17.6 dBm | -5 | -5 | -20 |
|  |  |  |  | Rx IM3 contribution (dBm) | Negligible | -70.8 dBm | -90.8 dBm | -100 dBm | -100 dBm | -140 | -131 | -101dBm |
|  |  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Negligible | No significant issues for medium range BS power level other than mentioned above |  |  |  | Marginal | negligible | N/A |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  |  | -74 to -79 dBm/20MHz (at 50 dBc ACS) | -70.8 dBm | -90.8 dBm | -100 dBm | -100 dBm | -110 | negligible | -87 dBm Note: provided by ①-③-④-⑤-⑥dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  |  | 0 dBc | 0-10 dBc RX beam nulling is in effect part of the digital baseband combining. See “frequency isolation techniques” |  |  |  | 0 | 5 | 5 dBc |  |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  | 0 dBc; should not assume further UL beamforming loss to maintain any UL gains |  |  |  |  | N/A | Limited | Limited, ~0dB |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  |  | 5-10 dBc (Improved linearization could provide here additional 5dB. Digital IC depends on the implementation) | 10-15 dBc (Transmitter) |  |  |  | 20 | 10 | 10 |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  |  | 110 dBc to 120 dBc | 109 dBc | 128-138 dBc | 135 dBc | 134 dBc | 130.5 | 146 | 134 dBc |
| Noise floor ⑩dBm |  |  |  |  | -92 dBm/CBW (20 MHz) | -90 dBm/CBW | -90 dBm/CBW | -90 dBm/CBW | -90 dBm/CBW | -91.0 | -90 dBm/20 MHz | -90 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  |  | -98 dBm | -96 dBm | -96 dBm | -96 dBm | -96 dBm | -97.0 | -96 dBm | -96dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  |  | 134 dBc | 134 dBc | 134 dBc | 134 dBc | 131 dBc | 127.0 | 134 | 134 dBc |
| SBFD configuration |  |  |  |  | DUD (40/20/40 MHz) | 40-20-40 |  |  |  |  | 40-20-40 | DUD(40-20-40MHz) |
| Guardband assumption (if exist) |  |  |  |  | 5 RB (1.8 MHz) | 5 PRB |  |  |  |  | Existing SU | 5 PRB |
| bandwidth over which suppression is achieved |  |  |  |  |  | <300MHz |  |  |  |  |  | 100MHz |
| Others |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |  |  |  |  |

#### 9.3.1.2	Feasibility study on self-interference

Editor’s note: This section captures the feasibility study on self-interference based on individual companies’ analysis.

##### 9.3.1.2.1	Nokia

Table 9.3.1.1-1 presents the company’s view on the self-interference mitigation analysis for a medium range base station with 36 dBm total output power. The RSIC capability corresponds to 110 to 120 dBc which is achieved using a combination of spatial isolation (60 dBc), Tx beam nulling (0-5 dB, only applicable in the Tx subband), frequency separation (45 dBc), and digital IC techniques (5-10 dBc). Under such considerations, the self-interference observed in the 20 MHz UL subband is dominated by transmitter ACLR and receiver selectivity with a magnitude of approximately -74 dBm prior to digital IC and up to -84 dBm after digital IC, while the noise floor is -91.0 dBm. This results in a non-negligible desensitization of the receiver which would degrade more or less the system performance depending on the deployment ISD, UL link budget, etc.

The assumptions and corresponding justification are presented below with focus on the main differences with respect to the wide-area analysis in Section 9.2.1.2.6.

Spatial isolation

The same techniques to achieve spatial isolation in FR1 wide-area base stations can be applied to medium-range base stations as well. Nevertheless, the following differences should be noted:

- Medium range BS have typically smaller form-factor than the wide-area BS, e.g. down to 30x30x10 cm. The absolute physical separation between Tx and Rx panels needs to be smaller than for wide-area BS to keep the relative increase of the BS enclosure to a reasonable level.

- Medium range base stations may have a lower number of TRXs as compared to wide-area base stations. This could result in higher coupling per Rx chain.

Considering these two aspects, 60 dBc of spatial isolation is considered.

TX Beam nulling / isolation in TX sub-band

With only a relatively low number of TRXs, the potential of Tx beam nulling techniques is reduced. Only 0-5 dBc is considered, under the assumption of at most 1 dB of EIRP loss.

RF IC and other tech. (before LNA)

If the number of TRX is significantly lowered compared to massive MIMO, there is a possibility for RF IC to be feasible. However, this all depends on the implementation. With a low number of TRXs, the spatial isolation to a single RX port would likely be lower than for an implementation with a large number of TRXs – possibility necessitating RF cancellation paths to compensate. The overall SIC capability would be similar in both cases.

Frequency isolation at RX

This all depends on the implementation.

Digital IC

If the number of TRX is significantly lowered compared to massive MIMO, there is a possibility for digital IC to be feasible. However, this all depends on the implementation. Digital IC can be achieved by improving the DPD in the transmitter side to reduce the unwanted transmitter leakage components that fall in the UL subband, while receiver-side digital IC techniques are also required to supress the self-interference signal in gNB RX subband caused by non-ideal RX selectivity.

##### 9.3.1.2.2	Ericsson

For the medium range BS, four implementation scenarios are presented in Table 9.3.1.1-1. In the first scenario, the RF is assumed to perform at the absolute minimum needed to meet the 3GPP requirements. In the second scenario, an improved RF performance that is still likely to be a reasonable commercial implementation is considered (called realistic). Two further scenarios are considered; one in which the receiver linearity is assumed to exceed the realistic scenario, which could lead to more significant compromises in power consumption, size etc. The other scenario is one in which realistic RF performance is assumed, but the transmitter is assumed to be 3dB lower than the maximum transmitter limit in power.

It can be observed that for a BS only built to meet 3GPP minimum requirements, the receiver performance is not sufficient to operate SBFD without significant desensitization. To operate SBFD, either receiver digital processing is needed with the realistic assumptions (the feasibility of achieving sufficient gain with such processing depends on the wider deployment scenario), or somewhat better receiver performance, or lower transmit power than the 3GPP maximum limit.

An explanation for the assumptions in the table are provided below.

Spatial isolation and beam nulling

For a medium range size AAS array, simulations show a spatial isolation of around 65-70dB, depending on the beam direction. With beam nulling, the isolation can be lifted to around 80dB, with in general less than 1dB cost in the downlink.

Analogue interference cancellation

With a smaller array size, analogue interference cancelation may be more feasible for a MR BS than for a WA BS. However, the number of interconnections is still significant. Furthermore, analogue IC requires that the same beam steering is applied on all RBs and all carriers, preventing sub-band precoding and multi-carrier operation. Also, in general analogue IC is not needed to avoid receiver saturation. Analogue IC has not been taken into account.

Analogue filtering

Analogue filtering is not realistic in an implementation for the same reasons described for the WA BS in section 9.2.1.2.2. Furthermore, analogue filtering is not really needed as the input power level to the MR receiver is generally manageable.

Digital interference cancellation and receiver processing

Digital sampling of the TX leakage interference and subtraction at RX is more feasible for a MR BS than for a WA BS. An alternative to digital cancellation of TX leakage could be an improved PA linearization at the transmitter side. The possibility of either improved PA linearization or digital IC is captured as 15dB digital IC.

At the receiver, the interference co-variance matrix can be estimated and the receiver MMSE-IRC algorithm can mitigate interference. The extent to which the receiver can mitigate interference depends on the overall interference structure, which depends on the profile of interfering UEs, other sectors and other base stations as well as the fading channel profiles in the environment. The study has not considered the deployment environment when considering feasibility of self-interference suppression, and hence a specific number for the suppression by means of receiver processing is not provided. However, it is noted that for the MR to operate with the “realistic” receiver and the maximum allowed transmit power, several dB of suppression would need to be achieved by digital processing.

##### 9.3.1.2.3	ZTE

Table 9.3.1.1-1 presents the company’s view on the self-interference mitigation analysis for a medium range base station.

##### 9.3.1.2.4	Samsung

Table 9.3.1.1-1 presents the company’s view on the self-interference mitigation analysis for a medium range base station.

9.3.1.3	 	Conclusion

Based on the self-interference analysis provided in Section 9.3.1 for FR1 medium range BS, it can be observed that the implementation feasibility of controlling the residual interference to meet the 1dB receiver desensitization target depends on the implementation aspects, including:

- Maximum BS transmit power

- Spatial isolation capability

- Beam nulling/ isolation capability

- Blocker suppression at the RX

- Frequency isolation at the TX and RX

- The digital interference suppression/ cancellation capabilities

Based on the different assumptions and/ or techniques adopted for the above-mentioned implementation aspects, based on 5 companies’ technical inputs, 4 companies have concluded that it is feasible to achieve the 1 dB desensitization target. In addition, 2 companies have shown that it is not feasible to achieve the 1 dB desensitization target for a base station that strictly meets the RAN4 minimum requirements and/or operates with the maximum BS transmit power.

### 9.3.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and analysis results

#### 9.3.2.1	Summary table for co-channel inter-subband co-site inter-sector interference  analysis

Table 9.3.2.1-1 summarizes the analysis on the co-channel inter-subband co-site inter-sector interference for FR1 MR base station. This section is based on the co-channel inter-subband co-site interference analysis framework.

Table 9.3.2.1-1: Summary of FR1 medium range BS co-channel inter-subband co-site inter-sector interference analysis

| FR1 |  |  |  | Ericsson | Nokia |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Medium Range BS | Medium Range BS |  |
| BS TX Power dBPTX = ① dBm |  |  |  | 38 dBm | 36 dBm |  |
| Number of co-site co-channel sectors considered |  |  |  | 2 | 2 |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαCSSI-frequency, TX = ② dBc |  | 45 dBc | 45 dBc |  |
|  |  | Frequency isolation techniques used |  | DPD, CFR | DPD, digital filtering |  |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI-spatial = ③ dBc |  | 75-90 dBc (Varies depending on scheduled beam directions) | 60-80 dBc |  |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Typical site layout with around 400mm between sectors | Spatial separation between TX/RX panel; cross polarization |  |
|  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαCSSI-beam = ④ dBc |  |  | 0 dBc | 0 dBc |  |
|  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  | 0 dB | N/A |  |
|  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPCSSI_leakage due to inter-sector interference (Note 1) |  |  | -94 to -79 dBm (Varies depending on scheduled beam directions) | -92 dBm to -72 dBm |  |
|  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  | -49 to -34 dBm (Varies depending on scheduled beam directions) | -44 dBm to -24 dBm |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαCSSI-frequency, RX= ⑥ dBc |  | 0 dBc | 0 dBc |  |
|  |  |  | Frequency isolation techniques |  | Filtering prior to the LNA would imply the need for the BS hardware to be specifically tuned to the SBFD carrier and no multi-carrier possibilities. Filter would be bulky to integrate into an AAS. Insertion loss would degrade sensitivity. Filtering in-between LNA stages could increase linearity, but still a large number of filters to incorporate multi-carrier configurations would not be feasible. Loss of integration would cause increases in size, energy etc. |  |  | Digital filtering, FFT frequency selectivity |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -13 dBm (“Optimistic” value) | Included in the NF model |  |
|  |  |  | Rx IM3 contribution (dBm) | -101 dBm to -74 dBm |  |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | (IM3 due to inter-sector interference only, assuming the presence of both self- and inter-sector interference in the receiver) |  |  |
|  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to co-site inter-sector co-channel interference only dBPCSSI_Selectivity(Note 1, 2) |  |  | -101 dBm to -74 dBm (Additional interference due to inter-sector interference only) (Depending on beam direction) | -90 dBm to -70 dBm |  |
|  | RX Beam nulling /isolation in RX sub-band dBαCSSI-beam = ⑨ dBc |  |  | 0 dB | 0 dB |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | 0 dB | N/A |  |
|  | Digital processing interference supression capability |  |  | No digital cancellation between sectors. | 0 dB |  |
| Total interference in RX SB (dBm) (Note 2) |  |  |  | -94 To -74 dBm (Additional interference due to inter-sector interference only) (Depending on beam direction) | -90 dBm to -70 dBm (Self-interference not considered here) |  |
| Noise floor ⑩dBm |  |  |  | -96 dBm/CBW | -96 dBm/ 20 MHz |  |
| Calculated Desensitization (dB) |  |  |  | 4 to 22 dB (Depending on beam direction. Additional degradation due to inter-sector interference only) | 6 to 26 dB |  |
| SBFD configuration |  |  |  | 40-20-40 MHz | 40 – 20 – 40 MHz |  |
| Guardband assumption (if exist) |  |  |  | 5 PRB. | 5 PRB |  |
| bandwidth over which suppression is achieved |  |  |  | >300 MHz |  |  |
| Others |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. Note 4: The abbreviation CSSI refers to co-site co-channel inter-sector interference in this table |  |  |  |  |  |  |

#### 9.3.2.2	Feasibility study on co-channel inter-subband co-site inter-sector interference  analysis

##### 9.3.2.2.1	Nokia

Medium range base stations are commonly deployed with omnidirectional antennas, but some deployments use directional antennas also with the target to boost coverage or capacity. In case of 3-sector site deployment with directional antennas, the techniques described in Section 9.2.2.2.4 apply here as well, although it is important to note that it may be more difficult to add large horizontal separation between sectors if the site footprint is small.

In a case a 3-sector site deployment are considered, our analysis is presented in table 9.3.2.1-1 above. We considered the same spatial isolation as considered in the urban macro cell case. As our input to the table shows, the total interference in RX subband is between -90 dBm and -70 dBm, which is already above the noise floor and would cause desensitization of the receiver. It is also important to note that this analysis does not consider the self-interference, so in reality the desensitization would be greater.

##### 9.3.2.2.2	Ericsson

Table 9.3.2.1-1 presents an analysis of the inter-sector interference effects for medium range FR1 BS. The table considers the “optimistic” receiver considered in section 9.3.2.1.2; i.e., a receiver that is more capable than in a current BS. If the “realistic” receiver would be considered, then the indicated desensitizations would be greater.

The level of inter-sector interference only varies depending on the beam direction from around 4dB to 22dB. The total desensitization would include self-interference and would be around 1dB greater. There may be some potential to increase inter-sector isolation using TX beam nulling (not shown in the table), however clearly mitigating inter-sector interference will be a challenge also for MR BS.

Inter-sector isolation

Isolation between sectors has been simulated using electromagnetic simulations in R4-2301885 with an assumption of 400mm sector separation. The isolation varies to some degree with separation, but not to an extent that would change the overall results. For most practical site deployments, addition of materials between sectors is not likely to be feasible (and may reduce network performance).

The insolation between sectors is highly dependent on the beam direction. Although an “average” isolation can be given, this would mask the fact that for certain beam directions isolation is good and for others it is not good.

Beam nulling

There may be some potential for beam nulling to mitigate interference between sectors. To achieve complete isolation between sectors, more than 20dB beam nulling would be needed.

Receiver filtering

Analogue filtering in the receiver is not assumed for reasons described in section 9.3.2.1.2.

#### 9.3.2.3 Conclusion

Based on the co-channel inter-sub-band co-site inter-sector interference analysis in Section 9.3.2, two companies provided technical inputs, concluding that the total interference caused by two co-site sectors would be above the noise floor, causing a desensitization of the receiver.

### 9.3.3	Co-channel inter-sub-band inter-site interference analysis

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

On the feasibility and how to model inter-site gNB-gNB CLI modelling considering unwanted emission and receiver selectivity, RAN4 agree that

- The same transmitter leakage and receiver impairment model as used for investigating gNB self-interference, but antenna isolation is replaced with inter-site isolation.

  - TX leakage baseline: gNB ACLR

  - Receiver impairment can be studied with gNB ACS as baseline for system level simulation and feasibility study, and further study on the possibility of improved receiver impairment performance compared to gNB ACS shall not be precluded in future RAN4 works.

### 9.3.4	Summary

Based on RAN4 feasibility study on FR1 medium range BS, specifically the analysis on self-interference, co-site inter-sector co-channel inter-subband interference and inter-site inter-subband interference, RAN4 concluded that:

- For self-interference analysis, the implementation feasibility of controlling the residual interference to meet the 1dB receiver desensitization target depends on the implementation aspects mentioned in clause 9.3.1. Based on the different assumptions and/or technique adoption for the above-mentioned implementation aspects, based on 5 companies’ technical inputs, companies have come to the following conclusions:

  - 1dB receiver desensitization target is achievable by self-interference cancellation capability according to 4 companies.

  - 1dB receiver desensitization target is not achievable or is challenging based on existing technology and technology roadmaps that are viewed as viable in the current time or foreseeable future according to 2 companies

- For co-site inter-sector co-channel inter-subband interference, companies concluded that the implementation feasibility of controlling the co-site inter-sector co-channel inter-subband interference to meet the target (i.e., being less than certain level of receiver desensitization) is not feasible when considering the interference caused by two co-site sectors.

- For inter-site co-channel inter-subband interference, since the feasibility is deployment-dependent, RAN4 has provided the inter-site gNB-gNB CLI modelling used for coexistence study by considering unwanted emission and receiver selectivity modelling.

## <<End of Change for TR 38.858>>