3GPP TSG-RAN WG4 Meeting # 109	R4-2320056

Chicago, US, November 13 – November 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 38.858 Section 11: Adjacent channel co-existence evaluation results

Agenda item:	8.19.2.1

Document for:	Approval

Introduction

In 2022, a new study item work on evolution of duplex operation [1] was started, with the target to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation.

The detailed objectives specified in [1] are as follows:

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution we discuss the co-existence conclusions agreed on RAN4#108bis and propose changes to the conclusions in a TP.

Discussion

The latest endorsed draft of the TP on adjacent channel co-existence simulations of subband non-overlapping full duplex (SBFD) is found in [2]. Here, we propose a TP for Section 11 on TR 38.858, with the following changes:

- Add a clarification for the criteria used to derive the co-existence conclusions.

- Add a clarification Table 11.3.2-1 for the effect of grid shift in the UMa-to-UMi scenario.

- Removing a sentence in Table 11.3.4-1 that indicates that the UE-to-UE distance decreases with the grid shift. This is not correct, since the grid shift only affects the BS-to-BS distance.

- Editorial changes:

  - Align terminology between gNB and BS.

  - Improve section readability.

  - Correction of typos.

# 3 References

- RP-223041, Revised SID: Study on evolution of NR duplex operation, CMCC, RAN #98-e, Electronic Meeting, December 12-16, 2022.

- R4-2316975, Draft TP to TR 38.858 on Chapter 11 - Adjacent channel co-existence evaluation results, RAN4#108 bis, Samsung

<Start of TP to TR 38.858>

# 11	Adjacent channel co-existence evaluation results.

## 11.1	Introduction

Editor's note: This section will capture adjacent channel co-existence simulation scenarios and cases. Besides, some brief introduction of what RAN4 have done in R18 for SBFD.

The adjacent channel co-existence studies were performed to the deployment scenarios described in Table 11-1 below. The co-existence cases are described in the Table 11-2 below, and they were performed for each scenarios listed in Table 11-1. The detailed assumptions associated with these scenarios and cases can be found in Annex E.

Table 11.1-1: Adjacent channel co-existence scenarios

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro |
| 2 | FR1 | Urban Hotspot | Urban Hotspot |
| 3 | FR1 | Indoor | Indoor |
| 4 | FR1 | Urban Macro | Micro |
| 5 | FR1 | Micro | Micro |
| 6 | FR2-1 | Urban Macro | Urban Macro |
| 71 | FR2-1 | Urban Hotspot | Urban Hotspot |
| 8 | FR2-1 | Urban Dense | Urban Dense |
| 9 | FR2-1 | Indoor | Indoor |
| Note 1: This scenario has been down-selected. |  |  |  |

Table 11.1-2: Adjacent channel co-existence cases

| Case | Aggressor | Victim | Slot allocation Aggressor Victim |
| --- | --- | --- | --- |
| 1 | SBFD | TDD DL |  |
| 2 | SBFD | TDD UL |  |
| 3 | TDD DL | SBFD |  |
| 4 | TDD UL | SBFD |  |
| Note: Case 3 and Case 4 are down-selected for Scenario 4. |  |  |  |

The Urban Hotspot reuses most parameter assumptions as Urban Macro, except that Urban Macro adopts random dropping method for UE while Urban Hotspot adopts cluster-based dropping method for UE. Other differences are described in Annex E, in Table E.2.1-1, Table E.2.1-2, and Table E.2.1-3.

The co-existence evaluation captures cases where TDD and SBFD are both victim and aggressor networks. This to evaluate impact on legacy TDD networks if SBFD is introduced in an adjacent channel as well as to understand the impact of legacy TDD network on SBFD network, as described in Annex E, Table E.1-2. It is worth noting that RAN4 has only considered the case of {D, U} as an SBFD configuration as it is comparable to the {D, U, D} SBFD configuration in terms of performance (based on the agreed RAN4 models and parameters related to leakage).

Additionally, it should be noted that the RAN4 co-existence studies have special assumptions due to the adopted simulation methodologies such as:

- Power control scheme is only used to compensate path loss. That’s the reason why final SINR for UL is less than assumed target SINR. But commercial UE UL SINR could meet target SINR value according to the power control scheme in TS 38.213.

- It is assumed that all the slots configurations are the same with the time-invariant ACLR modelling assumption. Compared with the average throughput over all time slots with different configuration, this is the worst case with largest degradation value.

Moreover, in the following sections, all the throughput degradation data are given in a range, where these data are defined as follows:

- The {positive number} means the co-existence study shows throughput loss.

- The {negative number} means the co-existence study shows throughput gain.

## The {n/a} means the co-existence study finds the performance basis have no throughput, thus throughput degradation percentage cannot be mathematically calculated from such basis.

## 11.2	Summary of all simulation results

Editor's note: This section will capture the excel files and word file that moderator summarized based on all companies’ input with some description of these files.

[

One example:

We list three files.

First excel file is the collection of all simulation results using the template format that companies use to provide simulation results.

Word file is the summary of all simulation results categorized in terms of scenario, case, victim and parameters values of four simulation assumptions, i.e. antenna configuration 1 or 2, gNB Tx power value, grid shift value and enhanced NF or not. In addition to, we list the min value, max value and median for relative ACIR, ACIR enhanced by 2, 4, 6, and 8dB. We also list the numbers of companies that show simulation results for each scenario, each case and each victim band.

Second excel file is the excel file of word file which is much straightforward compared with the word.

[placeholder for clarifying text on ACIR enhancement definition for simulation and results interpretation]

]

## 11.3	Conclusion

This section captures the co-existence conclusions per deployment scenario and case according to the adjacent channel co-existence simulation results included in Section 11.2. The conclusions are derived using the criteria described in Section 11.1 and based on the co-existence results with ACIR baseline.

### 11.3.1 Case 1: aggressor SBFD DU victim NR TDD DL

Case 1 considers legacy TDD in DL slot as a victim while SBFD is operating in the adjacent channel for both FR1 and FR2-1. The co-existence conclusions per scenario are summarized in Table 11.3.1-1.

Table 11.3.1-1: Case 1 co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 and FR2-1 | No DL throughput degradation on the victim legacy TDD DL network for both average throughput and cell edge throughput is observed for different BS Tx power (46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid shifts (5% to 100%), and different SBFD BS antenna configuration. |
| Urban Hotspot -> Urban Hotspot | FR1 | DL throughput degradation is observed only at cell edge due to inter-UE CLI for different grid shifts (5% to 100%) and BS Tx power (49 dBm to 53 dBm). |
| Urban Macro -> Urban Micro | FR1 | No DL throughput degradation for both average throughput and cell edge throughput is observed. |
| Indoor -> Indoor | FR1 and FR2-1 |  |
| Urban Micro/Dense -> Urban Micro/Dense | FR1 and FR2-1 |  |

### 11.3.2 Case 2: aggressor SBFD DU victim NR TDD UL

Case 2 considers legacy TDD in UL slot as a victim while SBFD is operating in the UL slot in the adjacent channel for both FR1 and FR2-1. The co-existence conclusions per scenario are summarized in Table 11.3.2-1.

Table 11.3.2-1: Case 2 co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 | The TDD UL has significant throughput degradation for different SBFD BS antenna configuration and different BS Tx power. - The cell edge throughput degradation is worse than the average throughput degradation. - The throughput degradation is due to the inter-BS ACI introduced by SBFD, which increases as grid shift (BS-to-BS distance) decreases in Scenarios 1, 2 and 5. In Scenario 4 (UMa-to-UMi), the throughput degradation increases and then decreases as the grid shift reduces. This is due to the relative distance and elevation angle between UMa and UMi base stations. - The throughput degradation increases with SBFD BS Tx power. - SBFD BS antenna configuration slightly impacts the throughput degradation. |
| Urban Hotspot -> Urban Hotspot |  |  |
| Urban Macro ->Urban Micro |  |  |
| Urban Micro/Dense -> Urban Micro/Dense |  |  |
| Indoor -> Indoor | FR1 and FR2-1 | No TDD UL throughput degradation is observed. |
| Urban Macro -> Urban Macro | FR2-1 | TDD UL throughput degradation is observed at cell edge, no strong degradation is observed for the average throughput. |
| Urban Dense -> Urban Dense |  |  |

### 11.3.3 Case 3: aggressor NR TDD DL victim SBFD DU

Case 3 considers SBFD as a victim while NR TDD is operating DL in the adjacent channel for both FR1 and FR2-1. The co-existence conclusions per scenario are summarized in Table 11.3.3-1 and Table 11.3.3-2 for SBFD DL and SBFD UL, respectively.

Table 11.3.3-1: Case 3 SBFD DL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 and FR2-1 | No observed throughput degradation on the SBFD DL for both average throughput and cell edge throughput for different BS Tx power, ranging from 46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1, grid shifts (5% to 100%), and different SBFD BS antenna configuration. |
| Urban Hotspot -> Urban Hotspot (N/A for FR2-1) |  |  |
| Indoor -> Indoor |  |  |
| Urban Micro/Dense -> Urban Micro/Dense |  |  |

Table 11.3.3-2: Case 3 SBFD UL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 | Under baseline assumptions, observed SBFD UL throughput degradation is observed only for cell edge throughput and [no degradation is observed for average throughput]. With higher BS Tx power and lower grid shifts, the degradation is increased for cell edge throughput and [average throughput]. |
|  | FR2-1 | Under baseline assumptions, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher BS Tx power and lower grid shifts. |
| Urban Hotspot -> Urban Hotspot | FR1 | Under baseline assumptions, observed throughput degradation is observed at cell edge throughput and average throughput. With higher BS Tx power and lower grid shifts, the degradation is increased. |
| Indoor -> Indoor | FR1 and FR2-1 | No SBFD UL throughput degradation for both average throughput and cell edge throughput is observed. |
| Urban Micro/Dense -> Urban Micro/Dense | FR1 | Under FR1 Urban micro 38dBm Tx power assumption, no degradation on the SBFD UL is observed for both cell edge throughput and average throughput. Throughput loss is observed with higher gNB BS power (46dBm) and lower grid shifts. |
|  | FR2-1 | Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput and no degradation is observed for average throughput. |

### 11.3.4 Case 4: aggressor NR TDD UL victim SBFD DU

Case 4 considers SBFD as a victim while NR TDD is operating UL in the adjacent channel for both FR1 and FR2-1. The co-existence conclusions per scenario are summarized in Table 11.3.4-1 and Table 11.3.4-2 for SBFD DL and SBFD UL, respectively.

Table 11.3.4-1: Case 4 SBFD DL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 and FR2-1 | No observed throughput degradation on the SBFD DL for both average throughput and cell edge throughput for different gNB Tx power, ranging from 46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1, grid shifts (5% to 100%), and SBFD BS antenna configuration (single and double panels for SBFD operation). |
| Indoor -> Indoor |  |  |
| Urban Micro/Dense -> Urban Micro/Dense |  |  |
| Urban Hotspot -> Urban Hotspot | FR1 | Some companies’ results show DL throughput degradation is observed only for cell edge throughput due to inter-UE CLI for different grid shifts (5% to 100%), BS Tx power (46 dBm to 53 dBm) and for all BS antenna configurations. However more companies show that there is no observed degradation for cell edge throughput and cell average throughput for 100% grid shift, 49 dBm BS Tx power and antenna configuration 2. |

Table 11.3.4-2: Case 4 SBFD UL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro | FR1 and FR2-1 | No observed throughput degradation on the SBFD UL for both average throughput and cell edge throughput for different BS Tx power, ranging from 46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1, grid shifts (5% to 100%), and SBFD BS antenna configuration (single and double panels for SBFD operation). |
| Indoor -> Indoor |  |  |
| Urban Micro/Dense -> Urban Micro/Dense |  |  |
| Urban Hotspot -> Urban Hotspot (N/A for FR2-1) |  |  |

### 11.3.5 General remarks on co-existence findings

[For the above cases where no throughput degradation has been observed, no additional coexistence measures are required for SBFD deployment. However, for other cases where throughput degradation has been observed, interference mitigation techniques might need to be considered.]

<End of TP to TR 38.858>