3GPP TSG-RAN WG4 Meeting #109	R4-2320085

Chicago, USA, 13th November – 17th November 2023

Source:	ZTE Corporation

Title:	TP for TR 38.869_Updates for guard band definition

Agenda Item:	8.20.2

Document for:	Approval

- Introduction

In this TP, we give some updates for the WUS guard band definition based on [1]

- Reference

- [1] R4-2317775, TP to TR 38.869 LP-WUS receiver architectures, vivo, etc.

- Text Proposal

----- Start of TP -----

    - 7.1.2	RAN4 studies of the receiver

Editor’s note: RAN4 RF related conclusions

      - 7.1.2.1	General

This Clause captures the study outcome of LP-WUR RF impacts. RAN4 focus on FR1 frequency ranges as first priority, 2.6GHz is selected as an example band for evaluation purpose.

For evaluation purpose, RAN4 define a new term named as guard RB for LP-WUS, which is Granularity of RB. Meanwhile, the traditional guardband for NR channel bandwidth defined in Clause 5.3, TS 38.101-1 is unchanged. There are two types of guard RBs for LP-WUS:

-	Depending on the LP-WUS location, the LP-WUS guard RBs could be the number of RBs between LP-WUS RB edge and the nearest RB edge of other NR signals, or the number RBs offset from LP-WUS carrier bandwidth edge to the nearest RB edge of NR maximum transmission configuration bandwidth as in spec TS 38.101-1

RAN4 agrees there is no need to restrict symmetric guard RBs for interference rejection of WUS.

- RAN4 agrees RBs within the number RB offset from LP-WUS carrier bandwidth edge to nearest edge of NR maximum transmission configuration bandwidth may not be blanked.

RAN4 suggests overall bandwidth of the wake-up signal in the NR channel i.e., desired signal along with all the required guard RBs shall fit in the maximum transmission bandwidth configuration as defined in Table 5.3.2-1 of TS 38.101-1.

Figure 7.1.2.1-1: Definition of the guardband of NR channel and guard RB for LP-WUS

----- End of TP -----