3GPP TSG-RAN WG4 Meeting #109	R4-2320100

Chicago, USA, 13th November – 17th November 2023

Source:	ZTE Corporation

Title:	TP for TR38.718-03-01_3DL_xUL CA_n34A-n39A-n41A/C

Agenda Item:	7.11.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to update the MSD for  3DL/xUL n34A-n39A-n41Ain TR38.718-03-01.

- Reference

- [1] TR38.718-03-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for3 bands DL with x bands UL (x=1,2),v0.8.0

- Text Proposal

----- Start of TP -----

  - 5.x			n34A-n39A-n41A

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| n34-n39-n41 | n34, n39, n41 |

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY+nZ

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n34A-n39A-n41A | CA_n34A-n39A CA_n34A-n41A CA_n39A-n41A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n39A-n41C | CA_n41C CA_n34A-n39A CA_n34A-n41A CA_n39A-n41A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |

      - 5.x.1.3	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c values are defined in the following:

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| n34-n39-n41 | 0.3 | 0.5 | 0.5 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-1: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| n34-n39-n41 | 0.3 | 0.3 | 0.2 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

    - 5.x.2	Specific for 2 bands UL CA

      - 5.x.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n34A-n39A, CA_n34A-n41A and CA_n39A-n41A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

No IMD issues caused by n39+n41 fall into the its own band n34 Rx;

No IMD issues caused by n34+n41 fall into the its own band n39 Rx;

No IMD issues caused by n34+n39 fall into the its own band n41 Rx.

      - 5.x.2.2	REFSENS requirements

Based on co-existence studies, there are no additional MSD needed to be defined

----- End of TP -----