3GPP TSG-RAN WG4 Meeting #109	R4-2320103

Chicago, USA, 13th November – 17th November 2023

Source:	ZTE Corporation

Title:	TP for TR38.718-03-01_3DL_xUL update MSD for CA_n8A-n39A-n79A

Agenda Item:	7.11.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to update the MSD for  3DL/2UL CA_n8A-n39A-n79A in TR38.718-03-01.

- Reference

- [1] TR38.718-03-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for3 bands DL with x bands UL (x=1,2),v0.8.0

- Text Proposal

----- Start of TP -----

  - 5.51			n8A-n39A-n79A

    - 5.51.1	Common for 1 band UL and 2 bands UL CA

      - 5.51.1.1	Operating bands for CA

Table 5.51.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| n8A-n39A-n79A | n8, n39, n79 |

      - 5.51.1.2	Channel bandwidths per operating band for CA

Table 5.51.1.2-1: Supported bandwidths per CA band combination of band nX+nY+nZ

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n39A-n79A | CA_n8A-n39A CA_n8A-n79A CA_n39A-n79A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

      - 5.51.1.3	∆TIB,c and ∆RIB,c values

For CA_n8-n39-n79, the TIB,c and RIB,c values have already been defined in the spec.

    - 5.51.2	Specific for 2 bands UL CA

      - 5.51.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n8A-n39A, CA_n8A-n79A and CA_n39A-n79A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

IMD3 and IMD4 issue caused by n8+n39 fall into the its own band n79 Rx;

IMD3 and IMD4 issue caused by n39+n79 fall into the its own band n8 Rx;

IMD4 issue caused by n8+n79 fall into the its own band n39 Rx.

      - 5.51.2.2	REFSENS requirements

Based on co-existence studies additional MSD is needed to be defined, shown in table 5.51.2.2-1., where some MSD requirements are reused form DC_8_n39-n79.

Table 5.51.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n8-n39-n79 | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |
|  | n79 | N/A | 40 | N/A | 4680 | 15.9 | IMD3 |
|  | n8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |
|  | n79 | N/A | 40 | N/A | 4560 | 12.1 | IMD4 |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n39 | N/A | 10 | N/A | 1907.5 | 13.8 | IMD4 |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | N/A |
|  | n8 | N/A | 5 | N/A | 940 | 15.1 | IMD3 |
|  | n39 | 1900 | 10 | 50 | 1900 | N/A | N/A |
|  |  | n79 |  | 4740 | 40 |  | 216 |  | 4740 |  |  | N/A |  | N/A |  |
|  |  | n8 |  | N/A | 5 |  | N/A |  | 940 |  |  | 7.1 |  | IMD4 |  |
|  |  | n39 |  | 1900 | 10 |  | 50 |  | 1900 |  |  | N/A |  | N/A |  |
|  | n79 | 4750 | 40 | 216 | 4750 | N/A | N/A |

----- End of TP -----