3GPP TSG-RAN WG4 Meeting #109 	R4- 2320116

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion paper on LS to RAN5

Agenda item:	8.31.1

Document for:	Discussion

Introduction

During the RAN4#108 following agreement was made regarding REFSENS of eRedcap UE ( BW3/PR3 + PR1) for wider channel BW. In this document we provide a draft LS to RAN5 in order to clarify the testability issue.

Agreement: - 25 contiguous RB placed in middle of channel BW both in UL and DL for both HD-FDD and FDD - where both UL and DL allocations are following the RedCap UE 5MHz REFSENS configurations and located at the center of the channel BW. - UL RB size follow the 5MHz UL configuration table - Add the note or send LS to RAN5 to clarification the applicability for testing. - The 5MHz requirements are applicable for all the channel bandwidths - For some bands which has less duplex distance, the additional relaxation factor is not excluded

Discussion

In case of REFSENS testing, for a bandwidth and peak rate limited UE, 5 MHz requirements apply for the larger channel bandwidths too. The UL and DL RB location stays the same irrespective of the channel bandwidth. This lead to a concern in RAN4 that the UE might be tested with the same configuration repeatedly. RAN4 had an opinion that it is not necessary to repeat verification when same requirements apply for multiple channel bandwidths. However, as RAN5 defines the test cases, it is up to them to decide whether to repeat these cases or not. With the attached draft LS we would like to ask RAN5 their preference, whether RAN4 opinion on not repeating verification should be noted down in the technical specification or is it not required.

Certain requirements stay the same irrespective of channel bandwidths.

RAN5 decides the test cases and can inform their preference whether RAN4 opinion needs to be noted in the TS or not.

Agree to send the attached LS to RAN5.

Conclusion

This paper has provided a discussion for a draft LS to be sent to RAN5. Based on the discussion, the following Observations and Proposals were made:

- Certain requirements stay the same irrespective of channel bandwidths.

RAN5 decides the test cases and can inform their preference whether RAN4 opinion needs to be noted in the TS or not.

- Agree to send the attached LS to RAN5.

- References

- R4-2314744, WF on eRedCap UE RF requirements, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, August 21st – August 25th, 2023.

- Annex LS on eRedCap testability

3GPP TSG-RAN WG4 Meeting #109 R4-23xxxxx Chicago, US, November 13 – 17, 2023 Title: LS on eRedCap testability Response to: Release: Release 18 Work Item: NR_redcap_enh Source: RAN4 To: RAN5 Cc: Contact Person: Name: Alok Sethi Tel. Number: E-mail Address: Alok.Sethi (at) nokia (dot) com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: - 1. Overall Description: RAN4 has been working on finalizing the requirements for eRedCap. In case of REFSENS testing, in some cases 5 MHz requirements apply for larger channel bandwidths. There is a concern in RAN4 that the UE might be tested with the same configuration repeatedly. RAN4 opinion is that it is not necessary to repeat verification when same requirements apply for multiple channel bandwidths. 2. Actions: To RAN5: RAN4 have discussed whether this recommendation to not test repeatedly needs to be noted in the RAN4 TS or it would be enough to request RAN5 to take this into account when defining their test cases. RAN4 respectfully request RAN5 to inform whether they have any preference. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting#110 Feb 26 – Mar 01, 2024 Athens, Greece RAN WG4 Meeting #110bis Apr 15 – Apr 19, 2024 China (TBC)