3GPP TSG-RAN WG4 Meeting # 109		R4-2320138

# Chicago, US, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7260 | rev | - | Current version: | 18.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PHR reporting requirements for NB-IoT over NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Perf |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 agreed to define separate PHR reporting tables for GSO and NGSO respectively. But the reporting values of GSO are in square brackets which need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: PHR reporting values for GSO for NB-IoT UEs not supporting enhanced PHR reporting are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Uncertainty in PHR reporting values for NB-IoT |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.23A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 9.1.23A	Power Headroom for UE Category NB1 for Satellite Access

The requirements in this clause shall apply for power headroom for UE Category NB1 as defined in [31].

The power headroom provides the serving eNB with information about the differences between the UE configured maximum output power (PCMAX,) defined in TS 36.102 [60] and the estimated power for UL-NSCH transmission of the serving cell [3].

Table 9.1.23A-1: The applicability of power headroom report mapping requirements for different power class UE

| Power class | Power headroom report mapping |
| --- | --- |
| PC3 and PC5 | As defined in section 9.1.23A.3 |
| PC6 | As defined in section 9.1.23A.4 |

#### 9.1.23A.1	Period

The reported power headroom shall be estimated over 1 slot of NPUSCH transmissions.

#### 9.1.23A.2	Reporting Delay

The power headroom reporting delay is defined as the time between the beginning of the power headroom reference period and the time when the UE starts transmitting the power headroom over the radio interface. The reporting delay of the power headroom shall be 0 ms, which is applicable for all configured triggering mechanisms for power headroom reporting.

#### 9.1.23A.3	Report Mapping for UE Category NB1 for Satellite Access

For UE category NB1 in GSO when the enhanced coverage level 0 is selected during the random access procedure [17], the report mapping is defined in Table 9.1.23A.3-1 for UEs not supporting enhanced PHR.

Table 9.1.23A.3-1: Power headroom report mapping for UE category NB1 UEs in GSO not supporting enhanced PHR [31] when the enhanced coverage level 0 is selected during random access procedure [17]

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

For UE category NB1 in GSO when enhanced coverage level other than 0 is selected during the random access procedure [17], the report mapping is defined in Table 9.1.23A.3-2 for UEs not supporting enhanced PHR.

Table 9.1.23A.3-2: Power headroom report mapping for UE category NB1 UEs in GSO not supporting enhanced PHR [31] when the enhanced coverage level other than 0 is selected during random access procedure [17]

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -40 |
| POWER_HEADROOM_1 | -40 PH -30 |
| POWER_HEADROOM_2 | -30 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

For UE category NB1 in GSO, the the report mapping defined in clause 9.1.23.3 applies for UEs supporting enhanced PHR.

For UE category NB1 in NGSO, the report mapping defined in clause 9.1.23.3 applies.

##### 9.1.23A.3.1	Void

##### 9.1.23A.3.2	Void

#### 9.1.23A.4	Report Mapping for UE Category NB1 for UE Power Class 6 for Satellite Access

For UE category NB1 in GSO when the enhanced coverage level 0 is selected during the random access procedure [17] for UE power class 6 [5], the report mapping is defined in Table 9.1.23A.4-1 for UEs not supporting enhanced PHR.

Table 9.1.23A.4-1: Power headroom report mapping for UE category NB1 UEs in GSO not supporting enhanced PHR when the enhanced coverage level 0 is selected during random access procedure [17] for UE PC6

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

For UE category NB1 in GSO when enhanced coverage level other than 0 is selected during the random access procedure [17] for UE power class 6 [5], the report mapping is defined in Table 9.1.23A.4-2 for UEs not supporting enhanced PHR.

Table 9.1.23A.4-2: Power headroom report mapping for UE category NB1 in GSO not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure [17] for UE PC6

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -40 |
| POWER_HEADROOM_1 | -40 PH -30 |
| POWER_HEADROOM_2 | -30 PH 0 |
| POWER_HEADROOM_3 | PH ≥ 0 |

For UE category NB1 in GSO for UE power class 6 [5], the the report mapping defined in clause 9.1.23.4 applies for UEs supporting enhanced PHR.

For UE category NB1 in NGSO for UE power class 6 [5], the report mapping defined in clause 9.1.23.4 applies.

<End of Change 1>