3GPP TSG-RAN WG4 Meeting # 109	R4-2320192

# Chicago, U.S, Nov.13rd – Nov.17th, 2023

Title: 	Discussions on UE performance for Non-collocated CA

Source: 	Huawei, HiSilicon

Agenda item:	5.11.4

Document for:	Discussion

Background

In last meeting, a WF on NonCol_intraB_ENDC_NR_CA Demod [1] was agreed. In this contribution we provide our views on remaining open issues.

Discussions

The main controversial issue is how to choose the MCS for each CC. The simulation assumptions and candidate options are shown in Table 2-1 and Table 2-2:

Table 2-1: Simulation assumptions

|  | Type 2 UE NR-CA demodulation requirements |
| --- | --- |
| Received power difference | 25 dB |
| Received time difference | 33 µs |
| Channel model | AWGN (TS38.101-4 B.1.1) |
| Maximum number of HARQ transmission | 1 |
| Signal/Noise setting | Set both Es and Noc to set SNR test points |
| Throughput measurement procedure | Measure both carriers |
| Antenna configuration | 2x2 for both carriers |
| MCS table and MCS index | FFS |
| Tx EVM | 6% for stronger CC (QPSK) 3% for weaker CC (256QAM) |
| Test metric | 70% of the maximum throughput |

Table 2-2: Candidate options for MCS pairs selection

|  | Rank | MCS table | MCS index |
| --- | --- | --- | --- |
| Carrier with lower power | Rank 1 | MCS table 1 | Option 1: MCS4 (QPSK, 0.30) Option 2: MCS5 (QPSK, 0.37) |
| Carrier with higher power | Rank 2 | MCS table 2 | Option 1: MCS22 (256QAM, 0.74) Option 2: MCS23 (256QAM, 0.78) Option 3: MCS24 (256QAM, 0.82) Option 4: MCS25 (256QAM, 0.86) |

Simulation results are provided in Table 2-3 (Weaker CC) and Table 2-4(Stronger CC).

Table 2-3: Simulation results for weaker CC

| MCS | SNR(dB)@70% of max TP (Ideal) | SNR(dB)@70% of max TP (Impairment) |
| --- | --- | --- |
| 4 | -4.3 | -3.3 |
| 5 | -3.4 | -2.4 |

Table 2-4: Simulation results for stronger CC

| MCS | SNR(dB)@70% of max TP (Ideal) | SNR(dB)@70% of max TP (Impairment) |
| --- | --- | --- |
| 22 | 20.4 | 21.4 |
| 23 | 22.1 | 23.1 |
| 24 | 22.6 | 23.6 |
| 25 | 24.4 | 25.4 |

Based on the simulation results, we propose the following MCS pairs.

Option1: MCS4 (Weaker CC), MCS22 (Stronger CC)

Option2: MCS5 (Weaker CC), MCS22 (Stronger CC)

- Proposal 1:  Choose one MCS pair from following candidate options:

Option1: MCS4 (Weaker CC), MCS22 (Stronger CC)

Option2: MCS5 (Weaker CC), MCS22 (Stronger CC)

Test setup and applicability rules

Following agreements are reached by RAN2:

Agreements

- RAN2 agree to introduce a new RRC signaling to indicate capability restriction is type 1 or type 2 only for Rel-18 UE and not to introduce for Rel-17/16/15 UE

- Detailed statements capturing detailed RAN4 spec references in RAN2 specs without terminologies “Type1” and “Type2”.

- For EN-DC - Some Rel-18 UE support the new BS signaling and some Rel-18 UE do not support.  A new UE capability will be introduced for this

- Introduce a new UE capability which indicates the support of the new BS signaling per UE. This UE capability is only applicable to the UE indicating “interBandMRDC-WithOverlapDL-Bands-r16”.   The new capability is included in MRDC container

- Introduce a new Per BC UE capability to indicate the supporting TDD-TDD intra-band Non-Collocated NR-CA.

-	UE supporting the new UE capability supports both Type 1 and Type 2 capability requirements, while the UE not supporting the new UE capability supports Type 1 capability requirement only.

- Introduce two separate indications under IE ”CellGroupConfig” in TS38.331, one is for EN-DC operation and the other is for NR-CA

RAN2 has agreed to define a new capability to indicate supporting TDD-TDD intra-band Non-Collocated NR-CA, which means RAN4 requirements should be only applicable for UE supporting such deployment. Meanwhile, it’s agreed to introduce a new RRC signaling to facilitate UE to switch the architecture between type1 and type2. Therefore, during the demod test, such new RRC signalling should be configured to guarantee the tested UE is operating on type2 architecture.

Meanwhile, RAN4 has agreed to introduce 256QAM for stronger CC, which is not supported by some UEs. Therefore related applicability rules should be defined: The requirements apply only in case modulation order configured for each CC doesn’t exceed UE per CC capability on supported maximum modulation order

- Proposal 2:  Following applicability rules and test setup should be considered:

The requirements are only applicable for UE supporting TDD-TDD intra-band Non-Collocated NR-CA [intraBandNonColocatedCA-r18].

The requirements apply only in case modulation order configured for each CC doesn’t exceed UE per CC capability on supported maximum modulation order

During the demod test, the new RRC signaling to be introduced by RAN2 should be configured to guarantee the tested UE is operating on type2 architecture.

Conclusion

In this paper we provide our views on UE performance for Non-collocated CA. The proposal are:

- Proposal 1:  Choose one MCS pair from following candidate options:

Option1: MCS4 (Weaker CC), MCS22 (Stronger CC)

Option2: MCS5 (Weaker CC), MCS22 (Stronger CC)

- Proposal 2:  Following applicability rules and test setup should be considered:

The requirements are only applicable for UE supporting TDD-TDD intra-band Non-Collocated NR-CA [intraBandNonColocatedCA-r18].

The requirements apply only in case modulation order configured for each CC doesn’t exceed UE per CC capability on supported maximum modulation order

During the demod test, the new RRC signaling to be introduced by RAN2 should be configured to guarantee the tested UE is operating on type2 architecture.

Reference

[1]    R4-2313876 WF for NonCol_intraB_ENDC_NR_CA_Demod. Ericsson

[2]    Chair notes for RAN2#123bis