3GPP TSG-RAN WG4 Meeting # 109	R4-2320193

# Chicago, U.S, Nov.13rd – Nov.17th, 2023

Title: 	Simulation results on UE performance for Non-collocated CA

Source: 	Huawei, HiSilicon

Agenda item:	5.11.4

Document for:	Discussion

Background

In last meeting, a WF on NonCol_intraB_ENDC_NR_CA Demod [1] was agreed. In this contribution we provide our simulation results.

Simulation results

Table 2-1: Simulation assumptions

|  | Type 2 UE NR-CA demodulation requirements |
| --- | --- |
| Received power difference | 25 dB |
| Received time difference | 33 µs |
| Channel model | AWGN (TS38.101-4 B.1.1) |
| Maximum number of HARQ transmission | 1 |
| Signal/Noise setting | Set both Es and Noc to set SNR test points |
| Throughput measurement procedure | Measure both carriers |
| Antenna configuration | 2x2 for both carriers |
| MCS table and MCS index | FFS |
| Tx EVM | 6% for stronger CC (QPSK) 3% for weaker CC (256QAM) |
| Test metric | 70% of the maximum throughput |

Table 2-2: Candidate options for MCS pairs selection

|  | Rank | MCS table | MCS index |
| --- | --- | --- | --- |
| Carrier with lower power | Rank 1 | MCS table 1 | Option 1: MCS4 (QPSK, 0.30) Option 2: MCS5 (QPSK, 0.37) |
| Carrier with higher power | Rank 2 | MCS table 2 | Option 1: MCS22 (256QAM, 0.74) Option 2: MCS23 (256QAM, 0.78) Option 3: MCS24 (256QAM, 0.82) Option 4: MCS25 (256QAM, 0.86) |

Simulation results are provided in Table 2-3 (Weaker CC) and Table 2-4(Stronger CC).

Table 2-3: Simulation results for weaker CC

| MCS | SNR(dB)@70% of max TP (Ideal) | SNR(dB)@70% of max TP (Impairment) |
| --- | --- | --- |
| 4 | -4.3 | -3.3 |
| 5 | -3.4 | -2.4 |

Table 2-4: Simulation results for stronger CC

| MCS | SNR(dB)@70% of max TP (Ideal) | SNR(dB)@70% of max TP (Impairment) |
| --- | --- | --- |
| 22 | 20.4 | 21.4 |
| 23 | 22.1 | 23.1 |
| 24 | 22.6 | 23.6 |
| 25 | 24.4 | 25.4 |

Conclusion

In this paper we provide our simulation results on UE performance for Non-collocated CA.

Reference

[1]    R4-2313876 WF for NonCol_intraB_ENDC_NR_CA_Demod. Ericsson