3GPP TSG-RAN WG4 Meeting # 109	R4-2320200

# Chicago, US, November 13-17, 2023

Title: 	Discussions on Rel-18 DSS performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.36.2

Document for:	Discussion

Background

In last meeting, a WF on enhancement of DSS demodulation requirements was approved and many open issues were left. In this contribution, we provide our views on these open issues and test setup.

- Discussions

The agreements and open issues according to [1] are captured as follows:

Whether to define PDCCH demodulation requirements for Rel-18 eDSS feature Agreement: - Define PDCCH demodulation requirements for eDSS Evaluation scenario Agreement: - 2-symbol CORESET - Basic assumptions - Rate matching pattern - Option 1: Single non-overlapping CRS rate matching patterns for PDCCH demodulation. - Other options not precluded - 4 CRS ports for LTE - Symbol #0: LTE PDCCH/PCFICH - Symbol #1: NR PDCCH overlapping with LTE CRS - Symbol #2: NR PDCCH gNB assumption for PDCCH symbols overlapped with LTE CRS Agreement: - PDCCH data and DMRS REs overlapped with LTE CRS are punctured. UE receiver assumption (e.g., channel estimation) Agreement: - PDCCH channel estimation is assumed to use only the clean PDCCH symbol. Simulation assumption for evaluation Way forward: - FFS on the parameter configuration for the PDCCH - Option 1 - Parameters; Values. Carrier frequency; 2 GHz. SCS; 15 kHz. Bandwidth; 10 MHz. Channel model; TDLC300-100. Antenna configuration; 1x2, 1x4; 2x2, 2x4. CRS; 4 port CRS without power boosted. DCI payload (excluding CRC); 60 bits. Interleaving; Non-interleaved. Precoding; Precoder cycling per REG bundle. REG bundle size; 6 REGs. Test number; Bandwidth (MHz); Symbols with PDCCH; Aggregation level; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 10; 2; 4; TDLA30-10; 1x2, 1x4 Low; 1. 2; 10; 2; 8; TDLC300-100; 2x2, 2x4 Low; 1. - Option 2 - Cover all PDCCH Aggregation levels for evaluation purpose. (Not for requirements definition purpose) Parameters; Values. Carrier frequency; 2 GHz. SCS; 15 kHz. Bandwidth; 20MHz. LTE Bandwidth; 20MHz. Channel model; TDL-C 300-100. Correlation; Low. Number of BS antennas; 4Tx. Number of UE antennas; 2 Rx. DCI payload (excluding CRC); 60 bits. Interleaving; Non-Interleaved. Precoding; Precoder cycling per REG bundle. REG bundle size; 6 PRBs. CRS; single 4 port CRS pattern. Aggregation level; 1,2,4,8,16. CORESET; Scenario a): CORESET: 2nd and 3rd symbols, LTE: 4 CRS Ports Scenario b): CORESET: 2nd symbol, LTE: 4 CRS Ports. Channel estimation; Scenario a): Use clean symbol Scenario b): Use legacy channel estimation. Transmitter; DMRS REs and data REs overlapped with CRS are punctured. Receiver; Puncture the CRS REs. Power ratio of LTE-CRS RE/NR PDCCH-DMRS RE; 3dB.

Regarding the Rate matching pattern, only single non-overlapping CRS rate matching pattern is sufficient since CRS rate matching patterns with different vShift leads to same number of punctured REs per RB which leads to same performance. Furthermore, supporting multiple non-overlapping CRS patterns is an additional optional feature. Therefore we don’t see any necessity to consider multiple non-overlapping CRS rate matching patterns

Observation 1: CRS patterns with different vShift leads to same number of punctured REs per RB which leads to same performance.

Proposal 1: Only consider single non-overlapping CRS rate matching pattern

It’s unnecessary to model LTE PDCCH/PCFICH transmission on symbol #0 since it is not used for NR PDCCH transmission.

Proposal 2: Don't model LTE PDCCH/PCFICH transmission on symbol #0

MIMO detection assumption for CRS REs was not discussed in last meeting. Conditioned that PDCCH DMRS and data on CRS REs are punctured, there are two potential implementations: 1) UE perform equalization and LLR computation as legacy way, 2) UE set LLR of PDCCH data to 0 for CRS REs. The latter method can achieve better performance with low complexity which could be considered as receiver assumption for performance requirements definition.(Check with Huameng about the receiver assumption during RAN1 evaluation)

Proposal 3: For performance requirements definition, consider receiver assumption: UE set LLR of PDCCH data to 0 for CRS REs.

Regarding the bandwidth/SCS, 10MHz/15kHz can be used for FDD, regarding TDD, we are open to discuss whether to use 10MHz/15kHz (used for R15 NR-LTE co-existence case) or 20MHz/15kHz (used for CRS-IM test)

Proposal 4:  Consider 10MHz/15kHz for FDD, regarding TDD, FFS 10MHz/15kHz or 20MHz/15kHz

Considering the limited TU assignment for Rel-18, test cases number should be minimized as much as possible. One Aggregation level for each duplex mode is enough. We provide the initial simulation results in Figure 2-1 and Table 2-1, to guarantee the reasonable SNR range, we propose to use AL 4.

Proposal 5:  Only consider AL4 for requirements definition

Figure 2-1: BLER-SNR curve

Table 2-1: Target SNR@ 1% of BLER summary

| AL | 1 | 2 | 4 | 8 | 16 |
| --- | --- | --- | --- | --- | --- |
| Target SNR @1% BLER | 15.9 | 7.0 | 1.5 | -2.5 | -6.0 |

- Conclusion

In this meeting, we provide our views on test setup for PDCCH requirements overlapped with LTE CRS. The proposals and observation are:

Observation 1: CRS patterns with different vShift leads to same number of punctured REs per RB which leads to same performance.

Proposal 1: Only consider single non-overlapping CRS rate matching pattern

Proposal 2: Don't model LTE PDCCH/PCFICH transmission on symbol #0

Proposal 3: For performance requirements definition, consider receiver assumption: UE set LLR of PDCCH data to 0 for CRS REs.

Proposal 4:  Consider 10MHz/15kHz for FDD, regarding TDD, FFS 10MHz/15kHz or 20MHz/15kHz

Proposal 5:  Only consider AL4 for requirements definition

- Reference

[1]    R1-2317000 WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements. Ericsson

- Reference

Table 5-1: Simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Bandwidth | 20MHz |
| LTE Bandwidth | 20MHz |
| Channel model | TDL-C 300-100 |
| Correlation | Low |
| Number of BS antennas | 4Tx |
| Number of UE antennas | 2 Rx |
| DCI payload (excluding CRC) | 60 bits without CRC |
| Interleaving | Non-Interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBs |
| CRS | single 4 port CRS pattern |
| Aggregation level | 1,2,4,8,16 |
| CORESET | Symbol 1 and 2 |
| Channel estimation | Use clean symbol |
| Transmitter | DMRS REs and data REs overlapped with CRS are punctured |
| Receiver | Puncture the CRS REs |
| Power ratio of LTE-CRS RE/NR PDCCH-DMRS RE | 3dB |