3GPP TSG-RAN WG4 Meeting #109	R4-2320216

# Chicago, 13th - 17th Nov, 2023

Title: 	Simulation results on FR2 UL 256QAM performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.6.4.1

Document for:	Discussion

Background

During RAN4#108b meeting, Way Forward [1] for 256 QAM BS Demod was approved. In this contribution, we provide our updated simulation results for FR2 UL 256QAM demodulation performance.

Simulation

| PTRS | SCS/BW | DMRS | Ideal | Impairment |
| --- | --- | --- | --- | --- |
| Enabled | 60kHz 50MHz | 1+1 | 16.8 | 19.4 |
|  |  | 1+0 | 17.1 | 19.9 |
|  | 120kHz 50MHz | 1+1 | 16.2 | 18.7 |
|  |  | 1+0 | 16.6 | 19.3 |
|  | 120kHz 100MHz | 1+1 | 16.1 | 18.8 |
|  |  | 1+0 | 16.4 | 19.2 |
|  | 120kHz 200MHz | 1+1 | 16.2 | 19 |
|  |  | 1+0 | 16.4 | 19.4 |
| Disabled | 60kHz 50MHz | 1+1 | 16.2 | 18.7 |
|  | 120kHz 50MHz | 1+1 | 15.5 | 18 |
|  | 120kHz 100MHz | 1+1 | 15.5 | 18.1 |
|  | 120kHz 200MHz | 1+1 | 15.5 | 18.2 |

Conclusion

In this contribution, we provide our updated simulation results for FR2 UL 256QAM demodulation performance.

Reference

R4-2316903, Way Forward for 256 QAM BS Demod, RAN4#108b, Nokia, Nokia Shanghai Bell