3GPP TSG-RAN WG4 Meeting #109	R4-2320233

# Chicago, 13th - 17th Nov, 2023

Title: 	Discussion on general issues for UE demodulation requirements for NR FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	8.7.4.1

Document for:	Discussion

Background

During RAN4#108b meeting, WF [1] on UE demodulation and CSI requirements for FR2 multi-Rx DL reception was approved. In this contribution, we share our views about general issues for UE demodulation requirements for NR FR2 multi-Rx chain DL reception.

Discussion

Receiver assumption for mDCI case

- Agreement: - Define requirement with separate processing as a baseline for fully overlapping (1+1) and non-overlapping (2+2) scenarios. - Other scenarios not precluded, such as fully overlapping (2+2) - FFS: Joint processing - Companies to evaluate performance of both separate and joint processing for the next meeting with the objective to make a decision next meeting. A minimum set of simulation scenarios may be agreed to manage the workload for the next meeting. - Baseline case is prioritized.

For the mDCI fully-overlapping case, the simulation results are shown as following Table 2.1-1.

Table 2.1-1 Ideal simulation results for mDCI mTRP fully-overlapping

| Case number | MCS | Rank | UE processing | ρ | SNR@70%maximum throughput |
| --- | --- | --- | --- | --- | --- |
| 1 | 17 | 1+1 | separate | -12dB | 9.8 |
| 2 | 17 | 1+1 | joint | -12dB | 9.4 |

We see that there is negligible performance difference between separate and joint UE processing for MCS 17 with rank 1 for mDCI mTRP fully-overlapping. Considering the balance of the performance and the complexity, we propose to define requirements only based on separate processing for FR2 multi-Rx mDCI mTRP fully-overlapping case in Rel-18.

Define requirements based on separate processing for FR2 mDCI mTRP fully-overlapping case in Rel-18.

For the mDCI non-overlapping case, the simulation results are shown as following Table 2.1-2.

Table 2.1-2 Ideal simulation results for mDCI mTRP non-overlapping

| Case number | MCS | Rank | UE processing | ρ | SNR@70%maximum throughput |
| --- | --- | --- | --- | --- | --- |
| 1 | 13 | 2+2 | separate | -12dB | 11.2 |

Receiver assumption for sDCI SDM case

- Agreement: - Evaluate joint and separate processing for sDCI. - For sDCI, consider separate processing as baseline. Joint processing is FFS.

For the sDCI SDM case, the simulation results are shown as following Table 2.2-1.

Table 2.2-1 Ideal simulation results for sDCI SDM

| Case number | MCS | Rank | UE processing | ρ | SNR@70%maximum throughput |
| --- | --- | --- | --- | --- | --- |
| 1 | 17 | 1+1 | separate | -12dB | 10.0 |
| 2 | 17 | 1+1 | joint | -12dB | 9.5 |

We see that there is negligible performance difference between separate and joint UE processing for MCS 17 with rank 1 for sDCI SDM. Considering the balance of the performance and the complexity, we propose to define requirements only based on separate processing for FR2 multi-Rx sDCI SDM case in Rel-18.

Define requirements based on separate processing for FR2 multi-Rx sDCI SDM case in Rel-18.

TxEVM

- Agreement: - If considered, then use Tx EVM at 6% since we are considering up to 64QAM modulation.

Considering that TxEVM is included in the PDSCH requirements for all other WIs, we confirm to use the same method, i.e. consider Tx EVM at 6% for FR2 multi-Rx demodulation requirements.

Consider Tx EVM at 6% for FR2 multi-Rx demodulation requirements.

TR update

<way forward> - FFS whether to consider expanding the scope of TR 38.751 751 to include demodulation related evaluation and study.

Concern about the workload, we propose to not consider expanding the scope of TR 38.751 to include demodulation related evaluation and study.

Do not consider expanding the scope of TR 38.751 to include demodulation related evaluation and study.

Proposals

In this contribution, we discuss on general issues for UE demodulation requirements for NR FR2 multi-Rx chain DL reception. Our observations and proposals are:

- Define requirements based on separate processing for FR2 mDCI mTRP fully-overlapping case in Rel-18.

Define requirements based on separate processing for FR2 multi-Rx sDCI SDM case in Rel-18.

Consider Tx EVM at 6% for FR2 multi-Rx demodulation requirements.

Do not consider expanding the scope of TR 38.751 to include demodulation related evaluation and study.

Reference

R4-2316963, WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception, RAN4#108b, Qualcomm