3GPP TSG-RAN WG4 Meeting #109	R4-2320237

# Chicago, 13th - 17th Nov, 2023

Title: 	Simulation results on demodulation performance requirements for NR network-controlled repeaters

Source: 	Huawei, HiSilicon

Agenda item:	8.28.6

Document for:	Discussion

Background

During RAN4#108b meeting, WF [1] on NCR-MT demodulation requirements was approved. In this contribution, we provide our updated simulation results about NR network-controlled repeaters.

Simulation

Based on our discussion Tdoc in the same topic, here we only list the cases that need to perform new simulation.

FR1 PDSCH

| Case number | Bandwidth (MHz) | SCS (kHz) | MCS | Propagation condition | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 70% max throughput | 1% BLER |
| 1 | 10 | 15 | 4 | TDLA30-10 | 2x2 | -3.5 | 1.7 |
| 2 | 10 | 15 | 4 | TDLA30-10 | 2x4 | -6.6 | -3.0 |
| 3 | 10 | 15 | 13 | TDLA30-10 | 2x2 | 4.2 | 9.4 |
| 4 | 10 | 15 | 13 | TDLA30-10 | 2x4 | 1.3 | 4.7 |
| 5 | 40 | 30 | 4 | TDLA30-10 | 2x2 | -3.4 | 1.2 |
| 6 | 40 | 30 | 4 | TDLA30-10 | 2x4 | -6.4 | -3.2 |
| 7 | 40 | 30 | 13 | TDLA30-10 | 2x2 | 4.4 | 8.9 |
| 8 | 40 | 30 | 13 | TDLA30-10 | 2x4 | 1.4 | 4.2 |

FR1 PDCCH

| Case number | Bandwidth (MHz) | SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna configuration | SNR@1% BLER (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 15 | 48 | 1 | 8 | TDLA30-10 | 2x4 | -5.5 |

FR2 PDSCH

| Case number | Bandwidth (MHz) | SCS (kHz) | Propagation condition | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 70% max throughput | 1% BLER |
| 1 | 100 | 120 | TDLA30-75 | 2x2 | -3.3 | 0.0 |

Conclusion

In this contribution, we provide our updated simulation results about NR network-controlled repeaters.

Reference

R4-2316923, WF on NCR-MT demodulation requirements, RAN4#108b, ZTE Corporation