3GPP TSG-RAN WG4 Meeting #109	R4-2320239

# Chicago, 13th - 17th Nov, 2023

Title: 	Discussion on SAN demodulation requirements for NR NTN enhancements

Source: 	Huawei, HiSilicon

Agenda item:	8.26.8.1

Document for:	Discussion

Background

During RAN4#108b meeting, WF [1] on NR_NTN_enh_SAN_UE_demod was approved. In this contribution, we share our views about SAN demodulation requirements for NR NTN enhancements.

Discussion

Test scope

PUSCH for above 10 GHz bands

- Agreement - Define PUSCH with DFT-S-OFDM and CP-OFDM demodulation performance requirements for above 10 GHz bands. - FFS PUSCH repetition Type A, companies can do some analysis on the link budget. - Do not consider UL timing adjustment

Based on RF co-existence evaluation [2], the link budget is not very good for the uplink, we think such repetition scheme has benefit for above 10 GHz bands. In addition, PUSCH repetition Type A is considered in legacy Rel-15 FR2 demodulation performance requirements and is also considered in Rel-17 FR1 NTN demodulation performance requirements. Therefore, we propose to define PUSCH repetition Type A demodulation performance requirements for above 10 GHz bands.

Define PUSCH repetition Type A demodulation performance requirements for above 10 GHz bands.

PUSCH for DMRS bundling

- Agreement - Define SAN PUSCH demodulation performance requirements for DMRS bundling for FR1 only if RF confirm that phase continuous is feasible.

In last meeting, there is agreement achieved in RF session that “Requirement to be defined in RAN4 for PUSCH DMRS bundling for NR NTN coverage enhancement in Rel-18 with the assumption of the zero Doppler shift and constant delay”. Therefore, we confirm to define SAN PUSCH demodulation performance requirements for DMRS bundling for FR1.

Confirm to define SAN PUSCH demodulation performance requirements for DMRS bundling for FR1.

PUCCH for Msg4 HARQ-ACK

- Agreement - Do not define PUCCH requirements Msg4 HARQ-ACK for NLOS channel. - Way forward - Option 1: Add new requirements under LOS channel for multi-slot PUCCH format 1. - Option 2: Do not define PUCCH requirements Msg4 HARQ-ACK for LOS channel.

In Rel-17, both LOS and NLOS channel model are candidate for evaluation. Considering that both LOS and NLOS channel model have already been covered by the PUSCH requirements, it is enough to only consider LOS channel for multi-slot PUCCH format 1 to avoid duplicate tests. In Rel-18, we prefer to follow the same logic to not define PUCCH requirements Msg4 HARQ-ACK for LOS channel.

Do not define PUCCH requirements Msg4 HARQ-ACK for LOS channel.

General issues for above 10GHz bands

Channel bandwidth

- Agreement - Start demod work with 120 kHz SCS for initial alignment. If RRM concludes that 120 kHz SCS is not feasible, then further discussion is needed at that time on how to proceed - FFS channel bandwidth

As per RF agreements, three new NTN satellite bands n512/n511/n510 is introduced with {50, 100, 200, 400}MHz bandwidth configuration for 120kHz SCS. Considering that 200MHz for 120kHz SCS is selected in RAN4 RF co-existence study, we propose to select the same value for demodulation performance requirements for FR2-NTN. In addition, the minimum bandwidth can also be considered to ensure the test coverage.

Select 50MHz and 200MHz for 120kHz SCS for demodulation performance requirements for FR2-NTN.

MCS & rank

- Agreement - Postpone to discuss this issue after channel model is selected. - Companies can do some link budget analysis until the next meeting.

Based on RF co-existence evaluation [2], the average link budget for Rel-18 FR2 LEO-600 NTN scenario is about 3.5dB better than Rel-17 FR1 LEO-600 NTN scenario at 95% percentile in the CDF curve. However, it is still difficult to schedule 16QAM or rank 2 (SINR=6.1dB). So we propose to select rank 1 with MCS4 (QPSK, 308/1024) and MCS5 (QPSK, 99/1024, Table 3, for PUSCH repetition Type A case only) that is same as Rel-17 FR1-NTN as the starting point.

Select rank 1 with MCS4 (QPSK, 308/1024) and MCS5 (QPSK, 99/1024, Table 3, for PUSCH repetition Type A case only) that is same as Rel-17 FR1-NTN as the starting point.

Antenna configuration

- Agreement - For the SAN Rx, we need both 1Rx and 2Rx. For UE Tx, more discussion is needed to cover 1 Tx only or also 2Tx.

For the antenna configuration, to simplify the test setup, RAN4 principle is to select 1Tx for rank 1 and 2Tx for rank 2. Since we prefer to only consider rank 1, 1Tx should be used.

Only consider 1Tx for FR2 NTN demodulation requirements.

SAN type

- Agreement - Prioritize to define SAN demodulation requirement for Type 2-O. FFS to define BS demodulation requirement for Type 2-H, pending on conclusion of RF core requirement.

Considering that there is no any RF core requirement defined for SAN Type 2-H, we propose to not define SAN demodulation requirement for SAN Type 2-H.

Do not define SAN demodulation requirement for SAN Type 2-H.

Test setup for above 10GHz bands

PUSCH

We propose to use the test parameters as following Table 2.3.1-1 for SAN PUSCH demodulation performance requirements and Table 2.3.1-2 for SAN PUSCH repetition Type A demodulation performance requirements.

Table 2.3.1-1 Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled, Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |

Table 2.3.1-2 Test parameters for testing PUSCH repetition Type A

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| NOTE 1: The effective RV sequence is {0,2,3,1} with slot aggregation |  |  |

Use the test parameters as above Table 2.3.1-1 for SAN PUSCH demodulation performance requirements and Table 2.3.1-2 for SAN PUSCH repetition Type A demodulation performance requirements.

PUCCH

We propose to use the test parameters as following Table 2.3.2-1 to Table 2.3.2-6 for SAN PUCCH demodulation performance requirements.

Table 2.3.2-1: Test Parameters for PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |

Table 2.3.2-2: Test Parameters for PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

Table 2.3.2-3: Test Parameters for PUCCH format 2 (ACK missed detection)

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |

Table 2.3.2-4: Test Parameters for PUCCH format 2 (UCI BLER)

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |

Table 2.3.2-5: Test Parameters for PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |

Table 2.3.2-6: Test Parameters for PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hoppingstartingPRB | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

Use the test parameters as above Table 2.3.2-1 to Table 2.3.2-6 for SAN PUCCH demodulation performance requirements.

PRACH

We propose to use the test parameters as following Table 2.3.3-1 to Table 2.3.3-3 for SAN PUCCH demodulation performance requirements.

Table 2.3.3-1: Time error tolerance for AWGN, NTN-TDLA100-3000

| PRACH preamble | PRACH SCS (kHz) | Time error tolerance |  |
| --- | --- | --- | --- |
|  |  | AWGN | NTN-TDLA100-3000 |
| B4, C2 | 120 | 0.065 us | 0.35 us |

Table 2.3.3-2: Frequency offset for AWGN, NTN-TDLA100-3000

| PRACH preamble | PRACH SCS (kHz) | Time error tolerance |  |
| --- | --- | --- | --- |
|  |  | AWGN | NTN-TDLA100-3000 |
| B4, C2 | 120 | 0 | 3000Hz |

Table 2.3.3-3 Test preambles

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| B4, C2 | 120 | 69 | 0 | 0 |

Use the test parameters as following Table 2.3.3-1 to Table 2.3.3-3 for SAN PUCCH demodulation performance requirements.

Test setup for UL coverage enhancement

Frequency and timing drift

- Proposals - Option 1: Further discuss whether assumption of no timing drift modelling can be applied for DMRS bundling requirement pending on RF conclusion.

As we discussed in section 2.1.2, zero Doppler shift and constant delay should be assumed based on RF agreement, so we propose to not consider timing drift modelling for DMRS bundling demodulation requirements.

Do not consider timing drift modelling for DMRS bundling demodulation requirements.

PUSCH with DMRS bundling

In RAN4#108, there is agreement achieved in RF session that “Taking existing requirements specified for the maximum allowable phase difference for DMRS bundling captured in Table 6.4.2.5-1 of TS38.101-1 as starting point”. So we propose to use the same PUSCH aggregation factor and pusch-TimeDomainWindowLength as the legacy DMRS bundling demodulation requirements.

Use the same PUSCH aggregation factor and pusch-TimeDomainWindowLength as the legacy DMRS bundling demodulation requirements.

We propose to use the test parameters as following Table 2.4.2-1 for SAN PUSCH with DMRS bundling demodulation performance requirements.

Table: 2.4.2-1 Test parameters for testing PUSCH with DM-RS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence [Note 1] | 0, 0, 0, 0 for FDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n8 for FDD |
| pusch-TimeDomainWindowLength |  | 8 slots for FDD |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Use the test parameters as following Table 2.4.2-1 for SAN PUSCH with DMRS bundling demodulation performance requirements.

Proposals

In this contribution, we discuss on SAN demodulation requirements for NR NTN enhancements. Our observations and proposals are:

- Define PUSCH repetition Type A demodulation performance requirements for above 10 GHz bands.

Confirm to define SAN PUSCH demodulation performance requirements for DMRS bundling for FR1.

Do not define PUCCH requirements Msg4 HARQ-ACK for LOS channel.

Select 50MHz and 200MHz for 120kHz SCS for demodulation performance requirements for FR2-NTN.

Select rank 1 with MCS4 (QPSK, 308/1024) and MCS5 (QPSK, 99/1024, Table 3, for PUSCH repetition Type A case only) that is same as Rel-17 FR1-NTN as the starting point.

Only consider 1Tx for FR2 NTN demodulation requirements.

Do not define SAN demodulation requirement for SAN Type 2-H.

Use the test parameters as above Table 2.3.1-1 for SAN PUSCH demodulation performance requirements and Table 2.3.1-2 for SAN PUSCH repetition Type A demodulation performance requirements.

Use the test parameters as above Table 2.3.2-1 to Table 2.3.2-6 for SAN PUCCH demodulation performance requirements.

Use the test parameters as following Table 2.3.3-1 to Table 2.3.3-3 for SAN PUCCH demodulation performance requirements.

Do not consider timing drift modelling for DMRS bundling demodulation requirements.

Use the same PUSCH aggregation factor and pusch-TimeDomainWindowLength as the legacy DMRS bundling demodulation requirements.

Use the test parameters as following Table 2.4.2-1 for SAN PUSCH with DMRS bundling demodulation performance requirements.

Reference

R4-2316921, WF on NR_NTN_enh_SAN_UE_demod, RAN4#108b, Huawei, HiSilicon

R4-2313986, NTN co-existence in above 10GHz calibration results, RAN4#108, Samsung